Aircraft Maintenance Publications

James Huff


Section One

1.

A. FUSL  Federal, DOT/FAA, TC/SPEC/STCS

ACSB  Airframe, Cessna, Single engine airframe

PHZT  Propeller, Hartzell, General publications

ETDR  Engine, Teledyne Continental, Opposed reciprocating engines

B.


1.
Cessna, Airframe, Piper, Reciprocating, Vendor, and Federal Complete are the libraries located in the Aviation I library area.

2.
Propeller/Governor, Turbo Prop. Engine, Vendor, and Federal-1A are the libraries located in the Aviation II library area.

3.
The card number for the FAA Type Certificate Data Sheet/Spec. Sheet indexes is FUSL 003.

4.
Card FUSJ has the Numerical Index for the FAA A.D. Notes.

5.
FUSH 056 is the number of the card that has FAA Advisory Circular 43.13-2A on it.

6.
FUSF 190-191 has FAR 147 (Aviation Maintenance Technician Schools) on it.

7.
EPWS is the alpha part of the alpha-numeric system for cards pertaining to a Pratt & Whitney turbine engine such as a PT-6.

2.

D.

1.
Temporary storage is a non-operating status for a maximum of 90 days.

2.
Fuel bays should be filled immediately after flight.

3.
A pre-flight inspection should be done before flying the aircraft each day.

4.
The battery is located aft of the right hand wheel well.

5.
The ground service receptacle is located on the right fuselage behind the wing.

6.
The power source is a 24-volt, DC, ground power unit.

7.
Ashless, dispersant oil should be used after 75 hours of operation.

8.
Oil filters should be changed every 6 months.

3.

C.

1.
Direct drive, four, six, and eight cylinder horizontally opposed, air-cooled models are covered in this manual.

2.
Cylinders on a Lycoming engine is air-cooled.

3.
GSO540 - Geared nose section, supercharged, opposed cylinders, 540 cu. in. displacement.


IO360 - Fuel injected, opposed cylinder, 360 cu. in. displacement.


HO320 - Horizontal Helicopter, opposed cylinder, 320 cu. in. displacement.

4.
Service instruction no. 1181 can be used to find the color code used to indicate differences in cylinder construction.

4.

C.

1.
Gray or unpainted



Standard steel barrels


Orange





Chrome plated steel barrels


Blue





Nitride hardened steel barrels


Green





Steel cylinder, .010 oversized


Yellow





Steel cylinder, .02 oversized

2.
Grey or unpainted


Cylinder design for short reach spark plugs

Yellow




Cylinder design for long reach spark plugs

3.
TEXTROM Lycoming does not sell oversized cylinders

Section Two

A.

1.
The color of the title strips for FAA Airworthiness Directives is red.

2.
The color of the title strips for Manufacturer Service Information is green.

3.
The color of the title strips for Parts Catalogs is blue.

B.

1.
Information on yellow cards pertains to overhauls.

2.
Information on purple cards pertains to vendor data and FAA advisory circulars.

3.
Information on green cards pertains to maintenance manuals and wiring diagram parameters.

4.
Information on orange cards pertains to the master reference.

5.
Information on red cards pertains to government publications.

C.
The part of the card that should be handled by the hand is the printed strip.

D.
The rest of the card should not be handled because it can be easily scratched.

E.
The cards should be put back in the proper place to they can be found for future use.

