Hong Kong Institute of Vocational Education

HD of Automotive Engineering
Visit Report 2

Name: Cheung Chi Wai 

Class: 91305/3 (7)

Venue: Transport department kowloon bay Vehicle Examination Centre

Objective: To observe the procedure of testing taxi and tilt test of buses (double Decker)  

Stats of vehicle examination centre: 12 links for testing different types of vehicles; taxi, mini bus, commercial vehicles, etc. 

A tilt tester for testing buses.

Procedure of testing taxi:
1. Testing the steering system of taxi, if there are any problem will be affecting the safety.

2. Using the lamp tester to test the lamp’s lighting position.
3. Testing the braking efficiently of the vehicle. Taxi driven onto the brake tester to test different components of brake following the signal of indicator, which mounted on the ceiling.

4.  Simulate the real running stats of taxi, to the test the calculator of taxi. Calculator shouldn’t be jumped until 2000m form start , second jump should on 100m later.

Tilt test of buses:

Because of several roll-over cases cause peoples die in the past year, tilt test have been introduced to prevent accident occur. Requirement of passing the test are both side buses shouldn’t be fell-down in 28 degree tilt (35 degree for single Decker), under the extreme condition (full load in upper Decker, only driver seat in lower Decker.

Procedure of tilt test:

1. drive the buses onto the tester 

2. Starting to move the tester.

3. Slightly move the tester to 28 degree.

4. If the vehicle didn’t fell-down, one side of the vehicles can be passed.

5. Repeat procedure of another side.  
