RIGGING FOR RESCUE SEMINAR NOTES SUMMARY

Prepared by Kevin Wright

Saturday, October 6, 2007

1900 hrs

-Orientation

-Introduction of seminar attendees, dialogue of desired skill attainment

-Safety Expectations

-Gloved hands when dealing with rope in motion

-Helmet for overhead hazards, on edge, over edge, any time on rope

-Buddy system for double checking harnesses, systems, etc.

-Within 2 meters of edge all personnel will be on an edge restraint

-When teams construct a system, an independent person/team will inspect

-Use visual and tactile inspections

-When belay is on, professionalism is expected

-Resources and history

-All R4R information and methods developed by British Columbia Council

Technical Rescue (BCCTR)

-Homework and Readings

-Belay definitions & summary

-Memorize material strengths list

-Memorize change of direction dynamics

-Go over S.I. units

Sunday, October 7, 2007

0800 hrs

-Physics 101

-Systems of measurement (Systems International- S.I.)

-mass, time, distance, velocity, acceleration, force

-Static System Safety Factor (SSSF)

-Ratio of breaking strength to applied force

-ex. 10kn:1kn		10:1

-The expected SSSF for intl. rescue applications is 10:1

-Margin of Safety

-The difference of breaking strength to applied force

-ex. 10kn-1kn=9kn  	(margins are always given in units)

-Fall Factor

-The ratio of a length of a fall to the rope in service

-ex. 10m fall : 5 m rope = Fall Factor of 2

-BCCTR Standard 10:1

-Worst case scenario: steep, large load, high fall factor, little rope in service

-1 meter drop (edge transition), 3 meters of rope in service, 200 kg load, 11mm

rope, less than or equal to 1meter of stopping distance

-Systems Analysis

-Go through every piece of system and find breaking strength

-Use materials breaking strength list

-5kn for any piece of rock pro as an anchor (including pitons)

-20kn for bolts (but use two for redundancy policy)

-Average knot decreases rope strength by 30% (thus 11mm rope decreases

to 20kn breaking strength, and this is often the weakest point in system)

-Weakest link sets the SSSF

-Pulley angles and change of direction dynamics

-0 degrees=equal force on each rope doubling force on pulley

-90 degrees=equal force on each rope putting 1.4 times force on pulley

-120 degrees=equal force on each rope putting 1 time of force on pulley

1200 hrs

-Ties

-Knots = termination

-Bends = joining ends

-Hitches = requires an object

-Considerations for choice and use

-Applied load

-Strength

-Application

-Ease of tying

-Ease of untying

-Ease of identification

-General material considerations (size and quality)

-Adjustability

-Security

-Amount of material

-Types

-Knots = 8 family, bowline, butterfly, overhand family

-Bends = double overhand, sheet, ring, European death knot

-Hitches = girth, clove, munter (Italian), prusik

-Rescue Belays

-Tandem Prusik Belay (TPB)

-2 prusiks =  1 long, 1 short

-8mm cord

-prusiks on spine side of biner, not pulley

-triple wrapped prusiks

-passes pinch test (prusiks are hitches- better too supple than too stiff)

-pretied prusik lengths 1.35m (53”) short, 1.65m (65”) long

-release hitch necessary

-540

-Green = 11mm, Blue = 12.5mm

-hands work rope, not load

-lowering=use back tension w/ one hand, feed into 540 with other

-when greater than 30meters of rope in service, use a descent device for

back tension

-release hitch unneeded

-Release Hitches

-Releasable under tension

-Can it withstand a dynamic event?

-Strength (10:1 SSSF)

-Easy to tie/untie

-Ease of identification

-Release distance

-Grip required

-Commonality of equipment (multiple use)

-Cost

-Radium Release Hitch

-10 meters 8mm cord

-3:1 with munter, half hitch, overhand back-up

Monday, October 8, 2007

0800 hrs

-Anchoring

-Anchor point = individual attachment point

-Anchor system = combination of anchor points

-Focal point = anchor system attachment point (ex. Main line, TPBelay)

-Use best anchor system for belay (potential for dynamic forces)

-Use prusiks, round turn, fig. 8 on a bight to use up extra rope

-Favor stronger anchors w/ more material

-20kn anchor system (at least 4 rock pro, 2 bolts)

-Try and put main line focal point far enough behind edge for haul space

-Rigging Scene Size Up

-Patient Location

-# of patients

-Patient Condition

-Objective Hazards

-Resources

-Personnel

-Terrain type

-Suggested Operating Procedure

1.Identify edge transition

-look for pinch points (worse than sharp edges), use edge protection

-think about rock fall potential

2.Identify rope alignment (fall line)

-if alignment isn’t obvious, think about change of direction

3.Focal points

4.Identify anchors

-Climber Commands

-Yosemite call system

1100 hrs

-Field Exercise

-Rappelling and prusiking passed a knot in a single line

-Will be belayed by second line

Tuesday, October 9, 2007

0800 hrs

-Pretensioned back ties for strengthening anchor systems, focal points

-Pretensioned front ties to get rid of potential issues from shock load stretch

-try and put pretensioning anchors in line (within 15 degrees of alignment)

-Probability

-coin comes up heads?  1st .5  2nd .5  Heads twice= .5x.5=.25

-main failure= 1 in 10,000; belay failure= 1 in 10,000; both fail= 1 in 100,000,000

-Pulley Systems

-Output : Input

-Mechanical Advantage (MA)

-Types

-Simple (all traveling pulleys move toward anchor at the same speed =

block and tackle)

-Compound (combination of more than one simple system)

-Complex (isn’t simple or compound)

-Natural progressions for pulley systems

-Odd (termination at load)= S3:1, S5:1, C9:1, C15:1 (S5:1 on S3:1)

-Even (termination at anchor)= S2:1, S4:1, C6:1

1030 hrs

-Field Objective: descend, switch to raise, and raise a 2kg load (attendant and patient)

-Tie in using long tailed intertwined bowlines

-Use centralized command and communication

-Team Roles

-Leader

-Control

-Belay

-Main Line

-Edge

-Attendant

-Communication SOP

1. Role Call (station check)

-Begin with belay, end with attendant (“Belay, ready?”…“Belay is ready”)

2. Position Load

-“Attendant approach edge.”

3. Pretension Main Line

-“Main Line.  Pretension main.”

-Response, “Control.  Main line pretensioned, 6 bars.”

4. Move Load

-“Attendant lean back”

-“Main down.”

Wednesday, October 10, 2007

0800 hrs

-T-method for figuring out MA in complex pulley systems

-Theoretical MA in T-method

-Artificial High Directional

-Rigging considerations

1.Resultant force vector in footprint

2.Tether AHD

3. Splay prevention @ legs

-Bridal concepts

-Adjustable vs. non-adjustable

-Most use (including R4R and OMR non-adjustable)

-10 degree angle up on head side balances load to level when in use

-long tailed intertwined bowlines

-belay to attendant (in case of low angle attendant disconnect)

-main to patient

-Patient tie in

-Double x (low body and upper body) into basket

-Hasty harness

-Feet harnessing

1030 hrs

-Field Objective: Practicing attendant position (lower and upper) with litter

-Extra short prusik helps attendant move into position

Thursday, October 11, 2007

0800 hrs

-Lowers with 30meters or more of rope in service

-Vectors

-magnitude and direction

-types

-component (one part)

-resultant (sum of all)

-arbitrary scale method

-How steep is too steep?

-R=resistant force

-Mg=mass x gravity (force)

-T=tension

-When figuring degrees one hand (four fingers) is approx. 10 degrees

1030 hrs

-Field Objective: practice pick offs

Friday, October 12, 2007

0800 hrs

-Pulley power point

-Tower lead climbing

-Options

-lanyard

-leading

-cable w/ grab

-aerial

-Guidelines if leading

-fall factor less than or equal to 1:3

-protect every body length

-pulley at first clip

1030 hrs

Field Exercise Objectives: Pike and Pivot Technique

Saturday, October 13, 2007

0800 hrs

-Kootenay Highline Systems (KHS)

-Suspension and tag line systems

-Used to span hazardous ground

-Video of highline system failures

-Tensioning Guidelines

-Multiple of 12 guideline (for 11mm ropes)

-# of pullers x MA = or less than 12

(then rope tension will be less than 3 kn)

-2 Main Ideas: Logistics and Communication

-Logistics

1. Establish Pilot and Messenger Lines

-2X span for messenger line

2. Build end supports with high directionals

3. Ferry Ropes

-pull track rope across to fixed side (high strength tie off)

-pull back far side tag line (w/ prusik & 30kn biner, tag line hangers

every 10 meters)

4. Finish end station rigging

5. Inspect

6. Go operational

-KHS Comm.

-Leader

-Track Rope= Track (main)

-Tag Rope= Tag (belay)

-Control

-Edge

-Hoist

-Attendant

-Op. Procedure

1. Roll Call

2. Position Load

3. Pretension Track Rope

4. Load System

5. Attendant Needs

1030 hrs

-Field Objective: KHS with and without hoist

1700 hrs

-End of seminar wrap-up, questions, and suggestions