AV Team
How To Set Up A Basic Sound System
Accessed 30/03/2007

How to Set Up a Basic Sound System at AY Jackson SS
What You’ll Need
	Equipment

	Shure wireless microphone
	*No picture*

	At least one wired microphone
	*No picture*

	Denon dual-deck CD player and control panel
	[image: image1.jpg]



[image: image2.jpg]DINET S@@IE

=
>

&
|
=

:





[image: image3.jpg]




	Fostex 24-channel mixer
	[image: image4.jpg]HANNEL
WATT

000"
POWERED MIXER





*This is only a 6-channel mixer, but it is virtually the same as the 24-channel one*

	QSC amplifier
	[image: image5.jpg]



[image: image6.jpg]




	Setup anywhere other than caf: Yamaha speakers (2)
	[image: image7.jpg]



*The below speakers are the built-in speakers in the caf*

[image: image8.jpg]




	Cables

	1-socket extension cord (1)
	*No picture*

	6-socket power bar (1)
	*No picture*

	5-foot XLR cable (1)
	*No picture*

	50-foot XLR cable (at least 1)
	*No picture*

	RCA-to-quarter-inch cable (4)
	*No picture*

	5-foot quarter-inch cable (2)
	*No picture*

	Setup in caf: 50-foot Speakon speaker cable (2)
	*No picture*

	Setup anywhere other than caf: 50-foot quarter-inch speaker cable (2)
	[image: image9.jpg]



*The Speakon cables are virtually identical, except the ends are different – they match the adaptors in the next picture*

	Setup anywhere other than caf: Quarter-inch to Speakon adapter (2)
	[image: image10.jpg]





What You Need to Do

First, bring all the equipment and cables mentioned above and move them to the location that you are setting up the sound system. Start by plugging your extension cord into an outlet, and then connect the power bar to the end of the extension cord. Then you want to plug in the power adapter for the mixer into both the power bar and the mixer at the other end. Using the two quarter-inch cables, the next thing you want to do is to connect the main output from the mixer to the input on the amplifier, and then, depending on you location, connect the speakers either with the two Speakon cables or the two quarter-inch speaker cables and Speakon-to-quarter-inch adapters. The Speakon cables are used with the built-in speakers in the cafeteria, and the quarter-inch cables and adapters are used anywhere else with the portable speakers. Make sure that once you connect one end of the speaker cables to the amp, connect the other end to the speakers (or to the jacks in the wall, in the case of the caf). Then you want to connect all of your sources to the inputs on the mixer. Starting with the first channel, connect the wireless microphone receiver with the 5-foor XLR cable. Make sure that the wireless mic handset has a 9-volt battery in it. Then in each channel above channel 1, connect as many wired microphones as you need using the 50-foot XLRs. Then, using the four quarter-inch-to-RCA cables, connect your CD player. Start with the left channel of deck 1 (it is probably easiest to connect it to channel 21). Then connect the right channel of deck 1, and the left and right channels of deck 2 in that order. If you started with channel 21, you then should use 22 to 24 for the other connections. Now make sure that everything I connected to the power bar. Test the equipment, and when everything is ready to go, you’re ready to start the show.
Taking Down the Equipment

Once the show is done, you can take down all the equipment. But before you disconnect anything, make sure all of your power is turned OFF. Then you can disconnect everything, coil the cables using the proper coiling method (half-turn with every loop), and then bring everything back to the AV office and put it away.
GPP3O
Mrs. Quattrocchi - Period 1
Page 1 of 4

