APPLIANCE CONSUMPTION WORKSHEET by RV Solar Electric Inc.® 
Helping you make the right choice!  http://www.rvsolarelectric.com

| Appliance | Approx.Current (amps@12volts) | Hrs/day | Amp Hours Consumed |
| --- | --- | --- | --- |
| Lights Incandescent 1 bulb (25 watt) | 2 | X ______ | = ______ |
| 1 bulb (50 watt) | 4 | X ______ | = ______ |
| Quartz halogen (25 watt) | 2 | X ______ | = ______ |
| Fluorescent 1 tube (15 watt) | 1 | X ______ | = ______ |
| 2 tubes (30 watt) | 1.6 | X ______ | = ______ |
| Entertainment 9-in color TV | 3 | X ______ | = ______ |
| 12-in B&W TV | 1.1 | X ______ | = ______ |
| CB receiver | 0.5 | X ______ | = ______ |
| Stereo am/fm | 1 | X ______ | = ______ |
| Satellite receiver | 2 | X ______ | = ______ |
| Cooling/Heating 500 cfm fan | 1.2 | X ______ | = ______ |
| 750 cfm fan | 2.5 | X ______ | = ______ |
| 1000 cfm fan | 5 | X ______ | = ______ |
| 750 cfm fan | 2.5 | X ______ | = ______ |
| Forced-air furnace | 5-8 | X ______ | = ______ |
| Vent & range hood fan | 2 | X ______ | = ______ |
| RV water pump | 8 | X ______ | = ______ |
| DC compressor refrig | 6 | X ______ | = ______ |
| 3-way frig on 12 V | 35 | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |

| The AC appliances listed below require the use of an inverter. The AC amps have been multiplied by 10 to show the DC amp draw from the battery (for example, 5 amp AC = 50 amp at 12VDC). |  |  |  |
| --- | --- | --- | --- |
| Appliance | Approx.Current (amps@12volts) | Hrs/day | Amp Hours Consumed |
| Microwave oven | 125 | X ______ | = ______ |
| Blender | 15 | X ______ | = ______ |
| Computer | 4 | X ______ | = ______ |
| 13-in color TV | 7 | X ______ | = ______ |
| B/I vacuum | 100 | X ______ | = ______ |
| Electric broom | 60 | X ______ | = ______ |
| Hair drier 1200w | 95 | X ______ | = ______ |
| Satellite receiver | 3 | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| __________________ | ______ | X ______ | = ______ |
| Total amp hours used per day |  |  | = ______ |