______________________________________________________________________

Motor
      limit SW
       motor

      R down
          R up
       Power

    1

2

3
X

4

5
         6

________________________________________________________________________

To/from       power
      wiper/
 ground       front down         front  up
         wiper

Wiper


        switch




              retract switch

   7

8

9
   10

11

12

13

_____________________________________________________________________






 











a











 











b
c






d
x


e
y

f
f

        g

#1-3 Main leads to window motor.
#4-5  #11-12 Up and Down switches for window operation. Paths to ground

#2 = path to ground with window open. Closed circuit disables wiper when window is down .

#6 20 amp power feeds rear wiper and powers window via relays controlling polarity  for up and down function. The primary side of the relays is powered off #8 from the ignition circuit.

#7 switch lead for wiper...drops out when #2 closes to ground.

#8 ignition power. Operates primaries only on relays.

#9 Main power to wiper and washer?

#10 Main ground, for relays and 20 amp circuit to window motor.

#13 Wiper retract switch creates path to ground and disables window when wiper is running

I have had some spells with my rear window. Part of it was damaged wires between the tail gate and the main harness inside the left rear fender. The whole tailgate harness has been cut and spliced at some time before I purchase the truck and I will always be leary 

of that bundle. 

I have had further problems with the main relay assembly.

 For whatever reason, the interior switch portion of the relay  no longer operates the window. I removed the two associated wires from the gang plug at the relay and spliced them to the two wires from the rear switch. It now works fine, even with the key "off". In the meantime, I went through enough that I feel I know the window wiring by heart. With help from Donald Tong, I put together a replacement unit. Its not as compact as the OEM unit, but its less than $50 versus $270 for the Toyota part. 







With 12 volt test light clipped to ground (wire #10 on gang plug), apply probe to diodes (oval symbols) #d -#g on right hand side. These should raise and lower the window. Test for power at # 6 and #8 at wiring connector. Test again at x & y. 














