Fulton County Department of Health and Wellness

Environmental Health Services 

Individual Septic Tank Construction Checklist

	1
	
	site approval performed, permit less than 12 months old

	2
	
	layout conforms to site approval drawing

	3
	
	contractor=s state certification number: ______________________

	4
	
	fee paid, if reinspection necessary - fee required -

	5
	
	linear feet of repair area remaining = the initial system

	6
	
	tank from an approved manufacturer - no cracks - tee, filter, etc. intact

	7
	
	tank > 50' from bored well or suction water line

	8
	
	tank > 25' from drilled well, lake, pond stream, water course

	9
	
	tank > 10' from P/L or pressure water line, building, pool wall (50’ from individual water supply)

	10
	
	tank > 15' from drainage ditch, wall, drainage ditch, wall, retention pond, illegal trash pit

	11
	
	tank not on fill dirt - firm foundation

	12
	
	tank installed for ready access, min.-12" of cover, max.- 24"

	13
	
	1 - 3 bdrms-1000 gallon tank, 4-5 bdrms.-1500 gallon tank, 250 gallon increase/ea. add. bedroom

	14
	
	tank > 15' from draw, embankment/retaining wall (uphill)

tank > 5" from pavement, retaining wall (downhill)

	Dosing Tank

	15
	
	dosing tank if trench > 500'

	16
	
	capacity 60-75% of volume of absorption lines

	17
	
	alternating siphons required if trench > 1000'

	Distribution Box

	18
	
	location - same as 8,9,10,11,12,13

	19
	
	concrete foundation extends 6" on all sides and level

	20
	
	lines terminate in box with submerged elbow

	21
	
	field not in fill dirt unless it is acceptable by compaction testing

	22
	
	field > 100' from bored well, springs, suction water line

	23
	
	field > 10' from pressure water lines or foundation, pool walls, property lines

	24
	
	field > 50' from drilled well or shore line

	25
	
	field > 50' from property line if wells are proposed or exist on adjacent property

	26
	
	not in flood plain or areas of ponding or runoff

	27
	
	benched if slope > 20%

	28
	
	no part of system covered by parking, buildings, etc.

	29
	
	> 30" deep and 12" of cover, < 48" deep

	30
	
	> 50' in length

	31
	
	lines at least 10' apart

	32
	
	> 5' to tank

	33
	
	perforations placed downward 

	34
	
	solid line not counted as drainfield length

	35
	
	minimum 6" gravel under line

	36
	
	minimum 2" gravel over line

	37
	
	approved cover on top of gravel

	38
	
	trench bottom and walls not smeared and minimum 36" wide

	Serial Distribution Method

	39
	
	trenches level and following contours

	40
	
	1st overflow invert at least 4" below tank outlet invert

	41
	
	undisturbed earth under overflow

	42
	
	overflow must be solid pipe

	43
	
	minimum 12" cover over overflow

	Absorption Pit

	44
	
	used only for repair, washing machine, bathroom not able to be served by main system

	45
	
	utilized with minimum 1000 gallon septic tank

	46
	
	residential - volume of aggregate = volume of conventional line - 100'/bedroom

	47
	
	commercial - volume of aggregate = peak sewage flow/day x 2, then 7.5 x100

	48
	
	4' minimum depth

	49
	
	aggregate covered with approved material

	50
	
	12" minimum soil cover over pit

	51
	
	located same as 22,23,24,25,26,28,39,35,41

	Sewage Lift Pump

	52
	
	separate pump chamber

	53
	
	pump controls: level control switch, warning buzzer, and/or light

	54
	
	vent

	55
	
	manhole cover

	56
	
	emergency bypass to approved absorption pit

	57
	
	piping and distribution box to an approved absorption field


FCDH&W, Environmental Health, Individual Septic Tank Construction Checklist - Page 1 of 3

