Description

Make:

Chevrolet

Model:

Cavalier

Trim:

LS Convertible 2D (luxury sport convertible two door)

Year:

1996

Color:

Manta green

Engine:
2200 (GM 122)

L4, OHV, MFI, 2.2 liter
110 hp (82 kW), 130 ft-lbs

Drivetrain:
Front wheel drive
4-speed automatic (electronic)
1st 2.96 || 2nd 1.63 || 3rd 1.00 || 4th 0.68 || R 2.13 || FD 3.63

Mileage:
141,000, with 21,000 less on motor (September 2007)



Bought car with 127,000 in April 2006

Equipment:

· Luxury

· Air conditioning

· Cruise control

· Power windows, locks, and convertible top

· Tilt wheel

· Alloy wheels

· Rear spoiler

· Sound system

· Sony head-unit (CD and AM/FM radio)

· Four performance Pioneer speakers

· Safety

· Antilock brakes (all 4 wheels)

· Dual front airbags

MSPR was $17,500
MPG: 33 highway, 23 city

Current Inspections

· Virginia Emissions (good until April 2008)
· Virginia Safety (good until June 2008)

	Maintenance History
	Work needed

	Engine

· Oil changes

· Oil change every 3,000-4,000 miles (August 2004 to April 2006)

· Oil and filter changed with 10W-30 and Fram filter (May 2006)

· Oil and filter changed with 10W-30 and Fram filter (August 2006)

· Oil and filter changed with 5W-30 and Purolator Premium Plus filter (November 2006)
· Oil and filter changed with 5W-30 and Purolator filter (March 2007)

· Oil and filter changed with 5W-30 and Purolator filter (July 2007)

· Whole engine replaced with junkyard one (August 2004)

· It seized when driving the car home from buying it, because it had been sitting and low oil

· New motor had about 21,000 less miles on it than the car

· Serpentine belt replaced with new one (April 2006)

· Valve cover gasket replaced; valve cover thoroughly cleaned (June 2007)


	Engine

· Oil leaks

· Rear main bearing seal (1 quart in 3 months; if oil is changed regularly, don’t have to add oil)


	Cooling, Heating, and Air Conditioning

· Cooling

· New coolant at engine replacement (August 2004)

· Decent amount of new coolant at water pump replacement (May 2006)

· Water pump replaced with junkyard one (May 2006)

· Heating

· Blower motor resistor replaced (August 2007)

· Air Conditioning

· Some new gaskets (2005)

	Cooling, Heating, and Air Conditioning

· Cooling

· Coolant bleeder screws are both broken (common problem… just bleed from radiator hose)
· Air Conditioning

· Compressor/clutch not working; check for leaks too


	Fuel, Exhaust, and Emissions Control

· Fuel

· Fuel injectors removed, inspected, and cleaned; one fuel injector replaced with junkyard one; new o-rings on all installed (November 2005)

· Air filter replaced with Purolator Pure One one (March 2007)

· Vacuum hose harness replaced with new one (May 2007)

· Fuel injector replaced in cylinder one with junkyard one (June 2007)

· Air intake plenum, IAC valve, and MAP sensor cleaned; new air intake plenum to intake manifold gasket (June 2007)

· Exhaust

· Exhaust pipes, catalytic converter, and muffler replaced (September 2005)

· Welded around catalytic converter to eliminate leaks from clamps (April 2006)
· New muffler (September 2007)

· Emissions

· PCV valve and hose replaced with Pure One PCV valve (March 2007)

	Fuel, Exhaust, and Emissions Control



	Engine Electrical Systems

· Sparkplugs replaced (2005)
· Starter replaced with rebuilt, heavy-duty one (2005)
· Sparkplug wires replaced with Duralast ones (June 2007)


	Engine Electrical Systems



	Transaxle and Drivetrain
· Fluid flushed and changed at engine replacement (August 2004)


	Transmission


	Brake System

· Front brakes
· Pads replaced with Bendix CT-3 ceramic ones (July 2006)

· Rotors replaced with Duralast ones (July 2006)

· Rear brakes
· Shoes replaced (2005)

· Drums replaced with Wearever ones, and some new hardware was installed; the shoes were not replaced as they were still in good condition (June 2007)
· ABS / ETS

· Wheel speed sensor wires replaced somewhere (2005)


	Brake System



	Suspension and Steering
· Alignment
· All four wheels (July 2006)

· Camber kit for suspension and shim-space kit for rear driver-side backing plate installed (July 2006)

· Suspension
· Ball joint replaced somewhere (2005)

· Wheel hub and bearing assembly replaced on front passenger-side (June 2006)

· Front struts replaced with KYB GR-2 ones (July 2006)

· Front strut mounts and bearing assemblies replaced with Monroe ones (July 2006)

· Steering

· Tires

· Four new Uniroyal Tiger Paw Touring H/V tires, P195/65R15 91H (July 2006, mileage 131k)
· Front driver-side tire leaking and plugged (August 2006)

· Rotated tires (January 2007, mileage 135k)
· Rotated tires (July 2007, mileage 140k)


	Suspension and Steering



	Body and Interior
· Convertible top

· Small cut on rear driver-side sealed (2005)

· Rear section of main top that connects to tacking strips repaired

· Trunk

· Power windows

· Driver-side window smashed out and replaced (pre-August 2004)

· Driver-side window motor replaced (pre-August 2004)

· Passenger-side window motor replaced with new one (May 2006)

· Driver-side window motor replaced with new one (April 2007)

· Driver-side window regulator holes on door body fixed and motor replaced (November 2007)

· Headlights

· Driver-side assembly replaced with junkyard one (June 2007)

· Side-view mirrors

· Both replaced with new ones (January 2007)

· Paint / Rust

· Filled and painted scratches in paint to avoid rust (2004)

· Sound system

· Replaced with four performance Pioneer speakers, Sony head-unit (2005)

	Body and Interior
· Convertible top

· Closing/Opening mechanism

· Driver-side goes down 95% (needs a push to go all the way)
· Driver-side rear window must be pushed outward when closing to fit over the weather-stripping
· Top has no leaks but is in fair condition (stitching coming undone, sealed cut, rear section repair)

· Trunk

· Hydraulic trunk supports are weak (trunk will not stay open by itself)
· Power windows

· Driver-side window has some minor scratches on it
· Passenger-side window goes a bit too high in the back, but not a big issue
· Headlights

· Passenger-side assembly has a broken adjuster screw
· Paint / Rust

· On the body, where driver-side door meets the body
· Rear left quarter panel

· Rear, under trunk gate
· Spare tire compartment (top used to leak into trunk)
· Some paint blemishes and scratches

· Other

· Crack in plastic panel beside radio (glued)
· Carpet and seats are in pretty good, but not perfect condition









