[image: image1.png]de

Holmesglen Training
and Development





Belt sanders

Belt sanders are commonly used for initial heavy sanding. They are used to remove coarse material, such as splinters and paint remnants, from the timber surface, and to smooth hardwood after you have finished working on it with other tools like saws and hammers. These sanders follow the grain of the timber. They have a spinning wheel which rotates a belt of sandpaper. The sandpaper loops are fitted from the side of the machine over the base. Once the loop is fitted a lever is pushed to centre the sandpaper on the base and to tension it to operating level. Belt sanders come in various sizes, and are sold by belt length and width. As they can be a little hard to handle, Scott recommends a 75mm wide x 457mm long sander for the home handyman. 

Always use eye protection when using a belt sander. You should also be aware of other safety tips.

(Tip: belt sanders must be held perfectly flat against the timber. If you lose your grip and do not hold the sander flat you can take small divots out of the timber surface. Scott suggests that you practise using your belt sander on a few pieces of scrap timber until you feel comfortable with it.)

Power drills

One of the most versatile tools you can own is the power drill. This is especially true if you get one with features such as variable speed and reverse. You want one with at least a 3/8" chuck. A 1/2" chuck will give you tremendous flexibility but is not generally needed for most home repair tasks.

If possible, get one with a keyless chuck. You'll never have to worry about keeping track of a little tool to tighten the drill bit into the chuck, it can be easily done by hand.

It’s a good idea to use eye protection when using a power drill. You should also be aware of other safety tips.

Jigsaws

The jigsaw (or sabresaw) is the first power cutting tool you should have in your toolkit. This is a very versatile tool that with some limitations on material thickness, can crosscut, bevel, miter and make scrolling curved cuts. Jigsaws use interchangeable blades depending on the material to be cut. This is a tool that can be used with one hand, and you will find it indispensable.

Always use eye protection when using this tool. See also safety tips.

Orbital sanders

Orbital sanders operate with an oscillating movement and are used for finer finishing work to remove any small imperfections. They are light and easy to handle, and are great for using on plaster and gyprock as well as timber. Orbital sanders with rectangular bases are the safest and lightest to use. Always use eye protection when using an orbital sander.

(Tip: if you use an orbital sander on wood the sanding plate may make a little mark on the timber. If this happens remove the mark by hand or with the belt sander. A fine sandpaper at low speed gives the smoothest finish.)

Eye protection

Eye protection is a requirement in any project where you may be susceptible to chemical splash or flying debris. Modern eye protection is comfortable and can even be fashionable.

Basic economy safety eye wear can be purchased for $2-$5 and fashionable safety eye wear between $3-$10.

Safety tips

· When using sanders always wear appropriate safety gear, such as a dust mask, clear safety glasses and earmuffs.

· Keep a firm, tight grip when operating a belt sander.

· Wear short sleeves or button up long sleeves, and wear long hair in a hair net or tied up. Keep ponytails well away from belt sanders!

· Keep sanders and other power tools in a locked cupboard, well out of the reach of children.

	[image: image2.png]H |
|
HOLMESGLEN

INSTITUTE OF TAFE




	1


F:\Text about tools.doc  4-Jun-07

[image: image1.png][image: image2.png]