









D35 Bonanza   (BE-35D)                            N3463B





BEFORE STARTING ENGINE


Preflight			Complete


Seat Belts		Adjusted


Doors			Secure


Passengers		Briefed


Fuel Selector + gauge	Fullest (L)


Cowl Flaps		Open


Avionics			Off


Circuit Breakers		Check / In


Brakes			Hold


Ignition			On


Battery 			On


Alternator		On


Landing Gear		Green


Watch Timer		Start





    





AFTER STARTING – IFR


ATIS			Copy


Altimeter		Set


VSI			Zero


Alternate Static		Increase


Marker Beacons		Test / Set


COM Radios		Set


NAV Radios		Set / Ident


ADF			Set / Ident


Clearance		Copy


Transponder		Set Code


Charts			Sequence





TAXI – IFR


Mag Compass	      Correct Heading


		        Turns Opposite


DG         	     	       Turns Correctly


Turn Coordinator             Skid in turns


Attitude Indicator              Stays Stable





STARTING ENGINE


Throttle		            Open 2 turns


Mixture			Rich


Propeller			High RPM


Wobble Pump		10-15 psi


CLEAR!


Starter Button	            Prime if cold


Throttle			1000 RPM


Oil Pressure		Check


Mixture			Lean


Avionics			On


Nav / Strobes		On


    





GEG Frequencies


GEG ATIS		124.325


GEG CLNC DEL		127.55


GEG GND		121.9


GEG TWR		118.3


GEG APP/DEP  	   133.35, 123.75


GEG VOR		115.5


GEG ILS		111.1





STARTING ENGINE – FLOODED


Mixture			Idle cut-off


Throttle			Full Open


Starter Button		Engage


Mixture		Rich w/engine start


Throttle			1000 RPM


Oil Pressure		Check


Mixture			Lean


Avionics			On


Nav / Strobes		On 


    


    





RUNUP


Flight Controls	               Free/Correct


Trim			Set


Fuel Selector + gauge	Fullest  (L)


Mixture			Rich


Throttle			1900 RPM


    Prop			Cycle


    Carb Heat		Check


Throttle			1600 RPM


    Magnetos		Check


    Suction		4.9” – 5.1”


    Engine Instruments	Check


    DG			Set


Throttle			1000 RPM


Autopilot		Test


Mixture			Lean


Windows		Closed











BEFORE TAKEOFF


Flaps			Set


Mixture			Rich


Pitot Heat		As  req.


Landing Lights		On


Transponder		On / Alt


TAKEOFF


Time			Note


DG / Engine Gauges	Verify





AFTER TAKEOFF (CLIMB)


Landing Gear		Retract


Flaps			Retract


Power			25”, 2500


Landing Lights		Off











SHORT FINAL


Landing Gear	                Down


Flaps			Set


Prop			High RPM


Mixture			Rich





CRUISE


Cruise Power		Set


Mixture			Set


Cowl Flaps		Closed


Gauges			Check





PRE-APPROACH


Beacons			Test / Set


ATIS		Copy, Set Altimeter


		Winds, IAP, Mins?


Radios			Set


    COM Radios		Sequence


    NAV Radios  	     	Ident


                               Set OBS, Sequence


DG			Set


Charts, Course		In Order


Seat Belts		Secure


Fuel Selector + gauge	Fullest


Mixture			Set





			





APPROACH


Timer, Throttle, Talk	Execute


Landing Check


    Cowl Flaps		Closed


    Carb Heat		On


    Gas (Fuel Selector)	Fullest


    Undercarriage	                Down


    Mixture		Rich


    Prop			High RPM


    Seat Belts		Secure


Altitudes		                MDA, DH


Missed Approach	Reviewed


Circle To Land		Reviewed


Landing Lights		On





AFTER LANDING


Flaps			Retract


Mixture			Lean


Cowl Flaps		Open


Landing Lights		Off


Pitot Heat 		Off


Transponder		Standby





SHUTDOWN


Avionics			Off


Mixture			Idle Cut-off


Lights			Off


Battery + Alternator	Off


Ignition			Off


Mixture			Rich





AFTER SHUTDOWN


Controls			Secure


Vents / Windows		Close


Aircraft			Secure


Hobbs / Tach		Record


Transponder		1200


FLIGHT PLAN		CLOSE





CLOSE FLIGHT PLAN!





N3463B Performance


Lift-off			73 mph


Vx			79 mph


Vy			104 mph


Vle			127 mph


Vfe			106 mph


Va			131 mph


Best Glide		121 mph


Cruise Climb 


  25”,  2300 RPM,  130 mph


High Speed Let Down


	  20”,  2000 RPM,  160 mph


Descent to Minimums


	  17”,  2300 RPM,  100 mph


Level Off at MDA


	  20”,  2300 RPM,  90 mph





Frequencies


SEA FSS	    122.65 122.55 122.2


Flight Watch 		122.0


Emergency		121.5 











