/* Galactical2.nqc


For a robot named Galactical Grockelbot. It is a line

following robot that turns by running a front wheel.*/

// Sensor(s) and Motor(s)

#define Eye

SENSOR_1

#define Driver
OUT_A

#define Turn
OUT_C

// Thresholds

#define Line  37

#define White 55

// Constants

#define STRAIGHT_TIME 50

task main()

{


SetSensor(Eye,SENSOR_LIGHT);


On(Driver);

  SetPower(Turn,3);


while(true){

   
if(Eye >= White - 2)

    { 


   
OnFwd(Turn);

   
until(Eye < White -5);

    Off(Turn);

    }

    else if (Eye <= Line + 2)

    {

   
OnRev(Turn);

   
until(Eye > Line +5);

   
Off(Turn);

    }

    else

    {

   
Off(Turn);

   
}


  }

}

