Multi Media Microcomputer Hardware – Prac Test 3
1. What does the term ‘lossy compression’ refer to?


2. What 2 factors determine how accurately digital data represents analog data?


3. Describe the methodology used to convert analog data to digital data?



4. MPEG is used to compress what type of data? Describe the compression technique.




5. Compare a DAC to an ADC. Where would you find these components?


6. Compare the two methods used to synthesise sound, FM and Wavetable.



7. What must be true before MMX and SSE technology can improve multimedia performance on a PC?


8. When sound is digitised, what is the number of bits used to hold each sample called?

9. What unit of measure is used to express the sampling rate of a sound card?


10.The purpose of a microphone is to convert





sound to 


voltage, which is then passed to a sound card.

11. Sound samples recorded on a single channel are called




, on two channels are called



 
and on six channels are called





.

12. What is the sampling rate of music CD’s (in Hertz)?

13. What are the two most common file extensions for sound files?

14. What two things does a DVD controller decode? What compression methods are used for both?

