Installing the Dash Door Lock Switch 

[image: image1.jpg]



1. Find the Black/Red wire in the interior portion of the door harness. 
	Quote, originally posted by GTI-VR6.net »

	Under the dashboard, right behind the hood release lever, you can see the
wiring to the driver's door connector. There are several single row,
multi-pin connectors snapped onto the back of the big round connector
that mates with the door's connector. Fortunately, the one that I wanted
was right in front of my nose. I snipped the second wire from the bottom
on that 5-pin connector (black/red 20 AWG) as close to the plug as possible.


2. Cut the wire. Leave the part that goes into the door disconnected. Run a new wire from the center console to the newly cut wire. Connect the new wire to the cut wire. 

3. There are 5 wires on the door lock switch. Numbered according to the harness: 

	Quote, originally posted by GTI-VR6.net »

	
1: Brown - Ground
2: Red/White - +12V
3: Grey/Green - Magic Lock Wire
4: (empty)
5: Brown - Ground
6: Grey/Blue - Illumination


Splice the grounds to the cigarette lighter ground. Splice the +12V to the cigarette lighter +12V. Splice the Magic Lock Wire to the new wire you ran from the door harness to the center console. Splice Illumination from another illuminated switch (defroster). Use Wire Tap-ins for simplicity. 

4. Enjoy your lock switch.

Installing the Dash Door Lock Switch   




Page 2 of 2

