Motor Mount DIY

Does your car vibration heavily when you accelerate up on-ramps.  If you front motor mount has not been replaced in 100,000 plus miles, it may be the culprit.  Or during periods of higher engine torque like starting at stoplights, it makes a growling sound and loses power.  Your top bolt of the front motor mount could have sheared off.
Time:  About 45 minutes 

Put the front of the car on jackstands. 

Removing the top bolt: If you have a pair of socket wrench extensions about 18" long, it will be a lot easier to remove the top bolt.
[image: image1.jpg]


[image: image2.jpg]



Before trying to remove the top mount: Unscrew your oil filter before trying to wiggle the mount out. Gives you much more room to maneuver the top of the mount out towards the oil filter space. 
[image: image3.jpg]



[image: image4.jpg]



Removing Lower Bolt:  The remove the lower Allen headed bolt.

To safely raise your engine off of the mount:  Jack up the engine.  Use a 12 inch 2x12 board under your oil pan with a scissor jack to evenly raise your engine off of the subframe so you can get that 4" to get the old mount out. The wider board will even distribute the weight of the engine on the pan and avoid crushing or ruining your pan. 

Torque Specs: Bentley states both top 9/16" bolt and the large hex nut on the bottom of the mount should both be torqued to 41 foot pounds.

