ARRL NET Q SIGNALS

QNA*
Answer in prearranged order.

QNB*
Act as relay between _____ and _____

QNC
All net stations copy. I have a message for all net stations.

QND*
Net is directed (controlled by net control station).

QNE*
Entire net stand by.

QNF
Net is free (not controlled).

QNG
Take over as net control station.

QNH
Your net frequency is high.

QNI
Net stations report in.*.


I am reporting into the net. (Follow with a list or traffic or QRU.)

QNJ
Can you copy me?


Can you copy _____?

QNK*
Transmit message for _____ to _____

QNL
Your net frequency is Low.

QNM*
You are QRM-ing the net. Stand by.

QNN
Net control station is _____


What station has net control?

QNO
Station is leaving the net.

QNP
Unable to copy you. Unable to copy _____

QNQ*
Move frequency to _____ and wait for _____ to finish handling traffic. Then send him traffic for _____

QNR
Answer _____ and receive traffic.

QNS*
Following stations are in the net.* (Follow with list.)


Request list of stations in the net.

QNT
I request permission to leave the net for _____ minutes.

QNU*
The net has traffic for you. Stand by.

QNV*
Establish contact with _____ on this frequency. If successful, move to _____ and send him traffic for _____

QNW
How do I route messages for _____?

QNX
You are excused from the net.* Request to be excused from the net.

QNY*
Shift to another frequency (or to _____ kHz) to clear traffic with _____

QNZ
Zero beat your signal with mine.

* For use only by the net control station.

