Providing a Triple Play over Existing Telephone Lines

Kevin DeMartino
 Independent Consultant 

kevindemar@verizon.net
Abstract
This paper describes how voice, video, and data communications, the so-called "triple play of communication services, can be provided over existing telephone lines. The approach for supporting these services is based on the use of very-high-speed digital subscriber line (VDSL) technology. To enable the high data rates required for these services, fiber optic cables must be run from central offices (COs) to remote terminals in the neighborhoods of the subscribers. These remote terminals demultiplex signals from the COs onto twisted pair lines connected to the subscribers and multiplex signals from the subscribers onto fiber optic lines connected to the COs. To accommodate a large number of broadcast audio/video channels, a capability to select and switch these channels would be provided in the remote terminals. Upgrades in the interoffice network to support the triple play capability are also described. A converged transport network based on multi-protocol label switching (MPLS) is presented. Transport of data associated with the various services would be similar and common network elements and protocols would be employed.   

Introduction to the Triple Play of Communication Services

Communication services are usually categorized as: voice (telephony); video, which includes video-on-demand and broadcast video (and associated audio); and data communications, i.e., computer communications. It is highly desirable to be able to provide a complete set of voice, video, and data communication services over a common, integrated communication network. This capability is commonly referred to as the "triple play" of communication services. 

Telephone networks, which were originally designed to support voice, can also support video and data. However, compared to coaxial cable networks, telephone networks are at a significant disadvantage with respect to broadband access.

In most cases, a telephone network subscriber is connected to a telephone central office (CO) by a pair of copper wires that are twisted together to reduce electromagnetic coupling to other wires. For typical distances between subscribers and the central offices, the maximum bandwidth of the signals that can be transmitted over twisted pair lines is limited to approximately 1 MHz. On the other hand, coaxial cable can support a bandwidth of approximately 1 GHz, which is a factor of 1000 greater than the twisted pair bandwidth. This bandwidth advantage for coaxial cable is reduced somewhat by the fact that a twisted pair is dedicated to an individual subscriber a while coaxial cable is typically shared by many subscribers. However, this not enough to overcome the coax bandwidth advantage, and having a shared medium is actually a significant advantage for providing broadcast services.

The ultimate solution to the limitation imposed by twisted pair access lines is to replace these lines with fiber. This is the approach being pursued by Verizon Communications. However, running fiber all the way to the subscriber premises, i.e., fiber-to-the-premise (FTTP), is very expensive. It would be much less expensive if existing twisted pair access lines could be used to support the desired services. This paper describes an alternative to FTTP that can be used in areas where FTTP is not cost-effective. Verizon has a patent pending for the methods and systems described in this paper.

Digital subscriber line (DSL) techniques employ bandwidths that are much wider than the standard voice channel bandwidth (4 KHz), which enables data rates that are much higher than the rates achievable on 4 KHz subscriber lines. With asymmetric DSL (ADSL), it is possible to achieve downstream data rates up to 6 Mb/s over twisted pair lines up to 12000 ft in length [1]. However, in many cases, line impairments limit the reach and data rate so that the ADSL rates for most subscribers are considerably less. Over most subscriber lines, ADSL data rates are adequate to support voice, relatively high speed Internet access, and limited audio/video-on-demand. However, ADSL rates are not adequate to support broadcast video.

DSL data rates can be greatly increased if the lengths of twisted pair subscriber lines are shortened by placing remote terminals in the subscriber neighborhoods and connecting these terminals to a CO via fiber optic lines. This approach is referred to as very-high-speed DSL (VDSL) and fiber-to-the-neighborhood (FTTN). If the remote terminal is within 3000 ft of the subscribers, downstream (i.e., network to subscriber) data rates up to 26 Mb/s are achievable [2]. VDSL can easily support video-on-demand, including multiple National Television Standards Committee (NTSC) channels or a high definition television (HDTV) channel. However, by itself, VDSL can support only a very limited broadcast capability. VDSL by itself cannot support an audio/video capability comparable to that provided by cable TV or by over-the-air TV systems. 

To enable a robust broadcast capability, a mechanism to select and switch broadcast audio/video channels must be provided to effectively exploit the access line capacity enhancements provided by VDSL. With this mechanism, the subscriber would be able to select a few channels from a menu, and these channels would be switched onto the subscriber's access line at the remote terminal.

Although upgrading the access network is the key to support a triple play capability, the interoffice network must be addressed as well. Interoffice networks have the potential capacity to support triple play services. However, this capacity needs to be further developed and utilized more efficiently. A more integrated approach for transporting data associated with the various services is needed.

A paper addressing VDSL and FTTN was published in a previous IEC comprehensive report [3]. This paper is a follow-up paper that expands on the material presented in the previous paper and addresses a broader range of issues. The following section addresses access networks, particularly, the limitations of current access networks and the capabilities and limitations of DSL technologies. Subsequent sections address topics related to the proposed FTTN access network, the services that are enabled by this network, and the transport of data associated with triple play services. Issues that are addressed  include: the access network topology, the operation of the remote terminals, and the method for the selecting and switching audio/video broadcast channels, various service options, common network elements and protocols supporting the end-to-end transport of triple play data.

Access Networks - The Key to the Triple Play
A telephone network can be viewed as being composed of  access networks, which connect subscribers to central offices (COs), and an interoffice network, which interconnects the COs. Figure 1 presents a simplified view of a telephone network on a small scale, with only a few COs and a small number of subscriber access line per CO. A relatively large telephone network may have thousands of COs and several thousand (or many thousand) access lines per CO. The number of access connections is typically more than 3 orders of magnitude greater than the number of interoffice connections, which implies that upgrading the access network for broadband operation would be much more expensive than upgrading the interoffice network. As a result, interoffice networks have been converted over to fiber optics, while access lines are still mostly twisted pairs.


[image: image1.wmf]CO

CO

CO

CO

CO

CO

To CO

To CO

To CO

To CO

To CO

Interoffice

Trunk

Interoffice

Trunk

Interoffice

Trunk

Interoffice

Trunk

Access

Lines

Access

Lines

Access

Lines

Access

Lines

Access

Lines


Figure 1 - Small Scale Telephone Network

In a typical interoffice network, the COs are interconnected by fiber optic cables containing multiple fibers. Each fiber supports multiple Synchronous Optical Network (SONET) channels [4], which are time division multiplexed. Each fiber may also support multiple wavelength division multiplexed (WDM) channels, which in turn, support multiple SONET channels. As shown by Figure 1, an interoffice network typically has a mesh topology [5]. However, a pattern of interlocking SONET rings [6] is superimposed on the underlying mesh so that data flows are constrained to follow these rings. On a SONET ring, data can flow in two directions, which enables a data flow to be quickly re-established if a failure occurs along the ring. A previous paper [7] described a mesh-based architecture for interoffice networks that provides greater efficiency, flexibility, and robustness than an architecture based on interlocking SONET rings.
Regardless of whether the interoffice network is ring-based or mesh-based, it must have sufficient capacity to handle the data flows to and from the access network. Currently, network throughput is usually limited by the capacity of the access networks. This paper addresses how the capacity of these access networks can be greatly increased, and used more effectively. To support the increased data rates through the access network, the capacity of the interoffice networks must be increased accordingly. However, as discussed in section 4, this is easier and less expensive than updating the access network.

Current Access Networks

Currently, most subscribers are connected to a class 5 CO (an end office) by a pair of copper wires that are twisted together to reduce interference.  In some cases, twisted pairs are terminated in a remote terminal in the subscribers' neighborhood, where signals on multiple subscriber lines are digitized, multiplexed onto a fiber, and sent to the end office. This latter type access is referred to as a digital loop carrier (DLC) system. End offices in the Verizon's domestic wireline network support an average of approximately 12000 subscriber access lines [8], which include twisted pairs from subscriber premises to the CO and DLC lines (each supporting multiple subscribes) from remote terminals. Large business (enterprise) customers may have fiber optic access lines running from the CO to the customer premises as part of a SONET access ring.

Until fairly recently, the bandwidth on most twisted pair lines was limited to 4 KHz, which limited the equivalent data rate to 64 Kb/s or less. The Integrated Services Digital Network (ISDN) basic rate interface (BRI) supports a data rate of 144 Kb/s over a twisted pair access line [9]. With digital subscriber line (DSL) techniques, which are discussed in the next two subsections, the bandwidth is greatly expanded beyond 4 KHz and data rates in the megabit per second range can be supported over twisted pair access lines. With the data rates provided by DSL, high speed Internet access and audio/video-on-demand can be supported.

In spite the expanded capabilities provided by DSL, current access networks greatly limit network services. The data rates that can be supported by DSL fall off rapidly as the 

length of the twisted pair access lines increase. Thus, high DSL data rates can be supported only for subscribers who are relatively close to the CO. As  discussed in a later section, the use of remote terminals in the subscribers' neighborhood allows high data rates to be supported for subscribers located relatively far from the CO. However, the introduction of remote terminals is insufficient to enable robust broadcast audio/video services.

DSL Capabilities and Limitations

The term digital subscriber line (DSL) refers to a set of techniques that enable high data rates to be transmitted over twisted pair access lines. Although there are a number of flavors of DSL, two are of primary interest here, asymmetric DSL (ADSL) and very-high-speed DSL (VDSL). ADSL may involve relatively long twisted pair access lines, running all the way from the subscriber premises to the CO, with a relatively wide bandwidth assigned for downstream traffic (CO to subscriber) and a relatively narrow bandwidth assigned for upstream traffic (subscriber to CO). 

With ADSL it is possible to achieve data rates of up to 9 Mb/s for twisted pair access lines up to 9000 ft in length [10]. In practice, various impairments splices, reduce the data rates that be achieved with ADSL. Over most of the access lines in the U.S., ADSL can provide downstream data rates that can support high speed Internet access for client users, who require relatively high downstream data rates, but can tolerate low upstream data rates. Servers, which require high upstream data rates, are not supported by ADSL, as currently implemented.

With video compression techniques, acceptable quality video-on-demand can be provided over ADSL lines that can support a downstream data rate of approximately 1.5 Mb/s [11]. Approximately 70 % of the access lines in the U.S. have the potential to support this ADSL rate [12]. For higher quality video, or two or more video channels, ADSL rates in the range 3 to 6 Mb/s are required [13], which can be supported on approximately 50 % of the lines in the U.S. High definition television (HDTV), which requires a data rate of approximately 20 Mb/s [14], even with a high degree of compression, cannot be supported over ADSL access lines.

As the name implies, symmetric high-rate DSL (SHDSL) supports the same data rate in both directions. Although SHDSL is not well suited for supporting the triple play, SHDSL systems employ various techniques that could be used to enhance the ability of ADSL to support video. For example, inverse multiplexing of signals over multiple twisted pairs [15], which is referred to as twisted pair bonding, allows the data rate to be increased at the cost of requiring additional twisted pairs for a subscriber. Also, spectral shaping and interference canceling techniques are employed to minimize interference among twisted pairs in the same bundle. A new SHDSL standard [16] (g.shdsl.bis) for supporting 5 Mb/s over a single twisted pair (with certain restrictions) is being developed. 

Techniques similar to the ones described above can be applied to ADSL, which would enable ADSL to accommodate HDTV, but at a price. Additional twisted pairs would be needed and additional costs incurred, which would limited the number of  subscribers that could be supported. Even with these enhancements the capability would fall far short of the broadcast capability  provided by cable TV systems, which can support hundreds of audio/video broadcast channels.

In summary, the type of triple play capability can be provided by the cable TV companies cannot be supported by DSL as currently implemented. To support a triple play capability, we must move beyond the current DSL approach and take a different tack. 

VDSL and Fiber-to-the-Neighborhood

VDSL is similar to ADSL except that VDSL involves relatively short access lines, which run from the subscriber premises to a remote terminal in the subscribers’ neighborhood. This enables VDSL to support much higher data rates than ADSL. Usually, VDSL is also asymmetric, with the downstream bandwidth and data rates higher than the upstream bandwidth and data rates. However, VDSL data rates can be symmetric.
As the name implies, very-high-speed digital subscriber lines (VDSL), can support much higher data rates than ADSL, or other types of DSL. With VDSL, twisted pair access lines from subscriber premises are terminated in a remote terminal in the subscribers' neighborhood. This remote terminal is then connected to a CO via fiber. This approach greatly shortens the length of the twisted pair lines and enables VDSL to support high data rates.

Table 1 shows the downstream data rates that can be achieved using VDSL as a function of the length of the twisted pair access line, or the distance from the subscriber premises to the remote terminal [17].

	Length of TP Access Line
	VDSL Downstream Data Rate

	1000 ft
	52 Mb/s

	3000 ft
	26 Mb/s

	4500 ft
	13 Mb/s


Table 1 - Achievable VDSL Rates Vs Length of Twisted Pair Access Lines
For an access line length of 3000 ft, VDSL can support high quality video and even HDTV. A data rate of 26 Mb/s can support multiple video channels, and could support a limited audio/video broadcasting capability. More recent publications [18,19] indicate that the rates in Table 1 may be achievable over somewhat greater distances.

VDSL can support a limited triple play capability. It can support voice, data communications, and video including HDTV. However, VDSL by itself cannot support the robust audio/video capability provided by the cable TV systems. A competitive triple play capability requires something beyond VDSL.

An FTTN Access Network That Enables the Triple Play
The fiber-to-the neighborhood (FTTN) access network described in this section provides much higher data rates than current access network at a cost that is much lower than fiber-to-the-premise (FTTP). The combination of the high subscriber data rates supported by the access lines and  the switching capability provided by remote terminals in the access network enables the network to provide a full range of communication services.

Subscriber Connections


As discussed in the previous section, high data rates over twisted pairs imply short access lines. Interpolation of the numbers in Table 2 indicates that data rates approaching 30 Mb/s can be supported if the length of the twisted pair access lines is half a mile of less. Figure 2 shows a remote terminal in the subscribers' neighborhood connected to 200 subscriber nodes, in a hub and spoke configuration. This terminal services an area with a diameter of 1 mile. In sparsely populated areas, the service area of the remote terminal would remain the same, however, the terminal would be connected to fewer subscribers. In densely populated areas, a remote terminal would connect to more subscribers over a smaller service area.

A remote terminal  would be connected to two COs via fiber optic lines. This enables protection against certain failures. If the fiber optic line connecting the remote terminal to the primary CO (primary end office) is cut, then the remote terminal would switch over to the secondary CO (secondary end office). Likewise, the remote terminal could switch over to the secondary CO if all or parts of the primary CO fail. Since there is only one twisted pair line between a subscriber node and the remote terminal, access lines are not fully protected. Subscribers requiring very high reliability could be connected to two remote terminals to provide comprehensive protection against access network failures.


[image: image2.wmf]Remote

Terminal

Fiber

To CO

To CO Via 

Remote

Terminal

Fiber

1 Mile

Diameter

Circle

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

TP

S1

S2

TP

S200

TP

Legend

.

S1

–

S200  

-

subscriber node

TP 

-

twisted pair line

fiber line to CO


Figure 2 - Remote Terminal with VDSL Access Lines


With the approach illustrated by Figure 2, existing twisted pair lines drop lines (from the subscriber premise to the curb) and distribution lines (from the curb) to the remote terminal would be utilized. Remote terminals would be placed at splice points where hundreds of twisted pairs connected to subscribers come together to be connected to the CO via feeder cables. The fiber optic lines from the remote terminals to the COs would replace existing feeder cables, each containing hundreds of twisted pairs. With this approach, the number of new lines is relatively small (on the order of 100 per CO), where with the FTTP approach, the number of new lines is much higher (on the order of 10000 per CO).

Access Network Topology


Subscriber nodes are unevenly distributed in two-dimensional space. The distribution of remote terminals would generally follow the population distribution. Since the number of subscribers supported by a remote terminal would be larger in heavily populated areas than in lightly populated areas, the distribution of remote terminals would be more uniform than the population distribution. Figure 3 shows the topology of remote terminals connected to COs as part of the access network. The remote terminals are distributed on a two-dimensional grid, which is not necessarily rectangular. Up to a few hundred existing subscriber lines in distribution cables would be connected to a remote terminal. Since these distribution cables are generally routed along roadways, remote terminals would also be placed along the roadways, either on utility poles or on the ground.


[image: image3.wmf]RT

CO

CO

CO

RT

RT

RT

RT

Legend

CO 

-

central office

RT 

–

remote terminal

To CO

via RTs

To CO

via RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT


Figure 3 - Connecting Remote Terminals to COs

Fiber optic cables containing multiple fibers would be run the COs to the remote terminals. At a remote terminal, one or more fibers would be dropped from the cable and the remaining fibers would be carried to other remote terminals along the grid. Generally, a fiber would pass through (or by) multiple remote terminals. Fibers that interconnect COs may follow the same routes as the fibers connecting COs to remote terminals. However, the interoffice cables carrying these fibers would be separate from the cables carrying the fibers connecting to the remote terminals.

Remote Terminal Operation

Figure 4 is a block diagram of a remote terminal, which interfaces with subscribers on one side and COs on the other side. Each remote terminal is connected to a primary and a secondary CO. If a there is a failure on the line connecting the remote terminal to the primary CO or a relevant failure at the primary CO, then signals from subscribers are switched onto the line connected to the secondary CO, and signals from the secondary CO are switched to onto subscriber lines. With wave length division multiplexing (WDM) [20], a single fiber can be employed to connect a series of remote terminals to a CO. In this case, 8 remote terminals are connected to the primary CO by a single fiber with 9 wavelength channels, 1 for carrying broadcast signals to the 8 remote terminals, and 8 for carrying individual subscriber signals to and from the 8 remote terminals. Another single fiber with 9 wavelength channels can be used to connect the eight remote terminals to the secondary CO. If more wavelength channels are provided, each of these fibers can support more connections between a CO and more than 8 remote terminals.

[image: image4.wmf]Optical

ADM

.

.

.

TP

26 Mb/s down

500 Kb/s up

To/From

User

#1

To/From

User

#200

Multiplexing

&

Distribution

VDSL

I/F

#1

To/From

Adjacent

RT/ CO

To/From

Adjacent RT

Optical

ADM

To/From

Adjacent 

RT/ CO

To/From

Adjacent RT

Fiber

Fiber

TP

26 Mb/s down

500 Kb/s up

VDSL

I/F

#1

ADSL

I/F

ADSL

I/F

1.5

-

6 Mb/s

500 Kb/s

9

-

24.5 Mb/s

Video

Switch

ADSL

6 Mb/s max

Video

2.5 Gb/s

Optical

Coupler

2.5 Gb/s

620 Mb/s

l

1

l

B

2.5 Gb/s

Optical

Coupler

Fiber

Fiber

620 Mb/s

2.5 Gb/s

l

1

2.5 Gb/s

l

B

Video

Switch

9

-

24.5 Mb/s

500 Kb/s

1.5

-

6 Mb/s

Video

2.5 Gb/s

ADSL

6 Mb/s max

.

.

.

Broadcast Video

•

200 NTSC channels (900 Mb/s)

•

50 HDTV channels (1 Gb/s)

•

Selectable video (up to 24.5 Mb/s)

ADSL

•

VOD channels (1.5

-

6 Mb/s)

•

IP channels (up to 1.5 Mb/s)


Figure 4 -VDSL Remote Terminal

There would 17 signal components on the fiber connecting a remote terminal to the CO. These components would include a transmit and receive component for each of the 8 wavelength channels assigned to a particular remote terminal and a broadcast component (receive only). The previous IEC paper [3] described the spectral distribution of the of signals on the fiber connecting the remote terminal to the CO.

Wavelength channel 1 contains a SONET framed receive signal with a data rate of 2.5 Gb/s (OC-48) and a SONET framed transmit signal with a data rate of 620 Mb/s (OC-12). These data rates include SONET overhead. The SONET signals each component signals for each of the attached subscribers. A spacing of 50 GHz between the transmit and receive signals is more than enough to prevent any significant overlapping of the receive and transmit spectra. Broadcast channel B contains a receive only signal with a data rate of 2.5 Gb/s. The center frequencies of the wavelength channels, which all have spectra similar to the spectrum of channel 1, are separated by 200 GHz. This is more than enough to prevent the spectrum of a channel from overlapping the spectrum of an adjacent channel.

In the optical add/drop multiplexer (ADM)(in the upper left of Figure 4), the signal on the fiber is demultiplexed and the receive component in the 1 channel is dropped at the remote terminal. The receive components in the other channels are passed through the ADM to other remote terminals. Similarly, the transmit component in the 1 channel is multiplexed (added) onto the fiber by the optical ADM. The broadcast signal power is split in the ADM, with some of the signal power being dropped and some of the signal being passed through. The optical coupler combines the transmit and receive signals on a common fiber connected to the ADM, with separate transmit and receive fibers connecting the coupler to the electronic multiplexing and distribution unit.

 The multiplexing and distribution unit converts the received optical signals  to electronic signals, demultiplexes the component signals in channel 1, and sends them to the appropriate ADSL interface unit. Signals received from the ADSL interface units are multiplexed together in the multiplexing and distribution unit and converted to an optical signal, which is sent to the optical coupler. Each ADSL receive signal has a maximum data rate of 6 Mb/s and maximum upstream data rate of  500 Kb/s. The multiplexing and distribution unit performs statistical multiplexing on the ADSL signals. Thus the composite date rate is less than the product of the maximum data rate times the number of subscriber lines. The total downstream and upstream data rates for 200 subscribers can be easily accommodated by the channel 1, which provides rates of 2.5 Gb/s downstream and 620 Mb/s upstream.

The multiplexing and distribution unit also converts the broadcast signal (receive-only) in channel B  and distributes the same signal to each of the video switches. This will require multiple line drivers. 
As discussed section 4, the broadcast channel can support many audio/video channels if data compression is employed. The 2.5 Gb/s data rate can easily accommodate 250 video channels and many audio channels.

ADSL services include high speed Internet access and streaming audio services, and streaming audio/video services, which are described in Section 4. These ADSL services are distinct from the broadcast service. The ADSL interface unit processes ADSL service data and performs the handshaking with subscribers associated with selection of services. The data rates for ADSL services are 1.5 to 6 Mb/s downstream and up to 500 Mb/s upstream. There is a separate ADSL interface unit for each subscriber. The ADSL interface unit would also handle plain-old-telephone service (POTS) and basic rate Integrated Services Digital Networks (ISDN) service. Data associated with these latter services would be passed through the ADSL interface unit without alteration or delay, and data rates listed above do not include POTS/ISDN data.

For each subscriber, there is a separate video (audio) switch. This switch allows each subscriber to select from a large set of channels a small subset  channels to be transmitted to the subscriber premises. In the case shown in Figure 4, the video switch selects audio/video channels having a composite data rate in the range of 9 to 24.5 Mb/s form a set of channels having a composite data rate of 2.5 Gb/s. A control signal in the upstream ADSL data is used to select the desired audio/video channels.

The VDSL interface unit receives digital data from the ADSL interface unit and the video switch and sends this data over a twisted pair line to the subscriber. Similarly, the VDSL interface unit receives upstream data from the subscriber and sends this data to the ADSL interface unit. ADSL downstream data and broadcast data are used by the VDSL interface unit to modulate the signal that is transmitted over the twisted pair. The VDSL interface unit demodulates the signal received over the twisted pair to extract upstream ADSL data. Note that POTS and ISDN data is passed through the interface unit without alteration, except for modulation and demodulation.

The VDSL spectrum is specified over the range of 200 KHz to 30 MHz [21]. However, this full spectral range is usually not employed. The previous IEC paper [3] described how a 10 MHz VDSL spectrum would be employed to support POTS, ISDN, the ADSL functions described above and audio/video broadcasting.
With the VDSL option described above, the downstream bandwidth and data rate are much greater than the upstream bandwidth and data rate. Other VDSL options are possible. If the upstream bandwidth is increased, the upstream data rate can be increased by a roughly proportional amount. One particular option is a symmetric VDSL implementation, where the upstream and downstream data rates are both approximately 13 Mb/s and the upstream and downstream bandwidths are approximately equal. These rates can be doubled if two twisted pairs are connected to a subscriber. To minimize crosstalk from the downstream channel to the upstream channel between twisted pairs in the same cable, the upstream and downstream bands should not overlap. Thus, the asymmetric and symmetric versions of VDSL should not be used within the same cable.

Support for Multi-Tenant Buildings and Complexes


Typically, large multi-tenant buildings (either residential or business) have T1 (DS1) or T3 (DS3) connections to the central office rather than having an individual twisted pair for each subscriber. In this case, it would be appropriate to provide the multi-tenant building with its own remote terminal, which would be installed in a common area of the building. The remote terminal would be connected to subscribers within the building by the existing wiring. Remote terminals within buildings would generally be smaller than remote terminal in the field and would support a smaller number of subscribers. Similarly, building complexes or campuses may be provided with dedicated remote terminals. 
Network Services That Make Up the Triple Play
Services supported by the proposed access network include: POTS, basic rate ISDN, high speed Internet access, audio/video-on-demand, broadcast audio and video, and various Internet Protocol (IP) services, such as high speed Internet access and voice over IP. POTS, ISDN, and Internet access would be similar the services currently provided over telephone interfaces (analog or digital) and ADSL interfaces. Video services are the most demanding services in terms of the required capacity of the access lines, and these services require special processing.

Audio and video data is generated in continuous streams rather than discrete blocks. For directly encoded digital audio/video, the data rates are fixed, but can be very high. If audio/video compression techniques, such as the Moving Pictures Experts Group (MPEG) techniques, are employed, these rates can be significantly reduced. For example, with MPEG-2 compression, National Standard Television Committee (NTSC) video rates can be reduced from 216 Mb/s to between 3 and 6 Mb/s [13]. Directly encoded High Definition TV (HDTV) video, which has approximately 5 times as many pixels as NTSC video, would require a data rate of approximately 1 Gb/s. However with MPEG-2 compression, the required HDTV data rate can be reduced to approximately 20 Mb/s [14]. Streaming audio is typically encoded using MPEG Layer 3 (MP3) and can be accommodated with a data rate of 128 Kb/s [24].

Clearly, HDTV is the driver. The standard data rate for HDTV is 19.2 Mb/s [23]. With an allowance for overhead, HDTV requires a data rate of approximately 20 Mb/s overhead. Recently, lower data rates have been proposed for HDTV.  Knowles [24] suggests that the HDTV data rate can be reduced to between 12 Mb/s and 15 Mb/s.

MPEG data rates vary from frame to frame and from scene to scene, with scenes containing a lot of motion requiring relatively high data rates. Buffering of frames can employed to significantly reduce variations in the transmission data rate due to frame-to-frame variations [25]. Transmission rate variations due to scene-to-scene variations can be reduced by statistical multiplexing of multiple video channels. MPEG-2 data can be separated into base layer data, which is necessary for operation, and enhancement layer data, which provides enhanced image

quality, but is not necessary for operation [26]. The maximum data rates for the base layer and the enhancement layer can be limited to certain specified values, with higher limits providing better image quality.

MPEG compression enables many broadcast channels to be accommodated within the 2.5 Gb/s capacity of the broadcast channel. For the case shown in Figure 4, 200 NTSC, 50 HDTV, and many audio channels can be accommodated by the broadcast channel. Thus, the limiting factor is not the fiber connection between the remote terminal and the CO, but instead the twisted pair connections between the remote terminal and the subscriber premises. 
The VDSL channel capacity must be sufficient to accommodate the composite base layers of all the video channels transmitted  over the channel. However, the channel capacity does not necessarily need to accommodate the maximum composite data rate of both the base layers and enhancement layers. Instead, if this composite data rate exceeds the channel capacity, the enhancement data for one or more video channels can be dropped and the associated images would suffer some temporary graceful degradation.

Many combinations of services can be supported over VDSL access lines. Three options for combining these VDSL services are listed in Table 2 and are described below. POTS/ISDN services can be supported for each of these options, but are not listed.

	Service Option
	Broadcast Capability
	 ADSL Capability 

	Option 1
	24.5 Mb/s

1 HDTV & 1 NTSC channels
	1.5 Mb/s

1 MPEG-1 VOD channel 

	Option 2
	20 Mb/s

1 HDTV channel
	6 Mb/s

2 MPEG-2 VOD channels

	Option 3
	13 Mb/s

3 NTSC channels
	13 Mb/s

3 MPEG VOD channels


Table 2 - Network Service Options

With Option 1 the broadcast data rate is 24.5 Mb/s and the downstream ADSL data rate is1.5 Mb/s. This option can support one HDTV broadcast channel (at 20 Mb/s) and one NTSC audio/video broadcast channel (at 4.5 Mb/s) with MPEG-2 compression. Multiple MP3 audio broadcast channels can be substituted for the NTSC video channel. Option 1 can also support one MPEG-1 video-on-demand (VOD) channel (at 1.5 Mb/s).  High speed Internet access for one or two computers (or other IP services) and/or multiple MP3 audio-on-demand channels can be substituted for this video-on-demand channel.

With Option 2 the broadcast data rate is 20 Mb/s and the downstream ADSL data rate 6 Mb/s. This option can support one HDTV broadcast channel (at 20 Mb/s) and two MPEG-2 video-on-demand channels (at 3 Mb/s each). Multiple NTSC broadcast channels can be substituted for the HDTV channel. High speed Internet access for multiple computers (or other IP services) and multiple MP3 audio-on-demand channels can be substituted for a video-on-demand channel.

With Option 3 the broadcast data rate is 13 Mb/s and the downstream ADSL data rate 13 Mb/s.  This option  can support three NTSC video broadcast channel (at 4.5 Mb/s each) and three MPEG-2 video-on-demand channels (at 4.5 Mb/s each). High speed Internet access for multiple computers (or other IP services) and multiple MP3 audio-on-demand channels can be substituted for a video-on-demand channel.

Other options are available. For example, two HDTV channels could be supported if the HDTV data rate is reduced to 12 Mb/s [23], or if the resolution of one HDTV channel (the channel driving a smaller screen TV) is degraded. 

The data rates listed above are nominal rates; the actual rates will vary. The sum of the maximum rates for the MPEG-2 base layers of video channels (including broadcast and on-demand video) must be less than the allocated capacity. For example, with Option 1, the sum of the rates for the HDTV base layer data and the NTSC base layer data must be less than 24.5 Mb/s. Excess capacity will be utilized by the enhancement layers. This excess capacity may be less than the sum of the maximum rates of the enhancement layer. In the case of Option 1, if the maximum enhancement layer rates for the HDTV and NTSC video channels occur at the same time, then the enhancement layer data for one of the channels may need to be dropped which will gracefully degrade (temporarily) the image quality for that channel. As the number of video channels increases, statistical multiplexing provides a greater advantage, and the probability the of exceeding the excess capacity, and suffering image degradation, decreases. Flow control could be used on the Internet access channels to free up more capacity for the video channels when needed. If the remote terminal switches over to the backup CO, then the services for each of the options would be scaled back.

Although the service options described above would be suitable for most residential customers, they would not be suitable for business customers. In commercial areas, the symmetric VDSL option alluded to in the previous subsection would be more appropriate. With the symmetric option, data rates of 13 Mb/s would be provided for both the upstream and downstream traffic, and the broadcast feature, which would be used primarily for entertainment, would not be supported. This 13 Mb/s data rate could support voice (including POTS voice and voice over IP), data transfers from servers, virtual private networks, Internet access, and video teleconferencing. As noted previously, 13 Mb/s data rate could be doubled if a subscriber is connected to the remote terminal by two twisted pairs. Normally, a particular remote terminal would support either the services associated with asymmetric VDSL or the services associated with symmetric VDSL, but not both.

Transporting Triple Play Data Thorough a Converged Network


Rather than having separate transport networks and/or separate network elements for voice, video, and data communications, it is desirable to have a single integrated network and common network elements. This section describes a converged transport network based on multi-protocol label switching (MPLS) [27]. This network transports data associated with the various services in a similar manner using common network elements.   

Connectivity and Data Flow Supporting the Triple Play


Figure 5 shows the network elements supporting the end-to-end transport of triple play and how these elements are interconnected. In this case, client equipment at the customer premise is connected through the network to a server, which may be either  part of the network or subscriber of the network. Service request would be sent from the client to the remote terminal. Data from multiple subscriber would be statistically multiplexed in the remote terminal an forwarded to a central office functioning as a serving wire center (SWC) (end office) for the subscribers. At the SWC, subscriber data from multiple remote terminals would be multiplexed and sent to the label switching router in a hub central office. Only a fraction of the central offices would contain routers. Most of the central offices would require only multiplexers. The data would be sent through the network to another router connected to a server. The data may traverse additional routers within the network.


[image: image5.wmf]Client

Remote

Terminal

TP

Multi

-

plexer

IP/MPLS

Router

Customer

Premise

Customer

Neighborhood

Customer

SWC

Central

Office

IP/MPLS

Network

IP/MPLS

Router

Central

Office

Multi

-

plexer

Server

SWC

Server

Server

Premise

FP

Legend

FP 

–

fiber pair

IP/MPLS 

–

Internet Protocol/

multi

-

protocol label switching

TP 

–

twisted pair

SWC 

–

serving wire center


Figure 5 - Network Elements Supporting the Triple Play


Typically, servers must accommodate a much higher throughput than clients. Therefore servers would often be connected to the network by fiber rather than by twisted pairs, as shown in Figure 5. In this case the server is connected to a multiplexer in its serving wire center (central office) by a fiber pair carrying data to and from multiple clients. The multiplexer would combine data from multiple servers and send this data to a label switching router, which would be connected through the network to multiple routers. Each of these routers would be connected to many clients and to other servers.

Communication Protocols Supporting the Triple Play


Figure 6 shows the communication protocols associated with the data flow described in the previous subsection. In this case illustrated by Figure 6, a streaming application (APP), such as video would run over the User Datagram Protocol (UDP). Applications involving file transfers would run over the Transmission Control Protocol (TCP). The application protocol and UDP (and TCP) are end-to-end protocols that are seen only by the client and server equipment. UDP (and TCP) runs over the Internet Protocol (IP), which contains global addresses used by conventional routers to establish paths through the network and forward data through the network.


[image: image6.wmf]APP

APP

UDP

UDP

IP

ENET

VDSL

IP

MPLS

GFP

SONET

Client

Server

TP

RT

Router

MUX

Router

SONET

SONET

GFP

GFP

MPLS

FP

SONET

VDSL

IP/MPLS

Network

ENET

MPLS

GFP

IP

MPLS

SONET

SONET

GFP

GFP

MPLS

SONET

SONET

GFP

GFP

Legend

APP 

–

application protocol, ENET 

–

Ethernet,  FP 

–

fiber pair, GFP 

–

Generic Framing Procedure,

IP 

–

Internet Protocol, MPLS 

–

multi

-

protocol label switching, MUX 

–

multiplexer, 

RT 

–

remote terminal, TP 

–

twisted pair, UDP 

–

User Datagram Protocol, 

VDSL 

–

very

-

high

-

speed digital subscriber line 

MPLS

GFP

GFP

SONET

SONET

MUX


Figure 6 - Protocol Diagram for Converged Transport of Triple Play Data

Between the customer premise and the remote terminal, IP would be run over Ethernet, which is the predominant local area network protocol and the layer protocol 2 preferred by many subscribers. Ethernet would run over VDSL, and both Ethernet and VDSL would be terminated in the remote terminal. Functioning similar to a service edge router, the remote terminal would establish label switched paths (LSPs) to the server for client data flows. Multi-protocol label switching (MPLS) headers that identify LSPs would be attached to IP packets at the remote terminal. These MPLS header would be used instead of the complete IP address to forward packets through the multiplexers and routers. At the remote terminal IP/MPLS packets would be mapped into SONET payloads using the Generic Framing Procedure (GFP) [28] and sent to the SWC. GFP can perform layer 2 functions, such as packet/frame synchronization, GFP would be the layer 2 protocol between the remote terminal and the multiplexer.


The multiplexer statistically multiplexes upstream IP/MPLS packets from the remote terminals and forwards them to the routers. Conversely, the multiplexer uses the MPLS labels to forward downstream packets received from the router to the appropriate remote terminals. Similarly, the router uses the MPLS label to forward packets (upstream and downstream) to remote terminals or to other routers. Typically, at the multiplexers and routers, IP/MPLS packets would be mapped into SONET payloads using GFP. Normally, the routers would operate at the MPLS layer and would not need to use the IP header. However, the IP address would be used to forward unlabeled packets, and routers in the network that do not have an MPLS capability (legacy routers) would forward packets based on the IP address.


At the server, an MPLS header would be attached to IP packet to enable forwarding of these packets along LSPs. Typically, downstream IP/MPLS packets would be mapped into SONET payloads at a server and sent the multiplexer. At the multiplexer, downstream packets from the server would be forwarded to the router, and the MPLS label would be used to forward upstream packets to the server. 

The basic approach illustrated by Figures 5 and 6 could be used for voice, video, and data communications. UDP would be used for continuous data flows, such as those associated  with voice over IP and video over IP. TCP would be used for bursty data flows, such as file transfers. Thus, convergence to IP data and IP/MPLS transport would be achieved.

Required Network Capacity 


The interoffice network must have sufficient capacity to support the data flows through the proposed access network. A end office (or SWC) must provide sufficient capacity to support broadcast and individual services for its attached subscribers. 

As discussed in the preceding subsection, the composite broadcast signal has a data rate of 2.5 Gb/s. This signal would be replicated throughout the interoffice network and in the remote terminals. Thus, only a single copy of this signal needs to be transmitted on any link. In Verizon's interoffice network, the average number of subscribers served by a CO is approximately 12000. If this many subscribers are operating continuously at the full individual downstream data rate of 6 Mb/s, then the aggregate downstream data rate that must be supported by a CO would be approximately 75 Gb/s, which includes the broadcast data. However, the actual data rate would be much less than this theoretical maximum. Not all subscribers will be active at the same time, and most subscribers will be receiving data at considerably less than the maximum rate. Since statistical multiplexing (and demultiplexing ) is performed in the remote terminals, the aggregate data rate at a particular time of day will be close to average data rate for that time of day times the number of active subscribers. If we assume 10000 active subscribes operating at an average downstream data rate of 1 Mb/s (excluding broadcast data), then the aggregate downstream data rate that must be supported by an end office would be approximately 12.5 Gb/s, which includes 2.5 Gb/s for broadcast data and 10 Gb/s for individual downstream data. The upstream data rate that must be supported by an end office would typically be much lower than this downstream data rate.

How the interoffice network can provide the capacity required to support the proposed FTTN access networks is beyond the scope of this paper. However, this required capacity can readily be supported by current transport technology. Even without WDM, data rates of 10 Gb/s can be supported over a single fiber. With WDM, data rates of close to 1 Tb/s over a single fiber are possible.

Summary 
Currently, telephone networks can support only a limited set of communication services, and cannot support the triple play of communication services. In particularly, broadcast audio/video cannot be supported over telephone lines. Fiber-to-the-premise (FTTP) is the ultimate solution, however, FTTP is very expensive and takes a long time to deploy.

This paper proposes an architecture for a network that can provide the triple play of services (voice, video, and data communications) over existing over telephone lines. Specifically, the support services include: voice, ISDN services, high speed Internet access, streaming audio/video-on-demand, and broadcast audio/video.

The architecture presented in this paper is based on fiber-to-the-neighborhood (FTTN) and very-high-speed digital subscriber line (VDSL) technology. Fiber is run from a central office to remote terminals in the subscribers' neighborhoods. Existing twisted pair access lines are used to connect these remote terminals to equipment in the subscriber premises. In this way, the length of the twisted pair lines can be kept short (less than half a mile) and high data rates can be supported over these lines. In residential areas, downstream data rates of 26 Mb/s can be provided, which can readily support voice, ISDN, high speed Internet access, and a few video channels. However, FTTN and VDSL by themselves cannot support a robust audio/video broadcast capability.

To support audio/video broadcasting ,,broadcast data would be replicated in the central offices (COs) and sent to the remote terminals, where it would be switched onto subscriber lines. The remote terminals can be located outdoors in the neighborhood of the subscribers or in  multi-tenant buildings. Using an upstream control channel, a subscriber would select a few audio/video channels from a much larger set of channels. With this approach, an audio/video broadcast capability comparable to the capability provided by cable TV systems can be provide over telephone lines.

Redundancy would be provided in the access network to enable protection against access network failures. Each remote terminal would be connected to two end offices, so that subscriber connections can be switched to a backup end office if a fiber access line failure or a failure at the primary end office disrupts service to the subscribers. Subscribers requiring very high reliability could be connected to two remote terminals to provide comprehensive protection against access network failures.

With the appropriate upgrades, the interoffice network would have sufficient capacity to accommodate triple play services. A converged transport network based on MPLS is recommended. Transport of data associated with the various services would be similar and common network elements and protocols would be employed.   

References
[1] K. Maxwell, "Asymmetric Digital Subscriber Line: Interim Technology for the Next Forty Years,"  IEEE Communications Magazine, Vol.34, No. 10, October 1996.

[2] K. Maxwell, "Asymmetric Digital Subscriber Line, etc." op cit.

[3] K. DeMartino, "Providing a Complete Set of Communication Services Over Telephone Lines," IEC Comprehensive Report on Broadband Services, Applications, and Networks: Enabling Technologies and Business Models, copyright 2004.

[4] C. Omidyar and A. Aldridge, “Introduction to SDH/SONET,” IEEE  Communications Magazine, Vol. 31, No. 9, September 1993.

[5] "Verizon Philadelphia LATA Modeling," Sycamore Networks presentation to Verizon, 9 March 2001.

[6] R. Ramaswami and K. Sivarajan, Optical Networks:  A Practical Perspective, Morgan Kaufmann: New York, pp. 434-441, 1998.

[7] K. DeMartino, "An Architecture for a Seamless Mesh Network," Proceedings of DRCN 2003 - Design of Reliable Communication Networks Workshop, Banff, Alberta, Canada, October 2003.

[8] "Verizon Wire Center Profile," correspondence with Verizon Network Planning organization, 28 August 2001.

[9] T. Bartee, editor, Digital Communications, Indianapolis, Indiana: Howard W. Sams & Co., 1986, pp. 116-124.

[10] K. Maxwell, "Asymmetric Digital Subscriber Line, etc." op cit.

[11] G. Hawley, "Systems Considerations for the Use of xDSL Technology for Data Access," IEEE Communications Magazine, Vol. 35, No. 3, March 1997.

[12] International Organization for Standardization, “Coding of Moving Pictures and Associated Audio for Digital Media At Up to About 1.5 Mbit/s - Part 1:Systems," ISO/IEC 11172-1:1993.

[13] K. Maxwell, Residential Broadband - An Insider’s Guide to the Battle for the Last
 Mile, New York: John Wiley & Sons, 1999, pp. 37-38.

[14] K. Maxwell, Residential Broadband, etc., op cit, pp. 41-43.

[15] "Extending the Reach and Rate of Symmetric DSL Service," Symmetricom White Paper,

http://www.symmetricom.com, 2004.

[16] C. Sikes and B. Ray, "Definition of Managed Objects for G.SHDSL.BIS Lines," IETF Networking Working Group, draft-ietf-adsl-mpls-framework-02.txt, November 1997.

[17] K. Maxwell, "Asymmetric Digital Subscriber Line, etc." op cit.

[18] "VDSL Tutorial," DSL Forum, http://www.dslforum.org/aboutdsl/vdsl_tutorial.html, 2001.

[19] "VDSL;" Wikipedia, the free encyclopedia, http://en.wikipedia.org/wiki/VDSL, downloaded June 2005.

[20] "Dense Wavelength Division Multiplexing," ATG's Communications & Networking Technology Guide Series, Ciena Corporation tutorial, 1997.

[21] "Very-High-Data-Rate Digital Subscriber Line (VDSL)," International Engineering Consortium Online Tutorial, http://www.iec.org/online/tutorial/vdsl/topic05.html, downloaded 23 September 2003.

[22] Fraunhofer-Gesellschaft, “MPEG Audio Layer - 3,”

http://www.iis.fhg.de/amm/techinf/layer3/index.html, 1999.

[23] D. Richardson, "Internet HDTV," University of Washington presentation, http://www.washington.edu.hdtv, downloaded June 2005.

[24] C. Knowles, "Impact of HDTV Transmission of DVT Multi-Channel Broadcasting," presentation to Australian Broadcasting Summit, February 2003. 
[25] P. Pancha and M. El Zarki, “MPEG Coding for Variable Bit Rate Video Transmission,” IEEE Communications Magazine, Vol.32, No. 5, May 1994.

[26] T. Sikora, “MPEG-1 and MPEG-2 Digital Video Coding Standards”,
 http://wwwam.hhi.de/mpeg-video/papers/sikora/mpeg1_2/mpeg1_2.htm, downloaded 3 October 1997.

[27] R. Callon, et al, "A Framework for Multiprotocol Label Switching," IETF Network Working Group, draft-ietf-mpls-framework-02.txt, November 1997.

[28] E. Hernandez-Valencia, et al, "The Generic Framing Procedure: An Overview," IEEE Communications Magazine, Vol. 40, No. 5, May 2002.

1
6

_1185253230.ppt


Remote

Terminal

Fiber

To CO

To CO Via 

Remote

Terminal

Fiber

1 Mile

Diameter

Circle

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

TP

S1

S2

TP

S200

TP

Legend

.

S1–S200  - subscriber node

       TP - twisted pair line

  fiber line to CO








_1185253309.ppt


Optical

ADM

.

.

.

 TP

26 Mb/s down

500 Kb/s up

To/From

User

#1

To/From

User

#200

Multiplexing

&

Distribution

VDSL

I/F

 #1

To/From

Adjacent

 RT/ CO

To/From

Adjacent RT

Optical

ADM

To/From

Adjacent 

RT/ CO

To/From

Adjacent RT

Fiber

Fiber

 TP

26 Mb/s down

500 Kb/s up

VDSL

I/F

#1

ADSL

I/F

ADSL

I/F

1.5-6 Mb/s

500 Kb/s

9-24.5 Mb/s

Video

Switch

ADSL

6 Mb/s max

Video

2.5 Gb/s

Optical

Coupler

2.5 Gb/s

620 Mb/s

l1

lB

2.5 Gb/s

Optical

Coupler

Fiber

Fiber

620 Mb/s

2.5 Gb/s

l1

2.5 Gb/s

lB

Video

Switch

9-24.5 Mb/s

500 Kb/s

1.5-6 Mb/s

Video

2.5 Gb/s

ADSL

6 Mb/s max

.

.

.

Broadcast Video

		200 NTSC channels (900 Mb/s)

		50 HDTV channels (1 Gb/s)

		Selectable video (up to 24.5 Mb/s)



ADSL

		VOD channels (1.5-6 Mb/s)

		IP channels (up to 1.5 Mb/s)










_1185253454.ppt


Client

Remote

Terminal

TP

Multi-

plexer

IP/MPLS

Router

Customer

Premise

Customer

Neighborhood

Customer

SWC



Central

Office

IP/MPLS

Network

IP/MPLS

Router

Central

Office

Multi-

plexer

Server

SWC

Server

Server

Premise

FP

Legend

FP – fiber pair

IP/MPLS – Internet Protocol/

multi-protocol label switching

TP – twisted pair

SWC – serving wire center








_1185253467.ppt


APP

APP

UDP

UDP

IP

ENET

VDSL

IP

MPLS

GFP

SONET

Client

Server

TP

RT

Router

MUX

Router

SONET

SONET

GFP

GFP

MPLS

FP

SONET

VDSL

IP/MPLS

Network

ENET

MPLS

GFP

IP

MPLS

SONET

SONET

GFP

GFP

MPLS

SONET

SONET

GFP

GFP



Legend

APP – application protocol, ENET – Ethernet,  FP – fiber pair, GFP – Generic Framing Procedure,

IP – Internet Protocol, MPLS – multi-protocol label switching, MUX – multiplexer, 

RT – remote terminal, TP – twisted pair, UDP – User Datagram Protocol, 

VDSL – very-high-speed digital subscriber line 

MPLS

GFP

GFP

SONET

SONET

MUX








_1185253269.ppt


RT

CO

CO

CO

RT

RT

RT

RT

Legend

CO - central office

RT – remote terminal

To CO

via RTs

To CO

via RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

To RTs

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT

RT








_1185253158.ppt


CO

CO

CO

CO

CO

CO

To CO

To CO

To CO

To CO

To CO

Interoffice

Trunk

Interoffice

Trunk

Interoffice

Trunk

Interoffice

Trunk

Access

Lines

Access

Lines

Access

Lines

Access

Lines

Access

Lines








