DMA Block Diagram

	
	
	
	

	
	
	Data Count
	

	
	
	
	

	Data Lines
	
	Data Register
	

	
	
	
	

	
Address Lines
	
	Address Register
	

	
	
	
	

	DMA Request

DMA acknowledge

Interrupt

Read

Write
	
	Control Logic
	

	
	
	
	


The Direct Memory Access (DMA) module handles all data transfers between the computer’s peripherals and memory without intervention by the processor.

A DMA module transfers a sector from the hard disk to memory by first accessing the system bus. It may either use the bus when the processor is not engaging it, or it must conduct cycle stealing. Cycle stealing is to force the processor to temporarily suspend its current operation. 

While the processor does some other operation not involving use of the system bus, the DMA module transfers the entire sector of data, one word at a time from the hard disk to memory. When the transfer is complete, the DMA module informs the processor by sending an interrupt signal to it.




















