Basic Operation of a Frequency Synthesizer
(Including phase locked loops (PLL) frequency multiplication and division)

Today most receivers use frequency synthesizers. Many of them advertise this fact by displaying words like "PLL", "Synthesized", or "Quartz" on their front panels or in the advertising literature. Whatever one thinks of the sales language, synthesisers offer tremendous advantages to the operation of a receiver. Not only do they enable receivers to have the same stability as the quartz reference, but they also enable many other facilities to be introduced because they can easily be controlled by a microprocessor. This enables facilities such as multiple memories, keypad frequency entry, scanning and much more to be incorporated into the set.
Synthesizers are widely used, but their operation is not always well understood. One of the reasons for this is that their design can involve some complicated math, but despite this the basic concepts are relatively easy to grasp.

Basics
A frequency synthesizer is based around a phase locked loop. This circuit uses the idea of phase comparison as the basis of its operation. From the block diagram of a basic loop shown in Fig. 1 it can be seen that there are three basic circuit blocks, a phase comparator, voltage controlled oscillator, and loop filter. A reference oscillator is sometimes included in the block diagram, although this is not strictly part of the loop itself even though a reference signal is required for its operation.


Fig. 1 Block diagram of a basic phase locked loop

The loop operates by comparing the phase of two signals. The signals from the voltage controlled oscillator and reference enter the phase comparator Here a third signal equal to the phase difference between the two input signals is produced.
The phase difference signal is then passed through the loop filter. This performs a number of functions including the removal of any unwanted products that are present on this signal. Once this has been accomplished it is applied to the control terminal of the voltage controlled oscillator. This tune voltage or error voltage is such that it tries to reduce the error between the two signals entering the phase comparator. This means that the voltage controlled oscillator will be pulled towards the frequency of the reference, and when in lock there is a steady state error voltage. This is proportional to the phase error between the two signals, and it is constant. Only when the phase between two signals is changing is there a frequency difference. As the phase difference remains constant when the loop is in lock this means that the frequency of the voltage controlled oscillator is exactly the same as the reference.

Synthesisizers
A phase locked loop needs some additional circuitry if it is to be converted into a frequency synthesizer. This is done by adding a frequency divider between the voltage controlled oscillator and the phase comparator as shown in Fig. 2.

[image: image1.png]utput

votage
contoted]
ks

reterance Programmabie
o e

Loy
e





Fig. 2 A programmable divider added into a phase locked loop enables the frequency to be changed.

Programmable dividers or counters are used in many areas of electronics, including many radio frequency applications. They take in a pulse train like that shown in Fig. 3, and give out a slower train. In a divide by two circuit only one pulse is given out for every two that are fed in and so forth. Some are fixed, having only one division ratio. Others are programmable and digital or logic information can be fed into them to set the division ratio.
[image: image2.png]‘nput signat

- Output signal





Fig. 3 Operation of a programmable divider

When the divider is added into the circuit the loop still tries to reduce the phase difference between the two signals entering the phase comparator. Again when the circuit is in lock both signals entering the comparator are exactly the same in frequency. For this to be true the voltage controlled oscillator must be running at a frequency equal to the phase comparison frequency times the division ratio.

It can be seen that if the division ratio is altered by one, then the voltage controlled oscillator will have to change to the next multiple of the reference frequency. This means that the step frequency of the synthesizer is equal to the frequency entering the comparator.

Most synthesizers need to be able to step in much smaller increments if they are to be of any use. This means that the comparison frequency must be reduced. This is usually accomplished by running the reference oscillator at a frequency of a megahertz or so, and then dividing this signal down to the required frequency using a fixed divider. In this way a low comparison frequency can be achieved.
[image: image3.png]Frogramming
infarmation

e Outpu




 

Fig. 4 Comparison frequency reduced by adding a fixed divider after the reference oscillator

Analog Techniques
Placing a digital divider is not the only method of making a synthesizer using a phase locked loop. It is also possible to use a mixer in the loop as shown in Fig. 5. Using this technique places an offset into the frequency generated by the loop.

[image: image4.png]oxernal
signal

‘sailator

Figures n brackets
used as an example




Fig. 5 A phase locked loop with mixer

The way in which the loop operates with the mixer incorporated can be analyzed in the same manner that was used for the loop with a divider. When the loop is in lock the signals entering the phase detector are at exactly the same frequencies. The mixer adds an offset equal to the frequency of the signal entering the other port of the mixer. To illustrate the way this operates figures have been included. If the reference oscillator is operating at a frequency of 10 MHz and the external signal is at 15 MHz then the VCO must operate at either 5 MHz or 25 MHz.. Normally the loop is set up so that mixer changes the frequency down and if this is the case then the oscillator will be operating at 25 MHz.

It can be seen that there may be problems with the possibility of two mix products being able to give the correct phase comparison frequency. It happens that as a result of the phasing in the loop, only one will enable it to lock. However to prevent the loop getting into an unwanted state the range of the VCO is limited. For loops that need to operate over a wide range a steering voltage is added to the main tune voltage so that the frequency of the loop is steered into the correct region for required conditions. It is relatively easy to generate a steering voltage by using digital information from a microprocessor and converting this into an analogue voltage using a digital to analogue converter (DAC). The fine tune voltage required to pull the loop into lock is provided by the loop in the normal way.

Multi-loop synthesizers
Many high performance synthesizers use several loops that incorporate both mixers and digital dividers. By using these techniques it is possible to produce high performance wide range signal sources with very small step sizes. If only a single loop is used then there may be short falls in the level of performance.
There is a large variety of ways in which multi-loop synthesizers can be made, dependent upon the requirements of the individual system. However as an illustration a two loop system is shown in Fig. 6. This uses one loop to give the smaller steps and the second provides larger steps. This principle can be expanded to give wider ranges and smaller steps.

[image: image5.png]Firstloop =secondloop.

20688
210 e 2

T on
(e 5 18)

(100 k)

Fixed
v

(Onizgnreto




Fig. 6 An example of a synthesizer using two loops

The first loop has a digital divider and operates over the range 19 to 28 MHz. Having a reference frequency of 1 MHz it provides steps of 1 MHz. The signal from this loop is fed into the mixer of the second one. The second loop has division ratios of 10 to 19, but as the reference frequency has been divided by 10 to 100 kHz to give smaller steps.
The operation of the whole loop can be examined by looking at extremes of the frequency range. With the first loop set to its lowest value the divider is set to 19 and the output from the loop is at 19 MHz. This feeds into the second loop. Again this is set to the minimum value and the frequency after the mixer must be at 1.0 MHz. With the input from the first loop at 19 MHz this means that the VCO must operate at 20 MHz if the loop is to remain in lock.
At the other end of the range the divider of the first loop is set to 28, giving a frequency of 28 MHz. The second loop has the divider set to 19, giving a frequency of 1.9 MHz between the mixer and divider. In turn this means that the frequency of the VCO must operate at 29.9 MHz. As the loops can be stepped independently it means that the whole synthesizer can move in steps of 100 kHz between the two extremes of frequency. As mentioned before this principle can be extended to give greater ranges and smaller steps, providing for the needs of modern receivers.

Synthesizers and Phase Noise
(Including receiver reciprocal mixing)

One of the main problems with frequency synthesisers and frequency synthesis using phase locked loops is the fact that some designs generate high levels of phase noise. However it is possible to design some very good low phase noise synthesizers. The problem is often that receivers and transceivers are designed for low production costs, and this naturally means that some short cuts are needed.

What is phase noise?
Phase noise is present on all signals to some degree and it is caused by small phase (and hence frequency) perturbations or jitter on the signal. It manifests itself as noise spreading out either side from the main carrier as shown in Figure 1. 

[image: image6.png]Amplitude

Wanted

Phase noise
signal

either side of
the signal

Frequency




Fig. 1 Phase noise on a signal

Some signal sources are better than others. Crystal oscillators are very good and have very low levels of phase noise. Free running variable frequency oscillators normally perform well. Unfortunately synthesizers, and especially those based around phase locked loops, do not always fare so well unless they are well designed. If significant levels of phase noise are present on a synthesizer used as a local oscillator in a receiver, it can adversely affect the performance of the radio in terms of reciprocal mixing.
Some oscillators have phase noise levels that are quoted in their specifications. Any high quality signal generator will have the level of phase noise specified, as do many high performance crystal oscillators used as standards. Their performance is generally specified in dBc/Hz and at a given offset from the carrier. The term dBc simply refers to the level of noise relative to the carrier, i.e. -10 dBc means that the level is 10 lower than the carrier.
The bandwidth in which the noise is measured also has to be specified. The reason for this is that noise spreads over the frequency spectrum. Obviously the wider bandwidth that is used, the greater the level of noise that will pass through the filter and be measured. To prove this, just try selecting a different bandwidth on a receiver and check what happens to the noise level. It will rise for a wider bandwidth and fall when a narrow bandwidth is used. Technically the most convenient bandwidth to use a 1 Hz bandwidth and so this is used. When measuring this a wider bandwidth is usually used because it is difficult to obtain 1 Hz bandwidth filters and a correction is made mathematically.
Finally the level of noise varies as different offsets from the carrier are taken. Accordingly this must be included in a specification. A very good oscillator might have a specification of -100 dBc/Hz at 10 kHz offset.
It has already been mentioned that the level of phase noise changes as the offset from the carrier changes and for "simple" signal sources like crystal oscillators or variable frequency oscillators the phase noise reduces as the frequency from the main carrier is increased. For frequency synthesizers the picture is a little more complicated as we shall see.

Phase noise in synthesizers
Each of the components in a frequency synthesizer produces noise that will contribute to the overall noise that appears at the output. The actual way in which the noise from any one element in the loop contributes to the output will depend upon where it is produced. Noise generated by the VCO will affect the output in a different way to that generated in the phase detector for example.
To see how this happens take the example of noise generated by the voltage controlled oscillator. This will pass through the divider chain and appear at the output of the phase detector. It will then have to pass through the loop filter. This will only allow through those components of the noise that are below the loop cut-off frequency. These will appear on the error voltage and have the effect of cancelling out the noise on the voltage controlled oscillator. As this effect will only take place within the loop bandwidth, it will reduce the level of noise within the loop bandwidth and have no effect on noise outside the loop bandwidth.
Noise generated by the phase detector is affected in a different way. Again only the components of the noise below the loop bandwidth will pass through the low pass filter. This means that there will be no components outside the loop bandwidth appearing on the tune voltage at the control terminal of the voltage controlled oscillator, and there will be no effect on the oscillator. Those components inside the loop bandwidth will appear at the oscillator control terminal. These will affect the oscillator and appear as phase noise on the output of the voltage controlled oscillator.
Matters are made worse by the fact that the division ratio has the effect of multiplying the noise level. This arises because the synthesizer effectively has the effect of multiplying the frequency of the reference. Consequently the noise level is also multiplied by a factor of 20 log N, where N is the division ratio.
Noise generated by the reference undergoes exactly the same treatments as that generated by the phase detector. It too is multiplied by the division ratio of the loop in the same way that the phase detector noise is. This means that even though the reference oscillator may have a very good phase noise performance this can be degraded significantly, especially if division ratios are high.
Dividers normally do not produce a significant noise contribution. Any noise they produce may be combined with that of the phase detector.
The combined noise of the loop at the output generally looks like that shown in Figure 2. Here it can be seen that the noise within the loop bandwidth arises from the phase detector and the reference. Outside the loop bandwidth it arises primarily from the voltage controlled oscillator. From this it can be seen that optimisation of the noise profile is heavily dependent upon the choice of the loop bandwidth. It is also necessary to keep the division ratio in any loop down to reasonable levels. For example a 150 MHz synthesizer with a 12.5 kHz step size will require a division ratio of 12000. In turn this will degrade the phase detector and reference phase noise figures by 81 dB inside the loop bandwidth - a significant degradation by anyone's standards! Provided that division ratios are not too high then a wide loop bandwidth can help keep the voltage controlled oscillator noise levels down as well.

[image: image7.png]Smplitude:

Carrier

Noise caused mairly
by reference and
phase tetector

Noise

caused
by VCo

Frequency




Figure 2 Noise profile of a typical synthesizer

Effects of phase noise
Phase noise can have a number of effects. For SSB transmitters like those used for HF communications for ship to shore, amateur radio and other applications the main effect is that splatter appears either side of the main signal. This results from the phase noise either side of the signal will rising and falling in line with the amplitude variations of the main signal. For digital transmissions using frequency or phase modulation, the noise can introduce errors causing the bit error rate (BER) to rise.

For receivers the main problem is an effect known as reciprocal mixing. To look at how this occurs take the case of a superhet receiver tuned to a strong signal. The signal will pass through the radio frequency stages, and then in the mixer it will be mixed with the local oscillator to produce a new signal at the right frequency to pass through the IF filters. When the local oscillator is tuned away by ten kilohertz, for example the signal will no longer be able to pass through the IF filters. However it will still be possible for the phase noise on the local oscillator to mix with the strong incoming signal to produce a signal that will fall inside the receiver pass-band as shown in Figure 3. This could be sufficiently strong to mask out a weak station.

[image: image8.png]Ampiude

Stong
ofiehannel Lot
Sinal Gaator

— Phasenose | . Wit
S mackedby o
o e

SPlnoboth cmes e T Frend
o mamediag frequency




Figure 3 The way in which phase noise on a signal results in reciprocal mixing

Specifications
A number of different methods are used to define the level of reciprocal mixing. Generally they involve the response of the receiver to a large off channel signal. To perform a reciprocal mixing measurement is rarely easy. The signal generator must always be much better than the receiver, otherwise the performance of the signal generator will be measured! To overcome this many people use an old valve generator because their performance is often very good in this respect.
A measurement can be made by noting the level of audio with a BFO on from a small signal. The signal is then tuned off channel by a given amount, normally about 20 kHz and then increased until the audio level rises to the same level as a result of the phase noise from the receiver. As the noise level is dependent upon the bandwidth of the receiver this has to be specified as well. Generally a bandwidth useable for SSB is used i.e. 2.7 kHz.
For example a good HF communications receiver might have a figure of 95 dB at a 20 kHz offset using a 2.7. kHz bandwidth. This figure will improve as the frequency offset from the main channel is increased. At 100 kHz one might expect to see a figure in excess of 105 dB or possibly more.
Another way of measuring the phase noise response is to inject a large signal into the receiver and monitor the level needed to give a 3 dB increase in background noise level.

PAGE  
10

