Introduction to the Superhet Radio Receiver
(Including mixing and mixers, local oscillators intermediate frequency or IF, filters and image performance)
The superhet radio is one of the most popular forms of receiver in use today. Virtually all broadcast radios, televisions and many more types of receiver use it almost exclusively. First developed at the end of the First World War, with its invention credited to the American Edwin Armstrong, its use has grown ever since the concept was first discovered.

Mixing
The idea of the superhet revolves around the process of mixing. Here RF mixers are used to multiply two signals together. (This is not the same as mixers used in audio desks where the signals are added together). When two signals are multiplied together the output is the product of the instantaneous level of the signal at one input and the instantaneous level of the signal at the other input. It is found that the output contains signals at frequencies other than the two input frequencies. New signals are seen at frequencies that are the sum and difference of the two input signals, i.e. if the two input frequencies are f1 and f2, then new signals are seen at frequencies of (f1+f2) and (f1-f2). To take an example, if two signals, one at a frequency of 5 MHz and another at a frequency of 6 MHz are mixed together then new signals at frequencies of 11 MHz and 1 MHz are generated.

 [image: image1.png]Amplitude:
Originating signals
Hesultant sgnal ¢ GOy ang  Hesultant sgnal

1-L00MH)  f Mg T+ (11 MHD)
'

Freguency




Figure 1 The signals generated by mixing or multiplying two signals together

Concept of the superhet
In the superhet radio, the received signal enters one input of the mixed. A locally generated signal (local oscillator signal) is fed into the other. The result is that new signals are generated. These are applied to a fixed frequency intermediate frequency (IF) amplifier and filter. Any signals that are converted down and then fall within the passband of the IF amplifier will be amplified and passed on to the next stages. Those that fall outside the passband of the IF are rejected. Tuning is accomplished very simply by varying the frequency of the local oscillator. The advantage of this process is that very selective fixed frequency filters can be used and these far out perform any variable frequency ones. They are also normally at a lower frequency than the incoming signal and again this enables their performance to be better and less costly.

To see how this operates in reality take the example of two signals, one at 6 MHz and another at 6.1 MHz. Also take the example of an IF situated at 1 MHz. If the local oscillator is set to 5 MHz, then the two signals generated by the mixer as a result of the 6 MHz signal fall at 1 MHz and 11 MHz. Naturally the 11 MHz signal is rejected, but the one at 1 MHz passes through the IF stages. The signal at 6.1 MHz produces a signal at 1.1 MHz (and 11.1 MHz) and this falls outside bandwidth of the IF so the only signal to pass through the IF is that from the signal on 6 MHz.
[image: image2.png]resuiting signal
1 MHz

Input signal
6.0 MHz 1MHzIF
amplifier

and filter

Oscillator signal
5 MHz




Figure 2 The basic concept of the superhet radio

If the local oscillator frequency is moved up by 0.1 MHz to 5.1 MHz then the signal at 6.1 MHz will give rise to a signal at 1 MHz and this will pass through the IF. The signal at 6 MHz will give rise to a signal of 0.9 MHz at the IF and will be rejected. In this way the receiver acts as a variable frequency filter, and tuning is accomplished.

Images
This all appears to be fine, but there is a problem. There are two signals that can enter the IF. With the local oscillator set to 5 MHz and with an IF it has already been seen that a signal at 6 MHz mixes with the local oscillator to produce a signal at 1 MHz that will pass through the IF filter. However is a signal at 4 MHz enters the mixer it produces two mix products, namely one at the sum frequency which is 10 MHz, whilst the difference frequency appears at 1 MHz. This would prove to be a problem because it is perfectly possible for two signals on completely different frequencies to enter the IF. The unwanted frequency is known as the image. Fortunately it is possible to place a tuned circuit before the mixer to prevent the signal entering the mixer, or more correctly reduce its level to acceptable value.

Fortunately this tuned circuit does not need to be very sharp. It does not need to reject signals on adjacent channels, but instead it needs to reject signals on the image frequency. These will be separated from the wanted channel by a frequency equal to twice the IF. In other words with an IG at 1 MHz, the image will be 2 MHz away from the wanted frequency.

 [image: image3.png]Armplitude
Local
Oscillator
Signal
Intermediate
frequency  Unwanted
image signal

TR tunng
RF funing

Wanted
7@3\

|
‘ N

>
Frequency




Figure 3 Using a tuned circuit to remove the image signal

Complete receiver
Having looked at the concepts behind the superhet it is helpful to look at a block diagram of a basic superhet shown in Figure 4. Signals enter the front end circuitry from the antenna. This contains the front end tuning to remove the image signal and often includes an RF amplifier to amplify the signals before they enter the mixer. The level of this amplification is carefully calculated so that it does not overload the mixer when strong signals are present, but enables the signals to be amplified sufficiently to ensure a good signal to noise ratio is achieved.

The tuned and amplified signal then enters one port of the mixer. The local oscillator signal enters the other port. The local oscillator may consist of a variable frequency oscillator that can be tuned by altering the setting on a variable capacitor. Alternatively it may be a frequency synthesizer that will enable greater levels of stability and setting accuracy.

Once the signals leave the mixer they enter the IF stages. These stages contain most of the amplification in the receiver as well as the filtering that enables signals on one frequency to be separated from those on the next. Filters may consist simply of LC tuned transformers providing interstage coupling, or they may be much higher performance ceramic or even crystal filters, dependent upon what is required.

Once the signals have passed through the IF stages they need to be demodulated. Different demodulators are required for different types of transmission, and as a result some receivers may have a variety of demodulators that can be switched in to accommodate the different types of transmission that are to be encountered. The output from the demodulator is the recovered audio. This is passed into the audio stages where they are amplified and presented to the headphones or loudspeaker.

	[image: image4.png]R
it





Figure 4 Block diagram of a basic superhet receiver


	 


This is a very basic superhet. Many sets these days are far more complicated. Some have more than one frequency conversion, and other areas of additional circuitry to provide the required levels of performance. However the basic concept remains the same, using the idea of mixing the incoming signal with a locally generated oscillation to convert the signals to a new frequency.
Receiver Selectivity
(Including filter performance and specifications, shape factor, crystal filters, LC filters, and ceramic filters)

Selectivity is one of the major specifications of any receiver. Whilst the sensitivity is important to ensure that it can pick up the signals and receive them at a sufficient strength, the selectivity is also very important. It is this parameter that determines whether the receiver is able to pick out the wanted signal from all the other ones around it. The filters used in receivers these days have very high levels of performance and enable receivers to select out individual signals even on today's crowded bands.

Superhet principle
Most of the receivers that are used today are superhet radios. In these sets the incoming signal is converted down to a fixed intermediate frequency. It is within the IF stages that the main filters are to be found. It is the filter in the IF stages that defines the selectivity performance of the whole set, and as a result the receiver selectivity specification is virtually that of the filter itself.

[image: image5.png]R
it





Figure 1 Block diagram of a basic superhet receiver

In some receivers simple LC filters may be used, although ceramic filters are better and are used more widely nowadays. For the highest performance crystal or mechanical filters may be used, although they are naturally more costly and this means they are only found in high performance sets.

Filter parameters
There are two main areas of interest for a filter, the pass band where it accepts signals and allows them through, and the stop band where it rejects them. In an ideal world a filter would have a response something like that shown in Figure 2. Here it can be seen that there is an immediate transition between the pass band and the stop band. Also in the pass band the filter does not introduce any loss and in the stop band no signal is allowed through.

[image: image6.png]Respanse FIEr response

Passhand

Stopband Stophand

Fregquency




Figure 2 The response of an ideal filter

In reality it is not possible to realise a filter with these characteristics and a typical response more like that shown in Figure 3. It is fairly obvious from the diagram that there are a number of differences. The first is that there is some loss in the pass band. Secondly the response does not fall away infinitely fast. Thirdly the stop band attenuation is not infinite, even though it is very large. Finally it will be noticed that there is some in band ripple.
[image: image7.png]rEeal fiiter
Response response

6008

Stopband

>
Frequency




Figure 3 Typical response of a real filter

In most filters the attenuation in the pass band is normally relatively small. For a typical crystal filter figures of 2 - 3 dB are fairly typical. However it is found that very narrow band filters like those used for Morse reception may be higher than this. Fortunately it is quite easy to counteract this loss simply by adding a little extra amplification in the intermediate frequency stages and this factor is not quoted as part of the receiver specification.

It can be seen that the filter response does not fall away infinitely fast, and it is necessary to define the points between which the pass band lies. For receivers the pass band is taken to be the bandwidth between the points where the response has fallen by 6 dB, i.e. where it is 6 dB down or -6 dB.

A stop band is also defined. For most receiver filters this is taken to start at the point where the response has fallen by 60 dB, although the specification for the filter should be checked this as some filters may not be as good. Sometimes a filter may have the stop band defined for a 50 dB attenuation rather than 60 dB.

Shape factor
It can be seen that it is very important for the filter to achieve its final level of rejection as quickly as possible once outside the pass band. In other words the response should fall as quickly as possible. To put a measure on this, a figure known as the shape factor is used. This is simply a ratio of the bandwidths of the pass band and the stop band. Thus a filter with a pass band of 3 kHz at -6dB and a figure of 6 kHz at -60 dB for the stop band would have a shape factor of 2:1. For this figure to have real meaning the two attenuation figures should also be quoted. As a result the full shape factor specification should be 2:1 at 6/60 dB.

Filter types
There is a variety of different types of filter that can be used in a receiver. The older broadcast sets used LC filters. The IF transformers in the receiver were tuned and it was possible to adjust the resonant frequency of each transformer using an adjustable ferrite core. 

Today ceramic filters are more widely used. Their operation is based on the piezoelectric effect. The incoming electrical signal is converted into mechanical vibrations by the piezoelectric effect. These vibrations are then affected by the mechanical resonances of the ceramic crystal. As the mechanical vibrations are then linked back to the electric signal, the overall effect is that the mechanical resonances of the ceramic crystal affect the electrical signal. The mechanical resonances of the ceramic exhibit a high level of Q and this is reflected in its performance as an electrical filter. In this way a high Q filter can be manufactured very easily.

Ceramic filters can be very cheap, some costing only a few cents. However higher performance ones are also available, and these are likely to be found in scanners and many other receivers.

For really high levels of filter performance crystal filters are used. Crystals are made from quartz, a naturally occurring form of silicon, although today's components are made from synthetically grown quartz. These crystals also use the piezoelectric effect and operate in the same way as ceramic filters but they exhibit much higher levels of Q and offer far superior degrees of selectivity. Being a resonant element they are used in many areas where an LC resonant element might be found. They are used in oscillators - many computers have crystal oscillators in them, but they are also widely used in high performance filters.

Normally crystal filters are made from a number of individual crystals. The most commonly used configuration is called the half lattice filter as shown in Figure 4. Further sections can be added to the filter to improve the performance. Often a filter will be quoted as having a certain number of poles. There is one pole per crystal, so a six pole crystal filter would contain six crystals and so forth. Many filters used in amateur communications receivers will contain either six or eight poles.

[image: image8.png]e
mm
I





Figure 4 A basic half lattice crystal filter section

Choosing the right bandwidth
It is important to choose the correct bandwidth for a give type of signal. It is obviously necessary to ensure that it is not too wide, otherwise unwanted off-channel signals will be able to pass though the filter. Conversely if the filter is too narrow then some of the wanted signal will be rejected and distortion will occur. As different types of transmission occupy different amounts of spectrum bandwidth it is necessary to tailor the filter bandwidth to the type of transmission being received. As a result many receivers switch in different filters for different types of transmission. This may be done either automatically as part of a mode switch, or using a separate filter switch. Typically a filter for AM reception on the short wave bands will have a bandwidth of around 6 kHz, and one for SSB will be approximately 2.5 kHz. For Morse reception 500 and 250 Hz filters are often used.

Summary
Selectivity is particularly important on today's crowded bands, and it is necessary to ensure that any receiver is able to select the wanted signal as well as it can. Obviously when signals occupy the same frequency there is little that can be done, but by having a good filter it is possible to ensure that you have the best chance or receiving and being able to copy the signal you want.

Receiver sensitivity
(Including noise, signal to noise ratio, SINAD, noise figure and measuring performance)
Receiver sensitivity is one of the key specifications of any radio. The two main requirements of any set are that it should be able to separate one station from another, i.e. selectivity, and signals should be amplified so that they can be brought to a sufficient level to be heard. As a result receiver designers battle with many elements to make sure that these requirements are fulfilled

Noise
Today technology is such that there is little problem in being able to achieve very large levels of amplification. This is not the limiting factor. In any receiving station the limiting factor is noise - weak signals are not limited by the actual signal level, but by the noise masks them out. This noise can come from a variety of sources. It can be picked up by the antenna or it can be generated within the receiver.
It is found that the level of noise that is picked up by a receiver falls as the frequency increases. At HF and frequencies below this the combination of galactic, atmospheric and man-made noise is relatively high and this means that there is little point in making a receiver superbly sensitive. Normally receivers are designed such that the internally generated noise is much lower than any received noise, even for the quietest locations.
At frequencies above 30 MHz the levels of noise start to reach a point where the receiver noise becomes far more important. By improving the noise performance of the set, it becomes possible to hear much weaker signals.
In terms of the receiver noise performance it is always the first stages or front end that is most crucial. At the front end the signal levels are at their lowest and even very small amounts of noise can be comparable with the incoming signal. At later stages in the set the signal will have been amplified and will be much larger. The same levels of noise as are present at the front end will be a much smaller proportion of the signal and will not have the same effect. Accordingly it is important that the noise performance of the font end is optimised for its noise performance.

Measuring noise performance
There are a number of ways in which the noise performance, and hence the sensitivity of a receiver can be measured. The most obvious method is to compare the signal and noise levels for a known signal level. Obviously the greater the difference between the signal and the unwanted noise, the better the sensitivity performance.
[image: image9.png]“The higher the signal i
above the noise, the
better the signalto
noise ratio

Freguency




Figure 1 Signal to noise ratio

The difference is normally shown as a ratio between the signal and the noise (S/N) and it is normally expressed in decibels. As the signal input level obviously has an effect on this ratio, the input signal level must be given. This is usually expressed in microvolts. Typically a certain input level required to give a 10 dB signal to noise ratio is specified.
A number of other factors apart from the basic performance of the set can affect the specification. The first is the actual bandwidth of the receiver. As the noise spreads out over all frequencies it is found that the wider the bandwidth of the receiver, the greater the level of the noise. Accordingly the receiver bandwidth needs to be stated.
Additionally it is found that when using AM the level of modulation has an effect. The greater the level of modulation, the higher the audio output from the receiver. When measuring the noise performance the audio output from the receiver is measured and accordingly the modulation level of the AM has an effect. Usually a modulation level of 30% is chosen for this measurement.
This method of measuring the performance is most commonly used for HF communications receivers. Typically one might expect to see a figure in the region of 0.5 microvolts for a 10 dB S/N in a 3 kHz bandwidth for SSB or Morse. For AM a figure of 1.5 microvolts for a 10 dB S/N in a 6 kHz bandwidth at 30% modulation for AM might be seen. 
SINAD
Whilst signal to noise specifications are often seen, another similar specification that is used is the SINAD measurement. This is most commonly used in conjunction with FM receivers, although there is no reason why it cannot be used for AM or even SSB. The measurement is similar to signal to noise ration, but includes distortion and is a ratio of signal plus noise plus distortion to noise plus distortion. To make the measurement a signal modulated with an audio tone is entered into the receiver. A measurement of the whole signal, i.e. the signal plus noise plus distortion is made. As the frequency of the tone is known and the regenerated audio is passed into a filter to remove the tone. The remaining noise and distortion is then measured.

[image: image10.png]#udio ton with il it
noise and dorton 304 diorson

Sgnalmoduized
i stone





Figure 2 Making a SINAD measurement

Normally the specification takes the form of a certain input level required to achieve a given SINAD. Typically a SINAD of 12dB is taken because this corresponds to distortion factor of 25%. A typical specification might be that a receiver has a sensitivity of 0.25 uV [microvolts] for a 12 dB SINAD. Obviously the lower the input voltage needed to achieve the given level of SINAD, the better the receiver performance.
Whilst the measurement is most commonly associated with FM equipment there is no reason why it cannot be used for AM, and in deed it often is. It can also be used for SSB, but it is necessary to ensure that the receiver is tuned into exactly the right frequency so that the audio tone is reconstituted with exactly the right pitch so that it can be properly notched out in the measurement.
To make the measurement a signal modulated with an audio tone is entered into the receiver. The frequency of the tone is known and the regenerated audio is passed into a filter to remove the tone. The remaining noise and distortion is then measured.

Noise Figure
For equipment that is used above 30 MHz a system known as Noise Figure is more widely used. However there is no reason why it cannot be used at any frequency and sometimes it is. The system is very versatile and can be used to measure the noise performance of a whole receiver, or a small part of a system like a preamplifier.
Essentially the measurement assesses the amount of noise each part of the system or the system as a whole introduces. If the system were perfect then no noise would be added to the signal when it passed through the system and the signal to noise ratio would be the same at the output as at the input. As we all know this is not the case and some noise is always added. This means that the signal to noise ratio at the output is worse than the signal to noise ratio at the input.
A figure known as the noise factor can be derived simply by taking the signal to noise ratio at the input and dividing it by the signal to noise ratio at the output. As the signal to noise ratio at the output will always be worse, i.e. lower, this means that the noise factor is always greater than one.
The noise factor is rarely seen in specifications. Instead the noise figure is always seen. This is simply the noise factor expressed in decibels.

[image: image11.png]nput signal = 5 N fiae Output signa

0 noise ratio A to noise ratio

=8

Noise figure = 10 \ngm%_




Figure 3 Noise figure

In the diagram S1 is the signal at the input, N1 is the noise at the input
and S2 is the signal at the output and N2 the noise at the output 

As an example if the signal to noise ratio at the input was 4:1, and it was 3:1 at the output then this would give a noise factor of 4/3 and a noise figure of 10 log (4/3) or 1.25 dB. Alternatively if the signal to noise ratios are expressed in decibels then it is quite easy to calculate the noise figure simply by subtracting one from another because two numbers are divided by subtracting their logarithms. In other words if the signal to noise ratio was 13 dB at the input and only 11 dB at the output then the circuit would have a noise figure of 13 - 11 or 2 dB.

Typical examples
The specifications of different pieces of equipment will vary quite widely. A typical HF receiver may have a noise figure of 15 dB of more and function quite satisfactorily. A better level of performance is not necessary because of the high level of atmospheric noise. However an amateur receiver used on Two metres, for example, might have a noise figure of 3 or 4 dB. Preamplifiers for this band often have a noise figure of around 1 dB. However it is interesting to note that even the best professional wide-band VHF UHF receivers may only have a noise figure of around 8 dB.

Summary
Receiver sensitivity is one of the vital specifications of any receiver. Whether measured as a signal to noise ratio, SINAD or noise figure it is essential that any receiver has a sufficient level of sensitivity. However this is not the complete story as other specifications are also important, as we shall see nest time when we look at dynamic range

Receiver Dynamic Range

(Including sensitivity, overload performance, intercept point, intermodulation distortion, and minimum discernible signal)

Sensitivity is one of the main specifications people look at when buying a receiver. However the sensitivity of a set is by no means the whole story. The specification for a set may show it to have an exceedingly good level of sensitivity, but when it is connected to an antenna its performance may be very disappointing because it is easily overloaded when strong signals are present, and this may impair its ability to receive weak signals.

The overall dynamic range of the receiver is very important. It is just as important for a set to be able to handle strong signals well as it is to be able to pick up weak ones. This becomes very important when trying to pick up weak signals in the presence of nearby strong ones. Under these circumstances a set with a poor dynamic range may not be able to hear the weak stations picked up by a less sensitive set with a better dynamic range. Problems like blocking, inter-modulation distortion and the like within the receiver may mask out the weak signals, despite the set having a very good level of sensitivity.

What is dynamic range?
The dynamic range of a receiver is essentially the range of signal levels over which it can operate. The low end of the range is governed by its sensitivity whilst at the high end it is governed by its overload or strong signal handling performance. Specifications generally use figures based on either the inter-modulation performance or the blocking performance. Unfortunately it is not always possible to compare one set with another because dynamic range like many other parameters can be quoted in a number of ways. However to gain an idea of exactly what the dynamic range of a receiver means it is worth looking at the ways in which the measurements are made to determine the range of the receiver. 

Sensitivity
The first specification to investigate is the sensitivity of a set. The main limiting factor in any receiver is the noise generated. For most applications either the signal to noise ratio or the noise figure is used as described in a previous issue of MT. However for dynamic range specifications a figure called the minimum discernible signal (MDS) is often used. This is normally taken as a signal equal in strength to the noise level. As the noise level is dependent upon the bandwidth used, this also has to be mentioned in the specification. Normally the level of the level of the MDS is given in dBm i.e. dB relative to a milliwatt and typical values are around -135 dBm in a 3 kHz bandwidth.

Strong signal handling
Although the sensitivity is important the way in which a receiver handles strong signals is also very important. Here the overload performance governs how well the receiver performance. 
In the ideal world the output of an amplifier would be proportional to the input for all signal levels. However amplifiers only have a limited output capability and it is found that beyond a certain level the output falls below the required level because it cannot handle the large levels required of it. This gives a characteristic like that shown in Fig. 1. From this it can be seen that amplifiers are linear for the lower part of the characteristic, but as the output stages are unable to handle the higher power levels the signals starts to become compressed as seen by the curve in the characteristic. 

[image: image12.png]Dutput

Amplifier in
comprassion

Linear region
of amplifier

Input




Fig. 1 A typical amplifier characteristic

The fact that the amplifier is non-linear does not create a major problem in itself. However the side effects do. When a signal is passed through a non-linear element there are two main effects which are noticed. The first is that harmonics are generated. Fortunately these are unlikely to cause a major problem. For a harmonic to fall near the frequency being received, a signal at half the received frequency must enter the amplifier. The front end tuning should reduce this by a sufficient degree for it not to be a noticeable problem under most circumstances.
The other problem that can be noticed is that signals mix together to form unwanted products. These again are unlikely to cause a problem because any signals which could mix together should be removed sufficiently by the front end tuning. Instead problems occur when harmonics of in-band signals mix together.

Third order products
Problems occur when harmonics of in-band signals mix together. It is found that a comb of signals can be produced as shown in Figure 2, and these may just fall on the same frequency as a weak and intersting station, thereby masking it out so it cannot be heard.

It is simple to calculate the frequencies where the spurious signals will fall. If the input frequencies are f1 and f2, then the new frequencies produced will be at 2f1 - f2, 3f1 - 2f2, 4f1 - 3f2 and so forth. On the other side of the two main or original signals products are produced at 2f2 - f1, 3f2 - 2f2, 4f2 - 3f1 and so forth as shown in the diagram. These are known as odd order inter-modulation products. Two times one signal plus one times another makes a third order product, three times one plus two times another is a fifth order product and so forth. It can be seen from the diagram that the signals either side of the main signals are first the third order product, then fifth, seventh and so forth.

To take an example with some real figures. If large signals appear at frequencies of 30.0 MHz and 30.01 MHz, then the inter-modulation products will appear at 30.02, 30.03, 30.4 ...MHz and 29.99, 29.98, 29.97 ..... MHz.

[image: image13.png]p——

Frequency




Fig. 2 Inter-modulation products

Blocking
Another problem that can occur when a strong signal is present is known as blocking. As the name implies it is possible for a strong signal to block or at least reduce the sensitivity of a receiver. The effect can be noticed when listening to a relatively weak station and a nearby transmitter starts to radiate, and the wanted signal reduces in strength. The effect is caused when the front-end amplifier starts to run into compression. When this occurs the strongest signal tends to "capture" the amplifier reducing the strength of the other signals. The effect is the same as the capture effect associated with FM signals. 
The amount of blocking is obviously dependent upon the level of the signal. It also depends on how far off channel the strong signal is. The further away, the more it will be reduced by the front end tuning and the less the effect will be. Normally blocking is quoted as the level of the unwanted signal at a given offset (normally 20 kHz) to give a 3 dB reduction in gain.

Dynamic range definition
When looking at dynamic range specifications, care must be taken when interpreting them. The MDS at the low signal end should be viewed carefully, but the limiting factors at the top end show a much greater variation tin the way they are specified. Where blocking is used a reduction of 3 dB sensitivity is normally specified, but in some cases may be 1 dB used. Where the inter-modulation products are chosen as the limiting point the input signal level for them to be the same as the MDS is often taken. However whatever specification is given, care should be taken to interpret the figures as they may be subtlety different in the way they are measured from one receiver to the next.

To gain a feel for the figures which may be obtained where inter-modulation is the limiting factor figures of between 80 and 90 dB range are typical, and where blocking is the limiting factor figures around 115 dB are generally achieved in a good receiver.

Designing for optimum performance
It is not an easy task to design a highly sensitive receiver that also has a wide dynamic range. To achieve this performance a number of methods can be used. The front-end stage is the most critical in terms of noise performance. It should be optimised for noise performance rather than gain. Input impedance matching is critical for this. It is interesting to note that the optimum match does not correspond exactly with the best noise performance. The amplifier should also have a relatively high output capability to ensure it does not overload. The mixer is also critical to the overload performance. To ensure the mixer is not overloaded there should not be excessive gain preceding it. A high level mixer should also be used (i.e. one designed to accept a high-level local oscillator signal). In this way it can tolerate high input signals without degradation in performance. Care should be taken in the later stages of the receiver to ensure that they can tolerate the level of signals likely to be encountered. A good AGC system also helps prevent overloading and the generation of unwanted spurious signals.
A receiver with a good dynamic range will be able to give a far better account of itself under exacting conditions than one designed purely for optimum sensitivity.

PAGE  
1

