ANN for fast and accurate design of spiral inductors
Rakhesh Singh Kshetrimayum, Member, IEEE , S. S. Karthikeyan and M. Vamsi Krishna 
 Radio Systems Laboratory, Department of Electronics and Communication Engineering,

Indian Institute of Technology, Guwahati, India,781039. Email: krs@iitg.ernet.in
Abstract --- Artificial Neural Network (ANN) has been employed for calculation of inductance and quality factor for spiral inductor. The number of turns (N), the width of the metal trace (W), the turn spacing (S), inner radius (
[image: image1.wmf]in

D

) and the frequency of operation (f) are taken as input parameters for the ANN model. It has been observed that as the number of training samples increases, the testing error decreases. The error in testing also decreases with the increase in the number of neurons in the hidden layer of the three layer multi-layer perceptron (MLP) network. The computational efficiency for this approach is very high in comparison to electromagnetic (EM) techniques, which takes more time. Usually it takes less than a fraction of a second for training and testing the 81 samples using ANN whereas the EM simulator takes few hours to generate a single data and few days to obtain 81 samples, which is a huge gain in terms of computational efficiency. The accuracy of this approach is also very close to the EM simulation technique within 1%-3% errors for Neuromodeler. A MATLAB based ANN has been developed for design of RF/microwave devices. This code gives a root mean square error (RMSE) for the same design of spiral inductor approximately 1%.   

I.   INTRODUCTION

The rapid development of commercial markets for wireless communication products over the past decade had led to an explosion of interest in improved circuit design approaches in the radio frequency (RF) and microwave areas. Electromagnetic (EM) simulation techniques for high frequency structures developed over the past decade have helped to bring the computer-aided design (CAD) for hybrid RF or microwave circuits to its current state of the art. But modeling still remains a major bottleneck for CAD of certain classes of RF/microwave circuits like coplanar waveguide (CPW) circuits, multi-layered circuits, integrated circuits (ICs), etc. Another factor in RF and microwave design is the increasing need for optimization–based design automation. There will be a trade-off between computation speed and accuracy in this approach. The recent development to overcome these issues is the use of artificial neural network (ANN) to the RF and microwave CAD problems. ANNs, are information processing systems with their design inspired by the studies of the ability of the human brain to learn from observations and to generalize by abstraction [1]. Training ANN configurations using the data obtained from the EM simulations develops an ANN model for each of these components. Such ANN models have been shown to retain the accuracy obtainable from EM simulators and at the same time exhibit the efficiency in computation. ANN is also suited for modeling active devices and for circuit optimization and statistical design.

Neural-network modeling is an unconventional and modern approach for RF and microwave device design [2]. Neural networks can be trained to learn the behavior of passive/active components/circuits [3]. A trained neural network can be used for high-level design, providing fast and accurate answers to the task it has learned [4]. Neural networks are attractive alternatives to conventional methods such as numerical modeling methods, which could be computationally expensive, or analytical methods that could be difficult to obtain for new devices, or empirical modeling solutions whose range and accuracy may be limited. No prior knowledge about the input/output mapping is required for ANN model development. Unknown relationships are inferred from the data provided for training. ANN can generalize, i.e., they can respond correctly to the new data that has not been used for the model development. ANN has the ability to model highly nonlinear as well as linear input/output mappings. ANN provides a general methodology for the development of accurate and computationally efficient electromagnetic trained ANN models for use in CAD of RF/microwave circuits, antennas and systems. 
In today’s portable wireless communications market, there is a demand for low cost, low power dissipation, high frequency IC building blocks that incorporate spiral inductors on the silicon substrate. The availability of spiral inductor models that meet the demands of the emerging wireless communication designs is a crucial element of a successful design flow. In the early 1990s, models were built using a discrete model library where a number of spiral inductors were fabricated and the measured data tabulated in lookup tables. This provided the end user with a model database that offered a limited number of spirals’ topologies and an even more limited parameter sets. This approach, even though it offered very good accuracy at the selected points, greatly limited design options. It was non-predictive and if the process was changed, the entire effort of manually building the model needed to be performed all over again. In conventional modeling techniques, numerical approaches such as solving algebraic and differential equations are computationally expensive to obtain accurate results. With empirical models including analytical expressions and equivalent circuits, the parasitic and coupling effects, especially in high frequency domain, are often missed [5].Yet for artificial neural network approach, once trained, the weights and biases will be fixed. The relationship between model output and input becomes a closed-form expression, which makes the computation time of the modeled parameters negligible [6]. Also owing to its accuracy in RF modeling [5], the artificial neural network-based approach is drawing intense attention in the above applications. In this paper, neural network based software known as Neuromodeler [7], developed Prof. Q. J. Zhang’s group at the Carleton Unievrsity, Canada is trained to model the inductance and quality factor of spiral inductor, which is a passive device. The number of turns (N), the width of the metal trace (W), the turn spacing (S), inner radius (
[image: image2.wmf]in

D

) and the frequency of operation (f) are taken as input parameters for the ANN model. Data generation for the spiral inductor is performed in a Finite Element Method based EM simulator. The inductance and quality factor are calculated from the Y- parameter values obtained from the EM simulator. A three layer Multi-layer Perceptron (MLP) is used for modeling of this device. We have also developed an MATLAB based ANN for designing RF/microwave devices. Both Neuromodeler as well as MATLAB based ANN for designing RF/Microwave devices give fast and accurate modeling of RF/Microwave devices. Usually it takes a fraction of a second for running these programs for RF/microwave modeling whereas the EM simulator takes hours and sometimes to generate those results.  
II. ANN MODELS FOR RF/MICROWAVE DESIGN
A. Neural network structures
A typical neural-network structure has at least two physical components, namely, the processing elements and the interconnections between them [1]. The processing elements are called neurons and the connections between the neurons are known as links or synapses. Every link has a corresponding weight parameter associated with it. Each neuron receives stimulus from other neurons connected to it, processes the information, and produces an output. Neurons that receive stimuli from outside the network are called input neurons, while neurons whose outputs are used externally are called output neurons. Neurons that receive stimuli from other neurons and whose outputs are stimuli for other neurons in the network are known as hidden neurons. Different neural-network structures can be constructed by using different types of neurons and by connecting them differently.

B. Generic notation
Let n and m represent the number of input and output neurons of a neural network. Let x be an n-vector containing the external inputs to the neural network, y be an m-vector containing the outputs from the output neurons, and w be a vector containing all the weight parameters representing various interconnections in the neural network. The definition of w, and the manner in which y is computed from x and w, determine the structure of the neural network.

C. Neural network modeling approach
The neural network can represent the behavior of any microwave device only after learning the original x – y relationship through a process called training. Samples of (x- y) data, called the training data, should first be generated from original device EM simulators or from the device measurements. Training is done to determine neural network weights w such that the neural model output best matches the training data. A trained neural network model can then be used during microwave design providing answers to the task it learned. The original EM based microwave device modeling problem cab be expressed as y=f(x) where f is the detailed EM based input–output relationship [2]. The neural network model for same device is defined as y= f (x, w). The neural-network approach can be compared with conventional approaches for a better understanding. The first type is the detailed modeling approach such as EM-based models for passive components and physics-based models for active components. The overall model, ideally, is defined by a well-established theory and no experimental data is needed for model determination. However, such detailed models are usually computationally expensive. The second type is an approximate modeling approach, which uses either empirical or equivalent-circuit-based models for passive and active components. The evaluation of approximate models is much faster than that of the detailed models. However, the models are limited in terms of accuracy and input parameter range over which they can be accurate. The neural-network approach is a new type of modeling approach where the model can be developed by learning from accurate data of the RF/microwave component. After training, the neural network becomes a fast and accurate model representing the original component behaviors.

D. MLP Neural Network
In the MLP neural network, the neurons are grouped into layers [1]. The first and the last layers are called input and output layers, respectively, and the remaining layers are called hidden layers. For example, an MLP neural network with an input layer, one hidden layer, and an output layer, is referred to as three-layer MLP (or MLP3). In the MLP network, each neuron processes the stimuli (inputs) received from other neurons. The process is done through a function called the activation function in the neuron, and the processed information becomes the output of the neuron. The universal approximation theorem states that there always exists a three-layer MLP neural network that can approximate any arbitrary nonlinear continuous multidimensional function to any desired accuracy. This forms a theoretical basis for employing neural networks to approximate RF/microwave behaviors, which can be functions of physical/geometrical/bias parameters. MLP neural networks are distributed models, i.e., no single neuron can produce the overall x – y relationship. For a given x, some neurons are switched on, some are off, and others are in transition. It is this combination of neuron switching states that enables the MLP to represent a given nonlinear input–output mapping. During training process, the MLPs weight parameters are adjusted and, at the end of training, they encode the component information from the corresponding x – y training data.

E. Network size and layers
For the neural network to be an accurate model of the problem to be learned, a suitable number of hidden neurons are needed. The number of hidden neurons depends upon the degree of non-linearity of f and the dimensionality of x and y (i.e., values of n and m). Highly nonlinear components need more neurons and smoother items need fewer neurons [3]-[4]. However, the universal approximation theorem does not specify as to what should be the size of the MLP network. The precise number of hidden neurons required for a given modeling task remains an open question. So, either by experience or a trial-and-error process is used to judge the number of hidden neurons. The appropriate number of neurons can also be determined through adaptive processes, which add/delete neurons during training. The number of layers in the MLP can reflect the degree of hierarchical information in the original modeling problem. In general, the MLPs with one or two hidden layers (i.e., three- or four-layer MLPs) are commonly used for RF/microwave applications.

III. MATLAB BASED ANN FOR RF/MICROWAVE DESIGN

Steps involved in the development of MLPNN model can be summarized as below:
1. Selecting and analyzing data for training and testing the model

2. Scaling of both training and test data

3. Selection of the no. of hidden neurons 

4. Creation of  feed-forward neural network

5. Selection of training algorithm 

6. Submission of training samples for computing feed forward response

7. Computing the training error and validation check of model

8. Training stops when training and validation errors are nearly equal

9. Testing of the neural model for calculating root mean square error (RMSE)
10. The number of neurons in the hidden layer are then changed and the entire process is repeated
11. The MLPNN exhibiting the lowest RMSE is selected as final model.
A MATLAB based ANN has been developed following the above steps in modeling of RF/microwave devices.

IV. Spiral Inductor RESULTS
The top view of a square spiral inductor fabricated in a sample CMOS process is shown in the figure 1. The geometry parameters of the spiral inductor are the number of turns (N), the width of the metal trace (W), the turn spacing (S), inner radius (
[image: image3.wmf]in

D

)

[image: image4.png]



Fig. 1. Top view of Spiral Inductor showing its dimensions

[image: image5.png]Cox





Fig. 2. A widely used circuit model of the spiral inductor 

A general circuit model of spiral inductors is depicted in figure 2. We will not discuss in detail the circuit model of spiral inductor in this paper (refer to [8]-[9] for a detailed explanation on spiral inductor circuit model). Instead we will directly generate data (input and output) of spiral inductor using EM simulator and use that data to train and test ANNs using Neuromodeler as well as MATLAB based ANN for RF/microwave device design. 
A. Data generation for ANN model of a Spiral Inductor

Square spiral inductor [10], the input parameters of the spiral inductor are the number of turns (N), the width of the metal trace (W), the turn spacing (S), inner radius (
[image: image6.wmf]in

D

) and the frequency of operation (f). The output parameters of the ANN model are inductance (L) and quality factor (Q). Data generation for the spiral inductor is performed in a Finite Element Method based EM simulator. 
The inductance and quality factor are calculated from the Y- parameter values obtained from the EM simulator. The equations used for the calculation of inductance and quality factor [11] are


[image: image7.wmf]1

2Im()

nn

nn

L

fY

p

-

=

××

                                                (1)


[image: image8.wmf]Im()

Re()

nn

nn

nn

Y

Q

Y

=

                                                             (2)

The square spiral inductor is developed on a silicon dioxide layer below which silicon substrate is present. The input port is excited using a lumped port. All of the spiral inductor structure is enclosed in an air box and the simulation is done in Driven-terminal mode to find the Y-parameters. Three spiral inductors with range 1.5 turn to 5.5 turn with step size 2 is simulated in driven terminal mode with inner radius varying from 30um to 90 um with step size 30, width of the spiral varying from 10um to 30um with step size 10, spacing varying from 1um to 5um with step size 2. It can be seen from Table 1, the ranges of input parameters are very different from one another. Hence all the input and output data are transformed to [-1, 1] by means of two-sided logarithmic scaling.
Table 1: Training and Test data

	Parameter
	Min
	Max
	Step

	Inner Radius(µm)
	30
	90
	30

	Width(µm)
	10
	30
	10

	Spacing(µm)
	1
	5
	2

	Number of Turns
	1.5
	5.5
	2

	Frequency(GHz)
	0.5
	20
	0.5


By using full- factorial method for sample distribution, we have 81 spirals for the modeling. Out of the 81 spirals, 75 spirals are used for training the neural network structure and 16 spirals are used for testing purpose. A three layer MLP network with 5 input nodes and 3 output nodes is constructed using the NeuroModeler [7] software to directly map the spiral inductor geometry characteristics to the inductor characteristics. The MLP neural model is then trained using the back- propagation algorithm until low rms training and testing errors of 0.01938693 and 0.02743347 are achieved. To demonstrate the accuracy and computational efficiency of the neural approach, extraction results for 16 test spirals are presented in the figures 3, 4, 5. The rms error between the EM simulated data and neural model output for the 16 spirals is below 5% which is the desired. In comparison to the EM simulator, which takes few hours to generate a single spiral data, ANN based approach takes fraction of a second to generate output once the network is trained, which is a huge gain in terms of computational efficiency.

B. NeuroModeler Results
Training error:

[image: image9.png]25

Training ertor vs o.of epachs

05

50

100

Epochs

150

200

250




Fig. 3. Graph of training error versus epochs

Training:

No. of epochs:      200

No. of samples:    2600

Training method: Back Propagation (MLP)

Training error:      0.01938693

[image: image10.png]an

50

45

0

E3

0

E3

Eil

15

10

Neural Model Output vs Test Data

— Data:Outputysuron, 1

—— Model Output euron, T

100

200

300

400 50 600 700 600 90D
Sarmple Nurmber

1000




Fig. 4. Graph showing the accuracy between Neural model output versus Test data for 
[image: image11.wmf]11

Q


[image: image12.png]50

45

0

E3

2

%30

euron,

Outpi e

10

Neural Model Output vs Test Data

—— Model-Output, euron,2
— DataOutputyuron,2

100

200

300

0 50 600
Sarmple Nurmber

700

800

900

1000




Fig. 5. Graph showing the accuracy between Neural model output versus Test data for 
[image: image13.wmf]21

Q


[image: image14.png]Output, euron, 3

x 10

Neural Model Output vs Test Data

— DataOutputyuron,3

—— Model Outputeuron,3

100

200

300 400 500 600 700 600 900
Sarmple Nurmber

1000




Fig. 6. Graph showing the accuracy between Neural model output versus Test data

Testing error:

No. of samples: 640

Test error        : 0.02743347

The MLP network which is constructed using the MATLAB software and then trained using the Back- propagation algorithm gave the rms training of 0.00437643. The training of three layered ANN is stopped after 68 epochs and converged to the error is 
[image: image15.wmf]2

10

-

:

 for the validation. The rms error between the EM simulated data and neural model output for the 16 spirals is between 1 to 3% which is the desired. 

C. MATLAB based ANN results
Training error:

[image: image16.png]Training-Blue Validation-Green Test-Red

Performance is 0.00437643, Goal is 0

10

il 0 a0 50
66 Epochs

60





Fig. 7. Graph of training and validation error versus no. of epochs

Testing error:
No. of samples:      640

Mean square error (MSE): 0.010612

V. CONCLUSION 

ANN has been employed for fast and accurate determination of the inductance and quality factor of spiral inductors. It has been observed that as the number of training samples increases, the testing error decreases, the error also decreases with the increase in the number of neurons in the hidden layer. The computational efficiency for this approach is very high when compared to EM technique, which takes more amount of time. Usually it takes less than a second for training and testing the 81 samples using ANN. In comparison to the EM simulator, which takes few hours to generate a single data and few days to generate 81 smaples, ANN based approach takes less a second to generate 81 results, which is a huge gain in terms of computational efficiency. The accuracy of this approach is also very close to the EM simulation technique within 1%-3% errors for the Neuromodeler and approximately 1% MSE for the MATLAB based ANN developed for RF/microwave design, which is quite accurate. In future we will try to explore other RF/microwave device design using ANN.   

REFERENCES

[1] S. Haykin, “Neural Networks: A comprehensive Foundation,” Prentice Hall of India, July 1998
[2] Q. J. Zhang and K. C. Gupta, “Neural Networks for RF and Microwave Design,” Artech House, July 2000

[3] Q. J. Zhang, K. C. Gupta, and V. K Devabhaktuni, “Artificial Neural Networks for RF and Microwave Design,” IEEE Trans. Microw. Theory Tech., pp. 1339-1350, Apr. 2003

[4] X. Ding, V. K. Devabhaktuni, B. Chattaraj, M. C. E. Yagoub, M. Deo, J. Xu, and Q.-J. Zhang, “Neural-Network Approaches to Electromagnetic-Based Modeling of Passive Components and their Applications to High-Frequency and High-Speed Nonlinear Circuit Optimization,” IEEE Trans. Microw. Theory Tech., pp. 436-449, Jan. 2004 

[5] Q. Zhang, K. C. Gupta, and V.K. Devabhaktuni, "Artificial neural networks for RF and microwave design - from theory to practice," IEEE Trans. Microwave Theory Tech., vol. 51, no. 4, 2003.

[6] G. L. Creech et al., "Artificial neural networks for fast and accurate EM-CAD of microwave circuits," IEEE Trans. Microwave Theory Tech., vol. 45, pp. 794-802, May 1997.

[7] Neuromodeler 1.5, Carleton University, Canada
[8] N.M. Nguyen and R.G. Meyer, “Si IC-compatible inductors and LC passive filters,” IEEE J. Solid-State Circuits, vol. 25, no. 4, pp. 1028- 1031, Aug. 1990.

[9] C.P. Yue, C. Ryu, J. Lau, T.H. Lee and S.S. Wong, “A physical model for planar spiral inductors on silicon,” Techn. Dig. IEDM, pp. 155-158, 1996.

[10] Tamura, S., and M. Tateishi, ‘‘Capabilities of a Four-Layered Feedforward Neural Network: Four Layer Versus Three,’’ IEEE Trans. Neural Networks, Vol. 8, 1997, pp. 251–255.

[11] K. Okada, H. Hoshino and H. Onodera, “Modeling and optimization of on-chip spiral inductor in S-parameter domain,” 2004 Int. Symp. Circuits and Systems, vol. 5, pp. 153-156, May 2004.















_1270459963.unknown

_1281584973.unknown

_1281585000.unknown

_1270460054.unknown

_1270517975.unknown

_1270453683.unknown

