PROCESS PLANNING FOR DIFFERENTIAL-HOUSING

S.No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters

0. 
Investment Casting
[image: image1.png]



Casting



1.
Facing


[image: image2.png]L

il

Focing

1





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 115 fpm

Feed 0.010 inch/tooth

Depth of cut 0.150 inches

2.
Turning
[image: image3.png]7
N

turning





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Facing
[image: image4.png][

. Facing





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 115 fpm

Feed 0.010 ipr

Depth of cut 0.150 inches

4.
Boring
[image: image5.png]i

Bori





Engine Lathe with 3 jaw chuck
High speed steel boring tool


Speed 60 fpm

Feed 0.008 ipr

Depth of cut o.1 inches.



5.
Chamfering
[image: image6.png]i

5. Chamfering




Engine Lathe with 3 jaw chuck


Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr



6.
Chamfering
[image: image7.png])
BN

6. Chamfering





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr

7.
Chamfering
[image: image8.png]AR

)

/. Chamfering




Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr



8.
Turning
[image: image9.png]|

i

5

8. Turning





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

9.
Chamfering
[image: image10.png]s

|

=

9. Chamfering





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

10.
Facing
[image: image11.png](@)

. Facing






Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 115 fpm

Feed 0.010 ipr

Depth of cut 0.150 inches

11.
Turning
[image: image12.png]11,

ful

Turning






Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

12.
Boring
[image: image13.png]17

!—‘;“4‘—\

Eoring






Engine Lathe with 3 jaw chuck
High speed steel boring tool
Speed 60 fpm

Feed 0.008 ipr

Depth of cut o.1 inches.

13.
Under cutting
[image: image14.png]T

§





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 85fpm

Feed 0.012 ipr

Depth of cut o.1 inches



14.
Chamfering
[image: image15.png]e

14 Chamfering





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55  fpm

Feed 0.003 ipr

15.
Chamfering
[image: image16.png]Ifl
[

15, Chamfering




Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

16.
Under cutting
[image: image17.png]I

v

6. Under Cutting




Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 85 fpm

Feed 0.012 ipr

Depth of cut o.1 inches

17.
Chamfering
[image: image18.png]anp

3

17. Chamfering




Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

18.
Facing
[image: image19.png]/ ﬁ/ﬁ

18, Focing





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 115 fpm

Feed 0.010 ipr

Depth of cut 0.0150 inches

19.
Drilling
[image: image20.png]19, Drilling




Engine Lathe using tail stock
Twist drill
Speed 85 fpm

Feed 0.012 ipr

20.
Reaming
[image: image21.png]=
=T =

20, Reaming





Engine Lathe using tail stock
Reamer
Speed 100 fpm

Feed 0.01ipr



21.
Drilling
[image: image22.png]2l Drilling




Engine Lathe using tail stock
Twist Drill
Speed 85 fpm

Feed 0.012 ipr

22.
Drilling
[image: image23.png]O

il

2. Drilling




Radial Drilling Machine
Twist Drill
Speed 85 fpm

Feed 0.012 ipr

23.
Reaming
[image: image24.png]23, Reaming





Radial Drilling Machine
Reamer
Speed 100 fpm

Feed 0.01 ipr

PROCESS PLANNING FOR SHAFT:

1.
Facing
[image: image25.png]Facing





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 115 fpm

Feed 0.010ipr

Depth of cut 0.150 inches 

2.
Rough Turning
[image: image26.png]Roughing





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015ipr

Depth of cut 0.150 inches

3.
Finish Turning
[image: image27.png]Finishing






Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 100 fpm 

Feed 0.015 ipr

Depth of cut 0.150

Inches

4. 
Chamfering
[image: image28.png]Chamfering





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr



5.
Parting
[image: image29.png]Sorting





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 50 fpm

Feed 0.01 ipr

6.
Chamfering
[image: image30.png]Fin

ﬁ

ol Chamfering





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

7.
Drilling
[image: image31.png]Drilling






Radial Drilling Machine
 Twist Drill
Speed 85 fpm

Feed 0.012 ipr

8.
Grinding
[image: image32.png]Grinding





Surface Grinding Machine
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



9.
Grinding
[image: image33.png]Grinding




Grinding Machine
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



10.
Grinding
[image: image34.png]Grinding




Grinding Machine
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass

11.
Grinding
[image: image35.png]Grinding





Grinding Machine
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



PROCESS PLANNING FOR BIG GEAR:

S. No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters



1.
Facing
[image: image36.png]FACING ONE END





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

2.
Drilling
[image: image37.png]DRILLING





Engine Lathe using tail stock
High Speed Steel Drilling Tool
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Facing
[image: image38.png]FACING OTHER END






Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

4.
Step Turning
[image: image39.png]STEP TURNING





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

5.


Grooving
[image: image40.png]GROOVING





Engine Lathe with 3 jaw chuck
High Speed Steel Grooving tool
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

6.
Hobbing
[image: image41.png]BBBBBBB





Hobbing machine
Bevel Gear Hob


7.
Slotting
[image: image42.png]SLOTTING





Slotting Machine
Slotting tool


8.
Grinding
[image: image43.png]N

=)

GRINDING




Grinding Machine
Grinding Wheel
Speed 550 fpm

In feed 0.003 inch per pass

PROCESS PLANNING FOR SMALL GEAR:

S. No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters

1.
Facing
[image: image44.png]Facing





Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

2.
Turning
[image: image45.png]Turning




 
Engine Lathe with 3 jaw chuck
Single point turning tool with carbide insert
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Drilling
[image: image46.png]Drilling




Engine lathe with three jaw chuck
Twist drill
Speed 85 fpm

Feed 0.012 ipr

4.
Radius Turning

[image: image47.png]Drilling






Bevel Gear Generator
Master Gear
Speed 90 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

5.
Grinding
[image: image48.png]Grinding





Grinding Machine
Form Grinding Wheel
Speed 550 fpm

In feed 0.003 inch per pass

6.
Hobbing
[image: image49.png]Hobbking





Gear Hobbing Machine
Bevel Gear Hob


PROCESS PLANNING FOR SMALL WASHER:

S. No
Operation
Drawing
Machine Tools


Cutting Tools
Cutting Parameters

1.
Blanking
[image: image50.png]Crzzrrrrrn




Mechanical Press
Progressive Die 


2.
Punching
[image: image51.png]Crzzrrrrrn




Mechanical Press
Progressive Die


3.
Forming
[image: image52.png]S




Mechanical Press
Progressive Die


PROCESS PLANNING FOR BIG WASHER:

S.No
Operation
Drawing
Machine Tools


Cutting Tools
Cutting Parameters

1.
Blanking
[image: image53.png]Crzzrrrrrn





Mechanical Press
Progressive Die 


2.
Punching
[image: image54.png]Crzzrrrrrn





Mechanical Press
Progressive Die


PROCESS PLANNING FOR LOCK PIN:

S.No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters 

1.
Sheet metal forming (rolling process)
[image: image55.png]



Rolling Mill


High speed steel rolls


PROCESS PLANNING FOR MASS PRODUCTION OF DIFFERENTIAL-HOUSING

S.No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters

0. 
Investment Casting
[image: image56.png]



Casting



1.
Facing


[image: image57.png]L

il

Focing

1





CNC Lathe
Single point turning tool with carbide insert
Speed 150 fpm

Feed 0.010 inch/tooth

Depth of cut 0.150 inches

2.
Turning
[image: image58.png]7
N

turning





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Facing
[image: image59.png][

. Facing





CNC Lathe
Single point turning tool with carbide insert
Speed 150 fpm

Feed 0.010 ipr

Depth of cut 0.150 inches

4.
Boring
[image: image60.png]i

Bori





CNC Lathe
High speed steel boring tool


Speed 60 fpm

Feed 0.008 ipr

Depth of cut o.1 inches.



5.
Chamfering
[image: image61.png]i

5. Chamfering




CNC Lathe


Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr



6.
Chamfering
[image: image62.png])
BN

6. Chamfering





CNC Lathe
Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr

7.
Chamfering
[image: image63.png]AR

)

/. Chamfering




CNC Lathe
Single point turning tool with carbide insert
Speed  55 fpm

Feed 0.003 ipr



8.
Turning
[image: image64.png]|

i

5

8. Turning





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

9.
Chamfering
[image: image65.png]s

|

=

9. Chamfering





CNC Lathe
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

10.
Facing
[image: image66.png](@)

. Facing






CNC Lathe
Single point turning tool with carbide insert
Speed 150 fpm

Feed 0.010 ipr

Depth of cut 0.150 inches

11.
Turning
[image: image67.png]11,

ful

Turning






CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

12.
Boring
[image: image68.png]17

!—‘;“4‘—\

Eoring






CNC Lathe
High speed steel boring tool
Speed 60 fpm

Feed 0.008 ipr

Depth of cut o.1 inches.

13.
Under cutting
[image: image69.png]T

§





CNC Lathe
Single point turning tool with carbide insert
Speed 85fpm

Feed 0.012 ipr

Depth of cut o.1 inches



14.
Chamfering
[image: image70.png]e

14 Chamfering





CNC Lathe
Single point turning tool with carbide insert
Speed 55  fpm

Feed 0.003 ipr

15.
Chamfering
[image: image71.png]Ifl
[

15, Chamfering




CNC Lathe
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

16.
Under cutting
[image: image72.png]I

v

6. Under Cutting




CNC Lathe
Single point turning tool with carbide insert
Speed 85 fpm

Feed 0.012 ipr

Depth of cut o.1 inches

17.
Chamfering
[image: image73.png]anp

3

17. Chamfering




CNC Lathe
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

18.
Facing
[image: image74.png]/ ﬁ/ﬁ

18, Focing





CNC Lathe
Single point turning tool with carbide insert
Speed 150 fpm

Feed 0.010 ipr

Depth of cut 0.0150 inches

19.
Drilling
[image: image75.png]19, Drilling




Engine Lathe using tail stock
Twist drill
Speed 85 fpm

Feed 0.012 ipr

20.
Reaming
[image: image76.png]=
=T =

20, Reaming





Engine Lathe using tail stock
Reamer
Speed 100 fpm

Feed 0.01ipr



21.
Drilling
[image: image77.png]2l Drilling




Engine Lathe using tail stock
Twist Drill
Speed 85 fpm

Feed 0.012 ipr

22.
Drilling
[image: image78.png]O

il

2. Drilling




Radial Drilling Machine
Twist Drill
Speed 85 fpm

Feed 0.012 ipr

23.
Reaming
[image: image79.png]23, Reaming





Radial Drilling Machine
Reamer
Speed 100 fpm

Feed 0.01 ipr

PROCESS PLANNING FOR MASS PRODUCTION OF SHAFT:

1.
Facing
[image: image80.png]Facing





CNC Lathe
Single point turning tool with carbide insert
Speed 150 fpm

Feed 0.010ipr

Depth of cut 0.150 inches 

2.
Rough Turning
[image: image81.png]Roughing





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015ipr

Depth of cut 0.150 inches

3.
Finish Turning
[image: image82.png]Finishing






CNC Lathe
Single point turning tool with carbide insert
Speed 100 fpm 

Feed 0.015 ipr

Depth of cut 0.150

Inches

4. 
Chamfering
[image: image83.png]Chamfering





CNC Lathe
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr



5.
Parting
[image: image84.png]Sorting





CNC Lathe
Single point turning tool with carbide insert
Speed 50 fpm

Feed 0.01 ipr

6.
Chamfering
[image: image85.png]Fin

ﬁ

ol Chamfering





CNC Lathe
Single point turning tool with carbide insert
Speed 55 fpm

Feed 0.003 ipr

7.
Drilling
[image: image86.png]Drilling






Radial Drilling Machine
 Twist Drill
Speed 85 fpm

Feed 0.012 ipr

8.
Grinding
[image: image87.png]Grinding





Surface CNC Lathe with grinding attachment
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



9.
Grinding
[image: image88.png]Grinding




CNC Lathe with grinding attachment
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



10.
Grinding
[image: image89.png]Grinding




CNC Lathe with grinding attachment
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass

11.
Grinding
[image: image90.png]Grinding





CNC Lathe with grinding attachment
Grinding wheel
Speed 550 fpm

In feed 0.003 inch per pass



PROCESS PLANNING FOR MASS PRODUCTION OF BIG GEAR:

S. No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters



1.
Facing
[image: image91.png]FACING ONE END





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

2.
Drilling
[image: image92.png]DRILLING





Engine Lathe using tail stock
High Speed Steel Drilling Tool
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Facing
[image: image93.png]FACING OTHER END






CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

4.
Step Turning
[image: image94.png]STEP TURNING





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

5.


Grooving
[image: image95.png]GROOVING





CNC Lathe
High Speed Steel Grooving tool
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

6.
Hobbing
[image: image96.png]BBBBBBB





Hobbing machine
Bevel Gear Hob


7.
Slotting
[image: image97.png]SLOTTING





Slotting Machine
Slotting tool


8.
Grinding
[image: image98.png]N

=)

GRINDING




CNC Lathe with grinding attachment
Grinding Wheel
Speed 550 fpm

In feed 0.003 inch per pass

PROCESS PLANNING FOR MASS PRODUCTION OF SMALL GEAR:

S. No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters

1.
Facing
[image: image99.png]Facing





CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

2.
Turning
[image: image100.png]Turning




 
CNC Lathe
Single point turning tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

3.
Drilling
[image: image101.png]Drilling




CNC Lathe
Twist drill
Speed 85 fpm

Feed 0.012 ipr

4.
Radius Turning

[image: image102.png]Drilling






CNC Lathe
Single point cutting tool with carbide insert
Speed 120 fpm

Feed 0.015 ipr

Depth of cut 0.150 inches

5.
Grinding
[image: image103.png]Grinding





CNC Lathe with grinding attachment
Form Grinding Wheel
Speed 550 fpm

In feed 0.003 inch per pass

6.
Hobbing
[image: image104.png]Hobbking





Gear Hobbing Machine
Bevel Gear Hob


PROCESS PLANNING FOR MASS PRODUCTION OF SMALL WASHER:

S. No
Operation
Drawing
Machine Tools


Cutting Tools
Cutting Parameters

1.
Blanking
[image: image105.png]Crzzrrrrrn




Mechanical Press
Progressive Die 


2.
Punching
[image: image106.png]Crzzrrrrrn




Mechanical Press
Progressive Die


3.
Forming
[image: image107.png]S




Mechanical Press
Progressive Die


PROCESS PLANNING FOR MASS PRODUCTION OF BIG WASHER:

S.No
Operation
Drawing
Machine Tools


Cutting Tools
Cutting Parameters

1.
Blanking
[image: image108.png]Crzzrrrrrn





Mechanical Press
Progressive Die 


2.
Punching
[image: image109.png]Crzzrrrrrn





Mechanical Press
Progressive Die


PROCESS PLANNING FOR MASS PRODUCTION OF LOCK PIN:

S.No
Operation
Drawing
Machine Tools
Cutting Tools
Cutting Parameters 

1.
Sheet metal forming (rolling process)
[image: image110.png]



Rolling Mill


High speed steel rolls


_1037204861.doc
[image: image1.png]Drilling








