Groom Table
A photo of a simple groom table. It's made out of plywood & 1 x 2 inch thick legs.

Table w/o top
here are just the legs of the table. Two pieces of nylon material keep the legs from opening too far. The legs are 36 inches long, and are spaced 17 inches apart.
[image: image1.jpg]



Underside of tabletop
Here is the underside of the tabletop. The two pieces of wood screwed to the bottom keep the top from sliding around while on the legs.
[image: image2.jpg]



Underside of Table
Here the top has been set on to the legs. The table fits snugly between the two dowel rods, which prevent it from moving around.
[image: image3.jpg]



Top View
Here is the top view of the table. The top measures 22 x 22 x 3 1/2 inches. The storage area in the back is 5 inches x 22 inches. The front of the table is tapered to 1 1/2 inches in height.
[image: image4.jpg]



The finished product
[image: image5.jpg]


measurements
[image: image6.jpg]4 Legs
36 inches long
spaced 17 inches apart.
1 Top
22 x 22 x 3 inches. The front is
only 1 1/2 inches high and the
storage area is 5 inches deep.
2 Doll Rods
1 inch thick, connect the
legs together.
2 Straps
20 inches long keep the legs
from opening too far.




