	 H.264 network video server >> NV904B >> NV904BT 

	[image: image1]
●     Top advanced arithmetic---- H.264 (MPEG4 Part 10) video compression.

●     High stability----Double system, double safety, more stable.

●     excellent image quality----D1 down to CIF resolution.

●     Professional audio----MP3 audio format, mono or stereo is selectable. 
●     Self-adjust the frame rate according to the network width to make network continuous transmission.

●     Support auto-connection when the network is cut.

●     Delay time is less than 150 MSEL(0.15 second) in LAN system.
●      Support real watermark technology so that compressed image file can’t be juggled.
●      Support Media Player to play the recording data.
 

 
NV901BT

NV902BT

NV904BT

Video

Compression: Hardware H.264 (MPEG4 Part10) 

Input interface: 1,2, 4 CH BNC 

Input level: VP-P=1V

Signal format: PAL/CCIR625 Line 50 Field NTSC/ EIA525 Line 60 Field
Output bit rate: 64Kbitps-8Mbitps

Resolution: 704*576(P) 704*480(N) 

704*288(P) 704*240(N) 352*288(P) 352*240(N)
Audio

Compression format: MP3

Input interface: 1,2,4 CH Microphone input( Vp-p=100Mv)

Sampling rate: 16K

Output bit rate: 8kbps－24Kbps
Communication

Web server: Built-in web server which is IE V6.0 or above

Dynamic IP: support internet connection assigned with dynamic IP address

Max. simultaneously users: 10

RS485 P/T/Z decoder interface and Keyboard interface、RJ-45 10M/100M Ethernet interface

Network Protocol: TCP/IP、UDP、MULTICAST

Network transmission mode: ISDN、ADSL、LAN, WAN

Alarm

Interface: 2 CH alarm input, 1 CH alarm output

motion detection: Yes 

pre-record: 5-30 seconds 

post-record: 30-120 seconds

Hardware

Power supply: AC 220V±10%，50Hz±1% 

Power consumption: ≤7Watts

Dimension：265×175×95mm(L*W*D)

Gross weight: 1.23KG

Temperate：-10—60℃
Humidity：<80%RH
SDK

network SDK , DEMO software and its source code

Optional Product

NV301B web decoder

Center management software

Frame rate per second

For transmit in ADSL(512K)

P:PAL  N:NTFC

NV901A 1CH : CIF 25F/30F(P/N)  D1: 6F/7F(P/N)

NV902A 2CH: CIF 50F/60F(P/N)  D1: 13F/15F

NV904B 4CH: CIF 100F/120F(P/N) D1: 25F/30F

NV901B 1CH: D1 25F/30F(P/N) 

NV902B 2CH: D1  50F/60F




