Source of Supply

The source of water commonly determines the nature of the collection, purification, transmission, and distribution works.

Municipal water supply may be derived from more than one source, the yield of available sources ordinarily being combined before distribution.

(1) Rainwater: from roofs, stored in cisterns, this is specially for individual supplies; and from larger, prepared watersheds, or catches, stored in reservoirs, for large communal supplies.

(2) Surface water: from streams, natural ponds, and lakes of sufficient size by continuous draft; or from their storage in reservoirs.

(3) Ground water: from natural springs, wells, and infiltration galleries.

Rainwater

· Rain is rarely the immediate provenance of municipal water supplies. Instead, the capture of rainwater is confined to farms and rural settlements usually in semiarid regions devoid of satisfactory ground or surface waters.

· Storage transforms the intermittent rainfall into a continuous supply. For municipal service, sheds or catches on ground naturally impervious or made tight by grouting, cementing, paving or similar means must usually be added.

· The storage to be provided in cisterns depends upon the distribution of rainfall. It varies with the length of dry spells and commonly approximate one-third to one-half the annual consumption.

Example:  
Make a rough estimate of the liters of water that can be caught by  100,000 m2 catchment area in a region where the mean annual rainfall is 400 mm.


Gross yield = 100,000 x 0.400 m3 annually = 40,000 m3 annually, or, = 110 m3/day.


Net yield approximates two-thirds gross yield = 26,666 m3 annually, or  73.333 m3/day, or  0.85 L/s.


About half the net yield, or 13,333 m3 must normally be stored in storage to tide the supply over dry spells.

Surface water

· Communities on or near streams, ponds, or lakes may take their supplies from them by continuous draft if stream flow and pond or lake capacity are high enough at all seasons of the year to furnish requisite water volumes.

· Collection works include ordinarily (1) an intake crib, gatehouse, or tower; (2) an intake conduit; and (3) in many places, a pumping station.

· On small streams serving communities of moderate size, an intake or diversion dam may create sufficient depth of water to submerge the intake pipe.

· Low stream flows are often left untouched, except in the case no other source of supply. To be of use, their annual discharge must equal or exceed the demands.

· Because of their dry season flows generally fall below the municipal requirements, their floodwaters must usually be stored in sufficient volume to assure an adequate supply.

· The amount of water so stored in reservoir must supply demands during season of unavailable stream flow. If draft is confined to a quarter year, for example, the reservoir must hold at least three-fourths of the annual supply.

Reservoir Storage

· The techniques for determining the storage volume required for a reservoir have been developed by Rippl which is an application of mass-balance approach.

· It is assumed that the only input is the flow into the reservoir (river flow, Qr) and that the only output is the flow out of the reservoir (flow abstraction for water supply, Qa).


[image: image20.wmf]Table 1: Calculation of Reservoir Volume for Storage, Data 1977-1978

T

Qr

Qr x T

Qa

Qa x T

D

S = (Qr-Qa) T

SD

S(+)

SD

S(-)

S

Qr

S

Qa

(Days)

(m

3

/s)

(m

3

)

(m

3

/s)

(m

3

)

(m

3

)

(m

3

)

(m

3

)

(m

3

)

(m

3

)

Jan

1

31

4.01

10,740,384

4

10,713,600

26,784

26,784

10,740,384

10,713,600

Feb

2

28

9.57

23,151,744

4

9,676,800

13,474,944

13,501,728

33,892,128

20,390,400

Mar

3

31

17.70

47,407,680

4

10,713,600

36,694,080

50,195,808

81,299,808

31,104,000

Apr

4

30

16.40

42,508,800

4

10,368,000

32,140,800

82,336,608

123,808,608

41,472,000

May

5

31

6.83

18,293,472

4

10,713,600

7,579,872

89,916,480

142,102,080

52,185,600

Jun

6

30

3.74

9,694,080

4

10,368,000

-673,920

-673,920

151,796,160

62,553,600

Jul

7

31

1.60

4,285,440

4

10,713,600

-6,428,160

-7,102,080

156,081,600

73,267,200

Aug

8

31

1.13

3,026,592

4

10,713,600

-7,687,008

-14,789,088

159,108,192

83,980,800

Sep

9

30

1.13

2,928,960

4

10,368,000

-7,439,040

-22,228,128

162,037,152

94,348,800

Oct

10

31

1.42

3,803,328

4

10,713,600

-6,910,272

-29,138,400

165,840,480

105,062,400

Nov

11

30

1.98

5,132,160

4

10,368,000

-5,235,840

-34,374,240

170,972,640

115,430,400

Dec

12

31

2.12

5,678,208

4

10,713,600

-5,035,392

-39,409,632

176,650,848

126,144,000

Jan

13

31

1.78

4,767,552

4

10,713,600

-5,946,048

-45,355,680

181,418,400

136,857,600

Feb

14

28

1.95

4,717,440

4

9,676,800

-4,959,360

-50,315,040

186,135,840

146,534,400

Mar

15

31

7.25

19,418,400

4

10,713,600

8,704,800

8,704,800

205,554,240

157,248,000

Apr

16

30

24.70

64,022,400

4

10,368,000

53,654,400

62,359,200

269,576,640

167,616,000

May

17

31

16.20

43,390,080

4

10,713,600

32,676,480

95,035,680

312,966,720

178,329,600

Jun

18

30

8.92

23,120,640

4

10,368,000

12,752,640

107,788,320

336,087,360

188,697,600

Jul

19

31

3.57

9,561,888

4

10,713,600

-1,151,712

-1,151,712

345,649,248

199,411,200

Aug

20

31

1.98

5,303,232

4

10,713,600

-5,410,368

-6,562,080

350,952,480

210,124,800

Sep

21

30

1.95

5,054,400

4

10,368,000

-5,313,600

-11,875,680

356,006,880

220,492,800

Oct

22

31

3.09

8,276,256

4

10,713,600

-2,437,344

-14,313,024

364,283,136

231,206,400

Nov

23

30

3.94

10,212,480

4

10,368,000

-155,520

-14,468,544

374,495,616

241,574,400

Dec

24

31

12.70

34,015,680

4

10,713,600

23,302,080

23,302,080

408,511,296

252,288,000

Note:

Qr =

River water flow

Qa =

Flow abstraction for water supply

Month





[image: image2.wmf]Qout

Qin

dt

dS

-

=



[image: image3.wmf]Qa

Qr

dt

dS

-

=



[image: image4.wmf])

(

)

(

dt

Qa

dt

Qr

dS

-

=



[image: image5.wmf])

(

)

(

t

Qa

t

Qr

S

D

-

D

=

D



[image: image6.wmf])

(

)

(

T

Qa

T

Qr

S

-

=

D



[image: image7.wmf]T

Qa

Qr

S

*

)

(

-

=

D


· Positive value of storage (
[image: image8.wmf]S

D

) or zero indicate there is enough water to meet the demand. If the storage (
[image: image9.wmf]S

D

) is negative, then the reservoir must have a capacity equal to the absolute value of cumulative storage (
[image: image10.wmf]S

D

) to meet the water demand.

· Table 1 and Figure 3 shows the application of Ripll method for the determination of Storage Reservoir.

[image: image1.wmf]dt

out

d

dt

in

d

dt

dS

)

(

)

(

-

=


Ground water

· Ground waters are often superior in quality to surface waters, are generally less expensive to develop for use, and usually provide a more certain supply.

· For these reasons, ground water is generally preferred as a source for municipal and industrial water supplies.

· Against these common advantages it must be noted that ground waters may be contaminated by toxic or hazardous materials leaking from landfills, waste treatment sites, or other sources (some natural) which may not be known.

· Groundwater-bearing formations, which are sufficiently permeable to yield usable quantities of water, are called aquifers.

· When an aquifer is not overlain by an aquiclude it is said to be unconfined. The top of the saturated zone in such an aquifers is called the water table of phreatic surface.

· Confined aquifers consist of a water-bearing layer contained between two less permeable layers.

· Water flow in an unconfined aquifer is analogous to that in an open channel, while flow in confined aquifers is analogous to that in pipes under pressure.

· The height to which water will rise in a tube penetrating a confined aquifer is called the piezometric surface. The height of this surface is a measure of the pressure in the aquifer.

[image: image16.wmf]
Wells

· Artesian wells are those which penetrate confined aquifers. Flowing wells are artesian wells in which the piezometric surface rises above the level of the ground.

· Steady flow in a confined aquifer was first described by Dupuit as the following equation (see Figure 4).

Q = 
[image: image11.wmf])

/

ln(

)

(

2

1

2

r

R

h

h

kD

-

p


where, 
k = permeability coefficient, m/s



D = thickness of artesian aquifer, m



T = kD = transmissibility, m2/s



h1 and h2 = height of piezometric surface above 

confining layer, m




R and r = radius from pumping well, m

Example:
A well in a confined aquifer with  a thickness of 15 m produces a flow of 25 L/s. The height of the piezometric surface is at an elevation of 114.6 m at an observation well 50 m away and at 112.5 m at an observation well 20 m away. Find k and T for the aquifer and estimate the height of the phreatic surface at the 0.5 m-diameter well.

0.025 = 
[image: image12.wmf])

20

/

50

ln(

)

5

.

112

6

.

114

)(

15

(

2

-

k

p


k = 0.0001 m/s = 10 m/day

T = k D = 15 * 10 m2/day

At the well,

0.025(86400) = 
[image: image13.wmf])

25

.

0

/

20

ln(

)

5

.

112

)(

150

(

2

h

-

p


h = 102.5 m

· Steady flow in an unconfined aquifer was then described by Theim as the following equation (see Figure 4).

Q = 
[image: image14.wmf])

/

ln(

)

(

2

1

2

2

r

R

h

h

k

-

p


[image: image17.wmf]
Example: 
A well in an unconfined aquifer is pumped at a rate of 25 L/s. The thickness of the aquifer is 15 m and the elevation of pheatric surface is 12.5 m above the underlying aquiclude at an observation well 20 m away from the pumped well and 14.6 m above at a well 50 m away. What is the value of k for this aquifer ?

0.025 = 
[image: image15.wmf])

20

/

50

ln(

)

5

.

12

6

.

14

(

2

2

-

k

p


k = 0.0001 m/s = 11.1 m/day

· Recharge of aquifers is sometimes deliberately augmented either to create barriers to other flows or to restore water to the aquifer for other use.

· Injecting water through recharge wells can prevent saltwater intrusion into coastal aquifers. Such injection wells raise the phreatic or piezometric surface and create freshwater flow toward the sea.

· Groundwater recharge through land disposal of treated sewage is an alternative which is considered in wastewater facility planning. A variety of industrial wastes can be handled in a similar fashion, provided they contain no materials which could contaminate the aquifer.

Springs

· Springs are usually developed to capture the natural flow of an aquifer.

· In favorable circumstances their yield can be increased by driving collecting pipes or galleries, more or less horizontally, into the water-bearing formations that feed them.

· Pollution generally originates close to the point of capture. It is prevented (1) by excluding shallow seepage water through encircling the spring with a watertight chamber penetrating a safe distance into the aquifer, and (2) by diverting surface run-off away from the immediate vicinity.

· Some spring yield less than 5 L/s; a few yield more than 200 L/s. However, 2000 L/s spring can be found under the background of large protected forest.

· Some springs are perennial, others are periodically or seasonally intermittent.

Infiltration Galleries

· Infiltration galleries laid more or less at right angles to the direction of flow can intercept groundwater traveling towards streams or lakes from neighboring upland.

· Some infiltration galleries yield as much as 25-50 L/s of water from 300-500 m long of gallery.

· Infiltration galleries are particularly useful in tapping aquifers of shallow depth or where deep saline waters are to be excluded.




� EMBED CDraw5  ���

















� EMBED CDraw5  ���











Web6-2; page 7

[image: image18.wmf][image: image19.wmf]_1065271302.unknown

_1065271941.unknown

_1065272142.unknown

_1065886010.unknown

_1067704602.unknown

_1065876880.unknown

_1065272021.unknown

_1065271387.unknown

_1065271210.unknown

_1065271245.unknown

_1064919973.unknown

_1064921438.unknown

_1065271067.unknown

_1064920166.unknown

_1064918974.unknown

