LANCIA DELTA HF 2.0 SPECIFICATIONS

	Body


	Type
	Hatchback or 3-door coupe

	No. of doors
	3-5

	No. of seats
	5


	The Engine 


	No. and arrangement of cylinders
	R4

	Arrangement of the engine
	In line, with dual anti-vibration counterweights

	Capacity (cc).
	1995

	Diameter of the cylinder and piston stroke
	84.0 x 90.0

	Compression Ratio
	8,0:1

	No. of valves per cylinder
	4

	Arrangement control shaft and valves
	Overhead cam shaft with double upper valve control 

	Fuel system
	Electronic multi-point injection 

	Turbocharger
	Turbo, Technics IHI 38mm or Garett, T45 (later versions)

	Turbo pressure
	IHI version 1,5 Overboost, 1,0 stabilized.

Garett version 0,9 Overboost 0,6 stabilized

	Output, PS (kW) at rpm
	187-193(137-142)/5750

	Maximum torque, Nm at rpm
	290-323/3500

	Type wheel drive
	Front-wheel drive

	No. speeds manual gearbox
	5

	Differential
	Electronically controlled viscous coupling (Viscodrive Version 1.4 – 3.0 [1994-1996])


	Running Gear


	Front Suspension
	Mc Pherson Strout,spring loaded cheek,transversal stabilizer

	Rear Suspension
	Longitudinal lever,cylindrical spiral spring,transversal stabilizer

	Power pump
	+

	Brakes front/rear
	Ventilating discs /discs (285/251 mm)

	A.B.S.
	Standard Bosch 5 channel

	Wheel, standard complete set
	205/50 ZR 15 (98-> 215/45 R16)


	Sizes and weights


	Wheelbase mm
	2640

	Track of wheels, front/rear, mm
	1498/1460

	Length x width x height, mm.
	4011 x 1801 x 1410

	Curb weight ready for the road, kg
	1382-1440

	Gross vehicle weight, kg
	1830

	Volume of a luggage compartment min/max, liters
	225/975 (350 Emergency wheel versions)

	Weight of the towed trailer, equipped brakes, kg .
	1300

	Capacity of a tank, l.
	60


	General data


	Acceleration up to 100 km/ch (sec)
	7.5

	Max. speed, km/h
	225

	Figures, liters/100 km (ECE)
	10,3

	Fuel grade
	Unleaded Premium, 98 RON


