REQUIREMENTS FOR EMERGENCY TOWING 
IN 1994 IMO AGREED AMENDMENTS TO THE SOLAS CONVENTION AS A NEW REGULATION CH V/15-1 (CH II-1/3-4 FROM 1/7/'98), WHICH CONTAINED THE FOLLOWING PROVISIONS.

· ALL "TANKERS" OF 20,000 DWT AND ABOVE CONSTRUCTED AFTER 1ST JANUARY 1996 ARE TO BE PROVIDED WITH AN EMERGENCY TOWING ARRANGEMEMNTS AT BOTH ENDS.

· ALL EXISTING TNAKERS OF 20,000 DWT AND ABOVE ARE TO BE PROVIDED WITH AN EMERGENCY TOWING ARRANGEMENTS AT BOTH ENDS AT THE FIRST SCHEDULED DRY-DOCKING AFTER 1ST JANUARY 1996, BUT IN ANY EVENT, NOT LATER THAN 1ST JANUARY 1999.

· THE TERM TANKERS INCLUDES OIL, CHEMICAL AND GAS TANKERS.

· THE MINIMUM COMONENTS OF THE EMERGENCY TOWING ARRANGEMENTS ARE TO COMPRISE OF THE FOLLOWING:

COMPONENT
FORWARD
AFT





TOWING PENNANT
OPTIONAL
REQUIRED

PICK-UP GEAR
OPTIONAL
REQUIRED

CHAFING GEAR
REQUIRED
DEPENDENT ON DESIGN

FAIRLEAD
REQUIRED
REQUIRED

STRONG POINT
REQUIRED
REQUIRED

ROLLER PEDENTAL LEAD
REQUIRED
DEPENDENT ON DESIGN

· THE FORWARD ARRANGEMENT, OF STRONG POINT, FAIRLEAD, CHAFING GEAR AND ROLLER PEDESTAL LEAD REFLECTS THE GUIDANCE PREVIOUSLY CONTAINED IN IMO ASSEMBLY RESOLUTION A.535 (13), WHICH ON MANY OIL TANKERS, MAY BE ACCOMODATED BY THE FITTINGS RECOMMENDED TO FACILITATE MOORING AT SPM'S.

· THE ARRANGEMENTS AFT CONTAINS A MAJOR NEW PROVISITIONINTRODUCED SINCE IMO ASSEMBLY RESOLUTION A.535 (13) WAS DEVELOPED, NAMELY THE REQUIREMENT FOR THE SHIP TO CARRY A PRE RIGGED TOWING PENNANT INCORPORATING PICK-UP GEAR. THE PICK-UP GEAR MUST BE CAPABLE OF BEING MANUALLY DEPLOYED BY ONE PERSONAND THE PENNANT MUST BE DEMONSTRATED TO BE CAPABLE OF FULL DEPLOYMENT WITHIN 15 MINUTES OF HARBOUR CONDITIONS.

