VIN Decoding

Only options available for the Daytona and Laser are listed to save space. Sorry, I don't have info on 91 -94 models, but I am guessing much of it is the same.

	
	1984
	1985
	1986
	1987
	1988
	1989
	1990

	1. Country of Origin
	1-US  

2.Canada 

3.Mexico 

4. Japan
	same
	same
	same
	same
	1-US  

2.Canada 

3.Mexico
	same

	2. Make
	B-Dodge  

C-Chrysler 

P-Plymouth
	same
	same
	same
	same
	same
	same

	3. Vehicle type
	3- Pass. Car
	same 
	same
	same
	same
	same
	same

	4. Safety System
	B-Manual Safety Belts
	same
	same
	same
	same
	X- Driver Airbag, Passenger Belt  

Y- Driver Airbag, Passenger Automatic Belt 

C- Automatic Belt A- Airbag B- Manual
	 

	5. Car Line
	?
	?
	A- V Daytona  

A- C Laser
	A- V Daytona
	A- V Daytona
	G- Daytona
	G- Daytona

	6. Series
	1-Economy 2-Low 4-High 5-Premium 6-Special/Sport 7- Performance Image
	 
	 

	7. Body Style
	4- 2 dr Hatchback
	same
	same
	same
	same
	same
	same

	8. Engine
	C- 2.2L  

D- 2.2L EFI 

E- 2.2L Turbo
	same
	C- 2.2L  

D- 2.2L EFI 

E- 2.2L Turbo 

K- 2.5L
	same
	C- 2.2L  

D- 2.2L EFI 

E- 2.2L Turbo 

K- 2.5L 

3- 3.0L
	same
	same

	9. Check Digit
	Used for verification, "1-9","0" or "X"
	 
	 
	 
	 
	 
	 

	10. Model Year
	E -84
	F-85
	G-86
	H-87
	J-88
	K- 89
	L- 90

	11. Assembly plant
	*
	 
	 
	 
	 
	 
	 

	12-17. Sequence Number
	**
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	


