Chapter 11 MedSurg

key terms

- Check against book before studying, I could have typed something wrong  - Linda

	intravenous (IV) therapy
	the administration of fluids, electrolytes, nutrients or medications by the venous route.



	phlebitis
	inflammation in the wall of a vein without clot formation



	butterfly needles
	“scalp vein or wing-tipped”

short, beveled needles with plastic flaps attached to the shaft.

The flaps (flexible) are held tight together to help ease insertion and then are flattened against skin to prevent dislodgment during infusion.



	intracath
	a plastic tube inserted into a vein



	angiocatheter 
	“angiocath” - a type of intracath with a metal stylet to pierce the skin and vein, after which the plastic catheter is threaded into the vein and the metal stylet is removed, leaving only the plastic catheter in the vein



	implantable port
	device made of a radiopaque silicone catheter and a plastic or stainless steel injection port with a self-sealing silicone-rubber septum.

(Only nurses who have been specially trained are allowed to access an iimplanted port bec. of the risk of infiltration into the tissue if needle placement is incorrect.)



	piggybacked
	intravenous medications may be piggybacked, connected to an existing IV to infuse concurrently.



	flow rate
	the volume of fluid to infuse over a set period to time as prescribed by the physician.

	hypervolemia
	increased circulating fluid volume



	infiltration
	seepage of foreign substances into the interstitial tissue, causing swelling and discomfort



	flashback
	Nurses can confirm that the needle is still in the vein by pinching the IV tubing.  Blood will rush into the tubing (flashback) if the needle is still in the vein.



	vesicant
	solution capable of causing tissue injury or destruction when it escapes into surrounding tissue



	drug incompatibilities
	cause an undesired chemical or physical reaction between a drug and a solution, between two drugs, or between a drug and the container or tubing.



	IV push
	“bolus”

- a method of administering a large dose of medication in a relatively short time, usually 1 to 30 minutes.


