Water Bottle Rocket 

LAUNCH PAD PLANS 

Designed for 2 Liter Bottles ONLY 

( by: Cubmaster Patrick O'Hara Pack #28 Galt, CA  5-14-1999 - E-mail: pohara@softcom.net )

SUPPLIES NEEDED

Glue using PVC cement

Make sure you put the valve stem into the 3/4" cap before you glue them all together

A)  1-10' (foot) x 3/4" (inch) SCH 40 PVC pipe  ( cut 1-4' - 4-12"  - 3-6" )
B)  3- PVC T's - 3/4" Slip x 3/4" Slip x 3/4" Slip

C)  1- PVC T - 3/4" Slip x 3/4" Slip x 1/2" Threaded

D)  5-3/4" PVC Slip caps ( glue one on each corner and use one for the valve stem )

F)  1-1/2" Threaded riser ( sand top thread down to fit 2-Ltiter bottle on )
G)  1-Auto Valve stem ( Drill 1/2" hole in 1 3/4" cap,  push up into and seat - BEFORE GLUING ) 

[image: image1.wmf]A-4'

B

B

C

A-12"

G

F

F

A-12"

D

D

A-6"

A-6"


SAFETY RULES

1) Please - USE 2 LITER BOTTLES ONLY, as they have a pressure capacity of 200psi

2) If possible, make sure kids have their own 'ROCKETS' ( keeps the fighting down ) 

3) Have each kid fill their own bottle up - between 1/4 to 3/4 - NO MORE THAN 3/4 full

4) Always have an adult run the air compressor that launches the 'ROCKETS'

5) Make sure the adult launching the bottles sets the 'rocket' on the riser ( to get 'the right fit' )

6) Make sure the adult launching the bottles keeps the kids at least 10 feet from launch pad

7) Try to launch from grass ( not dirt ) as dirt will become mud and WILL spray everywhere

BE SAFE  -  HAVE FUN  -  and  -  ENJOY !

� EMBED Word.Picture.8  ���








[image: image2.wmf]A-4'

B

B

C

A-12"

G

F

F

A-12"

D

D

A-6"

A-6"

_1082183920.doc
[image: image1.png]__\\}






A-4'







B







B







C







A-12"







GF







F







D







D











A-12"







A-6"







A-6"












