	SERDAR ÜLKER WEBSITE


Homepage   /   Meteoroloji
RUNWAY STATE MESSAGE ( 8 RAKKAMLI SNOWTAM AÇILIMI)

36150058

(36 pistinde, Dump, pistin %26-50’ni kaplıyor, kalınlığı 1mm’den az, frenleme iyi % 40 ve üzerinde)

DD / E / C / ee / BB

DD : RUNWAY (PİST KONUMLARI)  
00-36 50 to the “RIGHT” runway for paralles.

Designator
88= conditions reported apply to ALL RUNWAYS


99= no new report recived. Previous one repeated.

E: TYPE OF DEPOSIT (PİST ÜSTÜ BİRİKİNTİ)

0 Dry and Clean

1 Dump

2 Wet or Waterpatches

3 Rim or Frost

4 Dry snow

5 Wet snow

6 Slush

7 Ice

8 Compacted or rolled snow

9 Frozen rust ridges

/
type of deposit not reported

C : CONTAMINATION (BİRİKİNTİNİN PİSİ KAPLANMA ORANI) 
1 Less than 10%

2 11-25%

5 26-50%

9                                51-100%

/
not reported

ee : DEPTH (BİRİKİNTİNİN DERİNLİĞİ)


00 Less than 01 mm

01 01 mm

02 02 mm

03 03 mm

04 04 mm

.

.

89 89 mm

90 90 mm

92                                10 cm

93 
15 cm

94                                20 cm

95 
25 cm

96  
30 cm

97  
35 cm

98  
40 cm

99  
runway closed due to snow / slush / ice / large drift

//
not measured or operationally not important

BB BREAKING ACTION / FRICTION COEFFICIENT (FRENLEME KODU VE KATSAYISI)

95=good
=40 and above%

94=medium/good
=39-36%

93=medium
=35-30%

92=medium/poor
=29-26%

91=poor
=25 and below%

99=unreliable
=99

//
not reported or runway not operational

Not: Insome countries the word “SNOCLO” is added to actual weather report when the aerodrome is closed due to precipitation of the wintry type.

