Numerical Solution Of The Non Homogeneous Heat Equation

∂ u (x,t) /∂t = (1/R) ∂2 u (x,t) /∂x2  + P (1-exp(-αt)

by Reinaldo Baretti Machín,

and Alfonso Baretti Huertas





www.geocities.com/serienumerica4
www.geocities.com/serienumerica3
www.geocities.com/serienumerica2
www.geocities.com/serienumerica
www.geocities.com/fisicageneral2009
Para preguntas y sugerencias escriba a :

reibaretti2004@yahoo.com
References: 

  1.  Numerical integration of the complex function f(z)=1/(z^1/2 (z+1)) 

2. Numerical Inversion of Laplace Transform F(s), feb 26 2009 , D. G. Duffy page 58 
3. Solution of the Wave Equation by the Laplace Transform Method 
4. A First Course in Partial Differential Equations: with Complex Variables and Transform Methods (Dover Books on Mathematics) by H. F. Weinberger , page 115.

5. Applied mathematics for engineers and physicists by Louis Albert Pipes 

6. Transform Methods for Solving Partial Differential Equations, Second Edition by Dean G. Duffy , page 106.

7. Applied Complex Variables (Mathematics Series) by John W. Dettman

5. Complex Variables and the Laplace Transform for Engineers by Wilbur R. LePage
6. Schaum's Outline of Complex Variables, 2ed (Schaum's Outline Series) by Murray Spiegel
7. METHODS OF THEORETICAL PHYSICS [complete set]. by Philip Morse & Herman Feshbach
8. Prakash S. ,1967 :Nonsteady parallel viscous flow through a straight channel. Bull Cal. Math. Soc.,59 ,55-59.
The following problem is taken from ref 6.

Solve the non homogeneous heat equation

    ∂ u (x,t) /∂t = (1/R) ∂2 u (x,t) /∂x2  + P (1-exp(-αt) )     ,     (1)

            with    -1 ≤ x ≤ 1   , t ≥ 0        and boundary conditions (BC) 
u(-1,t)=u(1,t)=0 .The initial conditions is u(x,0)=0.

The analytical answer  obtained by transform methods in Ref. 6, is given as  

[image: image1.png](~1)" cos|(2n — 1)1rz/2] exp[—(2n — 1)*7°t/4R]
® ; (2n —1)%[a - (2n — 1)*7%/R] '




 .(2)
                                    .
The original source is in Ref 8.

The dimensions are (if u is temperature)  P~ temperature/time  , α ~ 1/time and 1/R ~ length2 /time.

Let  P = 100 kelvins/second , α =1./second   , R= 1.0 seconds/m2    .

We will solve (1) by the forward difference method. 

The length scale is L= 2 meters.
The time scales are 

t1 = R*L2 = 4. s  , t2 =1 / α =1. 
The forward difference method solution is 

u(x,t+∆t) = u(x,t) +(∆t) {   K (u(x+ ∆x,t)-2u(x,t) +u(x-∆x,t )  )/∆x2
                                                       + P (1-exp(-αt) )  }         .  (3)

There is a constraint on the length and time intervals,
                               k*∆t/(∆x2) <  1/2  
or                           (1/R) *∆t/(∆x2) <  1/2   .

RUN

tf=  0.5

 dt,dx,nt,nstep =  0.000159999996  0.0199999996 3125 100

t,x,temp(x,t),uxt=    0.500E+00   -0.100E+01    0.000E+00   -0.448E-06

t,x,temp(x,t),uxt=    0.500E+00   -0.900E+00    0.197E+01    0.197E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.800E+00    0.361E+01    0.361E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.700E+00    0.494E+01    0.494E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.600E+00    0.602E+01    0.602E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.500E+00    0.688E+01    0.688E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.400E+00    0.754E+01    0.754E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.300E+00    0.803E+01    0.803E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.200E+00    0.837E+01    0.837E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.100E+00    0.857E+01    0.857E+01

t,x,temp(x,t),uxt=    0.500E+00   -0.224E-07    0.863E+01    0.864E+01

t,x,temp(x,t),uxt=    0.500E+00    0.100E+00    0.857E+01    0.857E+01

t,x,temp(x,t),uxt=    0.500E+00    0.200E+00    0.837E+01    0.837E+01

t,x,temp(x,t),uxt=    0.500E+00    0.300E+00    0.803E+01    0.803E+01

t,x,temp(x,t),uxt=    0.500E+00    0.400E+00    0.754E+01    0.754E+01

t,x,temp(x,t),uxt=    0.500E+00    0.500E+00    0.688E+01    0.688E+01

t,x,temp(x,t),uxt=    0.500E+00    0.600E+00    0.602E+01    0.602E+01

t,x,temp(x,t),uxt=    0.500E+00    0.700E+00    0.494E+01    0.494E+01

t,x,temp(x,t),uxt=    0.500E+00    0.800E+00    0.361E+01    0.361E+01

t,x,temp(x,t),uxt=    0.500E+00    0.900E+00    0.197E+01    0.197E+01

t,x,temp(x,t),uxt=    0.500E+00    0.100E+01    0.000E+00    0.425E-07

[image: image2.emf]Solution of inhomogeneous heat equation

0.00E+00

1.00E+01

2.00E+01

3.00E+01

4.00E+01

5.00E+01

6.00E+01

-1.50E+00 -1.00E+00 -5.00E-01 0.00E+00 5.00E-01 1.00E+00 1.50E+00

X 

t=0.25 s

t=0.5 s

t=3 s

t=10


Fig 1. Solution of eq(1) for different times. The steady state has been reached at t=10 s.

FORTRAN code

c heat eq solution by forward difference k*dt/(dx**2) < 1/2

c inhomogenous PDE see D. G. Duffy PDE page 106

c    du/dt= K*d^2u/dx^2 +P*(1.-exp(-alfa*t))

c or    du/dt= (1./R)*d^2u/dx^2 +P*(1.-exp(-alfa*t))  ;K=1./R

c BC u(-1,t)=u(1,t)=0  IC is  u(x,0)=0.

      real k ,Lscale

c temp(x,t) is a one  dim array the array is for x values

      dimension Temp(0:200) ,Tempnew(0:200)

      data tf,nstep / 1.,100/

      data k,alfa,P,Lscale/1.,1.,100.,2./

      uinitial(x)=0.

      pi=2.*asin(1.)

      R=1./k

      xi=-Lscale/2.

      xf=Lscale/2.

      tsmall=amin1(R*Lscale**2,1./alfa)

      tf=10.*tsmall

c initial value of temp

      dx=(xf-xi)/float(nstep)

      dt= .8*(.5*dx**2/k)

      nt=int(tf/dt)

      print*,'tf=',tf

      print*,'dt,dx,nt,nstep =' ,dt,dx,nt,nstep

      print*,'  '

      do 30 i=0,nstep

      x=xi+dx*float(i)

      temp(i)=uinitial(x)

30    continue

c do 10 is the time loop

      do 10 j=1,nt

      t=dt*float(j)

      do 20 i=1,nstep-1

      x=xi+dx*float(i)

      tempnew(i)=temp(i)+ dt*( k*(temp(i+1)

     $ -2.*temp(i)+temp(i-1))/dx**2 +P*(1.-exp(-alfa*(t-dt))) )

20    continue

      tempnew(0)=0.

      tempnew(nstep)= 0.

c changes new and old

      do 50 i2=0,nstep

      temp(i2)=tempnew(i2)

50    continue

10    continue

      do 40 i=0,nstep,5

      x=xi+dx*float(i)

      call  trf(pi,P,k,R,alfa,x,t,utext)

      print 100, t,x,tempnew(i),utext

40    continue

100   format('t,x,temp(x,t),uxt=',4(3x,e10.3))

      stop

      end

      subroutine trf(pi,P,k,R,alfa,x,t,utext)

      real n ,k

      nterms=10

      steady1=(p*r/2.)*(1.-x**2)

      steady=steady1

      trans1=(p/alfa)*(1.-cos(x*sqrt(r*alfa))/cos(sqrt(r*alfa)))*

     $exp(-alfa*t)

      sum=steady+trans1

c     do 10 calculates the transient part of the solution

      do 10 i=1,nterms

      n=float(i)

      factor=(-1.)**n*64.*alfa*R*P/pi**3

      den=(2.*n-1.)**3*(4.*alfa-(2.*n-1.)**2*pi**2/r )

      argcos=(2.*n-1.)*pi*x/2.

      argexp= -(2.*n-1.)**2*pi**2*t/(4.*r)

      sum=sum+ factor*cos(argcos)*exp(argexp)/den

10    continue

      utext=sum

      return

      end

