Tier 3 Data Support

21 December 2003
Handset Settings

Rev 2

Tier 3 Data Support

Handset Settings

For errors or additions
please contact
data.support@cingular.com
ctl+click on 
Table of Contents items 
will jump to that location in the document

Hi All,
 
   Attached are the latest installments of the Tier 3 Handset Configuration documents.  These versions include many additional handsets, as well as new items such as the FastForward device.  There are also the usual corrections, updates, etc.  Note that the HandsetConfig.doc document has been renamed to HandsetInfo.doc.  This is intended to better reflect the contents of this document. If you have previous versions of these documents, they should be replaced.  
   These documents contain information and settings associated with using Cingular mobile handsets for data operations.  These would include SMS, MMS, Email, Internet access, Data Connect, etc.  They are intended as an aid in helping subscribers with data related problems.  There are two documents:

T3HandsetSettings.doc

This document contains the correct settings that various models of handsets need in order to successfully use SMS, MMS, Photo Messaging, Internet access, etc.  The settings are provided in a "flow chart" like manner that not only shows what the various settings should be set to, but also how to navigate the handset menu structure to get to them.  Verifying or changing the settings in the handset to match those in this document should insure that the handset is properly configured and not the source of any data related problems.  ALL of these settings should be verified, BEFORE escalating the problem to the next Cingular support tier.

T3HandsetInfo.doc

This document contains miscellaneous information on the various handset models that Cingular sells.  This includes information such as handset capabilities, manuals, menu paths for data settings, character input methods, misc. notes, etc.  In addition to the handsets that Cingular sells, it also contains some information on handsets not sold by Cingular.  These can usually be made to work with Cingular service.  The lack of documents and test models usually means this information is incomplete.  However it can be used as a starting place.  Additional information is added periodically based on new devices, or new data related settings that are useful.

 Please distribute these documents to any other Cingular employees who might benefit from them.  Also, please send any corrections or feedback on this document to data.support@cingular.com.
This document contains the correct settings that various models of handsets need in order to successfully use SMS, MMS, Photo Messaging, Internet access, etc.  The settings are provided in a "flow chart" like manner that not only shows what the various settings should be set to, but also how to navigate the handset menu structure to get to them.  Verifying or changing the settings in the handset to match those in this document should insure that the handset is properly configured and not the source of any data related problems.  ALL of these settings should be verified, BEFORE escalating the problem to the next Cingular support tier.

3LG


3LG-G4010


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


3LG-G4050


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


3Motorola


3T720


3CSD/GPRS/WAP Settings


3Messaging Settings


3Nokia


33100


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


33200


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


33300


3GPRS/WAP Settings


3Photo Messaging/WAP Settings


3Multi Media/WAP Settings


3CSD/WAP Settings


3Messaging Settings


33560


3CSD/WAP Settings


3Messaging Settings


33590


3CSD/GPRS/WAP Settings


3Messaging Settings


33595


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


33600/3650


3CSD/WAP Settings


3GPRS/WAP Settings


3MMS/WAP Settings


3Photo Messaging/WAP Settings


3Text Messaging Settings


3MMS Messaging Settings


3Email Messaging Settings


36200


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


36340i


3CSD/WAP Settings


3Messaging Settings


36560


3CSD/WAP Settings


3Messaging Settings


36800


3GPRS/WAP Settings


3CSD/WAP Settings


3Messaging Settings


3Samsung


3SGH-S307


3CSD/GPRS/WAP Settings


3Messaging Settings


3SGH-X427


3CSD/GPRS/WAP Settings


3Messaging Settings


3Siemens


3CT56


3CSD/GPRS/WAP Settings


3Messaging Settings


3S56


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3SL56


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3Sony/Ericsson


3P800


3GPRS Settings


3CSD/Messaging Settings


3T62u


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3T68i


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3T226


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3T306


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3T316


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3Messaging Settings


3T616


3CSD/GPRS Settings


3WAP Settings


3Messaging Settings


3Appendix





LG

LG-G4010

GPRS/WAP Settings

[image: image1.emf]Doesn't matter   Yes  (default)  

 


CSD/WAP Settings


Messaging Settings


LG-G4050

GPRS/WAP Settings


CSD/WAP Settings


Messaging Settings


Motorola

T720

CSD/GPRS/WAP Settings

These settings are factory set and not changeable.

Messaging Settings


Nokia

3100

GPRS/WAP Settings

CSD/WAP Settings


Messaging Settings


3200

GPRS/WAP Settings


CSD/WAP Settings


Messaging Settings


3300

GPRS/WAP Settings


Photo Messaging/WAP Settings


Multi Media/WAP Settings


CSD/WAP Settings


Messaging Settings


3560

CSD/WAP Settings


Messaging Settings


3590

CSD/GPRS/WAP Settings


Messaging Settings


3595

GPRS/WAP Settings


CSD/WAP Settings


Messaging Settings


3600/3650

CSD/WAP Settings


GPRS/WAP Settings


MMS/WAP Settings


Photo Messaging/WAP Settings


Text Messaging Settings


MMS Messaging Settings


Email Messaging Settings



6200

GPRS/WAP Settings


 CSD/WAP Settings


Messaging Settings


6340i

CSD/WAP Settings


Messaging Settings


6560

CSD/WAP Settings


Messaging Settings


6800

GPRS/WAP Settings


 CSD/WAP Settings


Messaging Settings



Samsung

SGH-S307

CSD/GPRS/WAP Settings


Messaging Settings


SGH-X427

CSD/GPRS/WAP Settings


Messaging Settings


Siemens

CT56

CSD/GPRS/WAP Settings


Messaging Settings


S56

CSD/GPRS Settings


WAP Settings


Messaging Settings


SL56

CSD/GPRS Settings


WAP Settings


Messaging Settings


Sony/Ericsson

P800

GPRS Settings

NOTE: The following are based on the manual and limited experience with a subscriber.  There may be errors.


CSD/Messaging Settings

No information is available at this time on these settings.

T62u

CSD/GPRS Settings


WAP Settings


Messaging Settings


T68i

CSD/GPRS Settings


WAP Settings

Messaging Settings


T226

CSD/GPRS Settings

WAP Settings

Messaging Settings


T306

CSD/GPRS Settings


WAP Settings

Messaging Settings


T316

CSD/GPRS Settings


WAP Settings



Messaging Settings


T616

CSD/GPRS Settings


WAP Settings


Messaging Settings


Appendix

LG-G4010�GPRS/WAP Settings





Home





Express





Home





Internet





Push messages





Dialup number





IP address





66.209.11.61





User ID





Password





Leave blank





Roam





wap.cingular





Menu





Profiles





Cache settings





Press �"Options"





Settings





Activate





Rename





Email server





Connect





User ID





WAP@CINGULARGPRS.COM





Leave blank





Invalid address





Edit address





http://





Password





CINGULAR1





WAP information





Reset profiles





Homepage





http://device.home





Display image





GPRS





Gateway information





GPRS� settings





Connect type





Continuous





Secure settings





Doesn't matter


Secure (default)





Bearer





Linger time





Doesn't matter


Yes (default)





300





LG-G4010�CSD/WAP Settings





Home





Express





Home





Internet





Push messages





Dialup number





IP address





66.209.11.61





User ID





Password





Leave blank





Roam





Menu





Profiles





Cache settings





Press �"Options"





Settings





Activate





Rename





Email server





Connect





User ID





WAP@CINGULARGPRS.COM





Leave blank





Invalid address





Edit address





http://





Password





CINGULAR1





Call type





ISDN





Call speed





14400





WAP information





Reset profiles





Homepage





http://device.home





Display image





DATA





Gateway information





Data� settings





Connect type





Continuous





Secure settings





Doesn't matter


Secure (default)





Bearer





Linger time





Doesn't matter


Yes (default)





300





14152441012 (or)�18472549271





Messages





LG-4010�Messaging Settings





Text messages





Voice messages





Name card





Templates





Write message





Message center number





Email gateway





Subject





Leave blank





Check memory





Bearer settings





36245





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Inbox





Outbox





Draft





Settings





Delete





GSM only





LG-G4050�GPRS/WAP Settings





Home





Express





Home





Internet





Push messages





Bookmarks





Homepage





http://device.home





APN





IP address





66.209.11.61





User ID





Password





Display image





Doesn't matter


Yes (default)





WAP@CINGULARGPRS.COM





CINGULAR1





Roam





GPRS





Bearer





wap.cingular





Menu





Profiles





Goto URL





Cache settings





Secruity certificates





Reset profiles





WAP information





Delete all





Press �"Optionws"





GPRS settings





Connect type





Continuous





Secure settings





Doesn't matter


Secure (default)





Settings





Activate





Rename





Delete





New profile





LG-G4050�CSD/WAP Settings





Home





Express





Home





Internet





Push messages





Bookmarks





User ID





Password





Call type





Dial-up Number





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





Roam





IP address





66.209.11.61





Menu





Profiles





Goto URL





Cache settings





Secruity certificates





Reset profiles





WAP information





Delete all





Press �"Optionws"





Settings





Activate





Rename





Delete





New profile





Homepage





http://device.home





Display image





Doesn't matter


Yes (default)





Data





Bearer





Data �settings





Connect type





Continuous





Secure settings





Doesn't matter


Secure (default)





Call speed





14400





Linger time





300





Messages 





LG-4050�Messaging Settings





Text messages





Voice messages





Multimedia messages





Name card





Templates





Write message





Message center number





Email gateway





Subject





Leave blank





Check memory





Voice Mail





Memory Status





Broadcast





Bearer settings





36245





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Inbox





Outbox





Drafts





Settings





Delete





GSM only





Message center number





Valid period





Auto download





+14047259015 (or)�+13123149870 (or)


+16363848815





Inbox





Outbox





Draft





Settings





Write





Message title





Slide duration





Alignment





Profiles





Leave blank�





Doesn't matter


15 (default)





Doesn't matter


Image top (default)





Doesn't matter


Maximum (default)





Doesn't matter


Confirm (default)





Home





Express





Roam





Delete all





Check this item





Message





Motorola T720�Messaging Settings





Cleanup





Reply Type





SMS





Press �"Menu" Key





Inbox





Voicemail





Create �Message





Quick Notes





Browser Msgs





Email Msgs





Drafts





Outbox





Memory Meter





Send/Receive Mail





Create �Message





Email Msg Setup





Inbox Setup





Voicemail Setup





Cleanup Now





Outbox Setup





Brwsr Msg Setup





Service Center No.





+14047259015 (or)�+13123149870 (or)


+16363848815





Doesn't matter


5 Days (default)





Active service Settings





Nokia 3100�GPRS/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Express





GPRS





Express





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





GPRS access point





wap.cingular





Authentication type





Secure





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Active service Settings





Nokia 3100�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM Data





Home





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





Authentication type





Normal





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





Data call speed





14400





Data call type





ISDN





14152441012 (or)�18472549271





Messages





Nokia 3100�Messaging Settings





Text Messages





E-mail





Multimedia Msgs.





Yes





Allow multimedia reception





Incoming multimedia messages





Service commands





Message settings





Text messages





Multimedia msgs.





Active multimedia settings





Edit multimedia settings








Allow advertisements





Settings name





Homepage





Session mode





Sending profile





Default profile





Message center number





Default recipient number





Use GPRS





Connection settings





Retrieve





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





Daily  reports





Default slide timing





GPRS





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





No





Other settings





Save sent  messages





Doesn't matter


Yes (default)








Doesn't matter


No (default)








Data bearer





Leave blank





Voice messages





E-mail address





Subject





Message





Send �e-mail





E-mail gateway number





121 (or)


36245





Normal (or)�Secure





+14047259015 (or)�+13123149870 (or)


+16363848815





Doesn't matter


Off (default)








Doesn't matter


Off (default)








GPRS access point





IP address





66.209.11.61





wap.cingular





Active service Settings





Nokia 3200�GPRS/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Express





GPRS





Express





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





GPRS access point





wap.cingular





Authentication type





Secure





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Active service Settings





Nokia 3200�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM Data





Home





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





Authentication type





Normal





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





Data call speed





14400





Data call type





ISDN





14152441012 (or)�18472549271





Messages





Nokia 3200�Messaging Settings





Text Messages





E-mail





Multimedia Msgs.





Yes





Allow multimedia reception





Incoming multimedia messages





Service commands





Message settings





Text messages





Multimedia msgs.





Active multimedia settings





Edit multimedia settings








Allow advertisements





Settings name





Homepage





Session mode





Sending profile





Default profile





Message center number





Default recipient number





Use GPRS





Connection settings





Retrieve





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





Daily  reports





Default slide timing





GPRS





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





No





Other settings





Save sent  messages





Doesn't matter


Yes (default)








Doesn't matter


No (default)








Data bearer





Leave blank





Voice messages





E-mail address





Subject





Message





Send �e-mail





E-mail gateway number





121 (or)


36245





Normal (or)�Secure





+14047259015 (or)�+13123149870 (or)


+16363848815





Doesn't matter


Off (default)








Doesn't matter


Off (default)








GPRS access point





IP address





66.209.11.61





wap.cingular





Active service Settings





Nokia 3300�GPRS/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Express





GPRS





Express





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





GPRS access point





wap.cingular





Authentication type





Secure





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





GPRS connection





Always online





Active service Settings





Nokia 3300�Photo Messaging/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Photo Messaging





GPRS





Photo Messaging





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





GPRS access point





isp.cingular





Authentication type





Normal





User name





ISP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





GPRS connection





Always online





Active service Settings





Nokia 3300�Multi Media/WAP Settings





Home page





Session Mode





http://mmsc.cingular.com





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Multi Media





GPRS





Multi Media





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





GPRS access point





wap.cingular





Authentication type





Normal





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





GPRS connection





Always online





Active service Settings





Nokia 3300�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM Data





Home





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





Authentication type





Normal





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





Data call speed





9600





Data call type





ISDN





14152441012 (or)�18472549271





Messages





Nokia 3300�Messaging Settings





Text Messages





E-mail





Multimedia Msgs.





Yes





Allow multimedia reception





Incoming multimedia messages





Service commands





Message settings





Text messages





Multimedia msgs.





Active multimedia settings





Edit multimedia settings








Allow advertisements





Settings name





Homepage





Session mode





Sending profile





Default profile





Message center number





Default recipient number





Use GPRS





Connection settings





Retrieve





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





GPRS access point





IP address





GPRS





66.209.11.61





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





wap.cingular





No





Other settings





Save sent  messages





Doesn't matter


Yes (default)








Doesn't matter


No (default)








Data bearer





Leave blank





Voice messages





E-mail address





Subject





Message





Send �e-mail





E-mail gateway number





121 (or)


36245





Normal (or)�Secure





+14047259015 (or)�+13123149870 (or)


+16363848815





Services





Settings





Active service Settings





Cookie settings





Nokia 3560�CSD/WAP Settings





Appearance settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Dial-up number





IP address





66.209.11.61





Authentication type





User name





Edit active service settings





Settings name





Home





14152441012 (or)�18472549271





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





Authority certificates





Home





Normal (or)�Secure





Messages 





Nokia 3560�Messaging Settings





Inbox





Create message





Chat





Create �e-mail





Templates





Archive





Sent items





Message settings





Voice messages





Delete messages





E-mail address





Subject





Message





Send �e-mail





E-mail gateway number





121 (or)


36245





Services





Settings





Active service Settings





Cookie settings





Nokia 3590�CSD/GPRS/WAP Settings





Appearance settings





Home page





Session Mode





http://device.home





Permanent





Off





Data bearer





GPRS access point





wap.cingular





Authentication type





User name





Edit active service settings





Settings name





Express





GPRS





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





Authority certificates





Express





IP address





66.209.11.61





GPRS connection





When needed





Normal (or)�Secure





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Dial-up number





IP address





66.209.11.61





Authentication type





Data call speed





Settings name





Home





9600





Automatic





Data call type





ISDN





Login type





Normal (or)�Secure





Data bearer





GSM data





User name





WAP@CINGULAR.COM





CINGULAR1





Password





Use these settings for configuring CSD only!





Connection security





+14047259015 (or)�+13123149870 (or)


+16363848815





Messages





Nokia 3590�Messaging Settings





Inbox





Write message





Chat





Write �e-mail





Templates





Pictures





Message settings





Voice messages





Multimedia Inbox





Outbox





Archive





My folders





Erase messages





Service command editor





Multimedia msg.





Sending profile





Text messages





Email Sett





Default profile





Message center number





Default recipient number





Use GPRS





No





Leave blank





Connection settings





Allow multimedia reception





Allow advertisements





Yes





Incoming multimedia messages





Retrieve





Active multimedia settings





Edit multimedia settings








Settings name





Homepage





Session mode





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





GPRS access point





IP address





GPRS





66.209.11.61





Authentication type





Secure





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





wap.cingular





Doesn't matter


No (default)








+14047259015 (or)�+13123149870 (or)


+16363848815





E-mail server





Rename sending profile





Message center number





121 (or)


36245





Use GPRS





No





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Settings





Active service settings





Cookie settings





Nokia 3595�GPRS/WAP Settings





Appearance settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





GPRS access point





wap.cingular





Authentication type





User name





Edit active service settings





Settings name





Express





GPRS





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





Authority certificates





Express





IP address





66.209.11.61





GPRS connection





Always online





Normal (or)�Secure





Connection settings





Bearer settings





Nokia 3595�CSD/WAP Settings





CINGULAR1





Dial-up number





IP address





66.209.11.61





Authentication type





Data call speed





9600





Automatic





Data call type





ISDN





Login type





Normal (or)�Secure





User name





WAP@CINGULAR.COM





CINGULAR1





Password





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Settings





Active service settings





Cookie settings





Appearance settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM data





Authority certificates





Home





Connection settings





Bearer settings





Messages 





Nokia 3595�Messaging Settings





Inbox





Write message





Chat





Write �e-mail





Templates





Pictures





Message settings





Voice messages





Multimedia Inbox





Outbox





Archive





My folders





Erase messages





Service command editor





Multimedia msg.





Sending profile





Text messages





Email Sett





Default profile





Message center number





Default recipient number





Use GPRS





No





Leave blank





Connection settings





Allow multimedia reception





Allow advertisements





Yes





Incoming multimedia messages





Retrieve





Active multimedia settings





Edit multimedia settings








Settings name





Homepage





Session mode





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





GPRS access point





IP address





GPRS





66.209.11.61





Authentication type





Secure





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





wap.cingular





Doesn't matter


No (default)








Use GPRS





No





E-mail server





Rename sending profile





Message center number





121 (or)


36245





+14047259015 (or)�+13123149870 (or)


+16363848815





+14047259015 (or)�+13123149870 (or)


+16363848815





Access points





Nokia 3600/3650�CSD/WAP Settings





GPRS





Phone





Tools





Settings





Connection





Call





Date and time





Normal (or)�Secure





Authentication type





Data call type





ISDN v.110





9600





Homepage





http://device.home





Maximum data speed





Gateway IP address





66.209.11.61





System





Security





Accessory





Data call





Internet-CSD Roam





Internet GPRS





Multimedia





Photo Messaging





Internet-CSD-Home





Connection Name





Internet-CSD-Home





Dial-up number





User name





No





Prompt password





Data bearer





GSM Data





User name





WAP@CINGULAR.COM





CINGULAR1





Password





+14047259015 (or)�+13123149870 (or)


+16363848815





Connection security





On (or)�Off





Permanent





Session mode





Access points





Nokia 3600/3650�GPRS/WAP Settings





GPRS





Phone





Tools





Settings





Connection





Call





Date and time





Normal (or)�Secure





Authentication type





Homepage





http://device.home





Gateway IP address





66.209.11.61





System





Security





Accessory





Data call





Internet-CSD Roam





Internet GPRS





Multimedia





Photo Messaging





Internet-CSD-Home





wap.cingular





Connection Name





Internet GPRS





Access point name





User name





WAP@CINGULARGPRS.COM





No





Prompt password





Data bearer





GPRS





User name





CINGULAR1





Password





Connection security





On (or)�Off





Permanent





Session mode





Access points





Nokia 3600/3650�MMS/WAP Settings





GPRS





Phone





Tools





Settings





Connection





Call





Date and time





Normal (or)�Secure





Authentication type





Homepage





http://mmsc.cingular.com





Gateway IP address





66.209.11.61





System





Security





Accessory





Data call





Internet-CSD Roam





Internet GPRS





Multimedia





Photo Messaging





Internet-CSD-Home





wap.cingular





Connection Name





Multimedia





Access point name





User name





WAP@CINGULARGPRS.COM





No





Prompt password





Data bearer





GPRS





User name





CINGULAR1





Password





Connection security





Off (must be off)





Permanent





Session mode





Access points





Nokia 3600/3650�Photo Messaging/WAP Settings





GPRS





Phone





Tools





Settings





Connection





Call





Date and time





Normal (or)�Secure





Authentication type





Homepage





http://device.home





Gateway IP address





66.209.11.61





System





Security





Accessory





Data call





Internet-CSD Roam





Internet GPRS





Multimedia





Photo Messaging





Internet-CSD-Home





isp.cingular





Connection Name





Photo Messaging





Access point name





User name





ISP@CINGULARGPRS.COM





No





Prompt password





Data bearer





GPRS





User name





CINGULAR1





Password





Connection security





Permanent





Session mode





On (or)�Off





Messaging





Nokia 3600/3650�Text Messaging Settings





Open





SIM messages





Create message





Service commands





Cell broadcast





Text message





Multimedia message





Message centers





SMS Settings





SMS Settings





E-mail





Press �"Options"





Settings





Help





Exit





Service msg.





Cell broadcast





Other 





Msg. center in use





+14047259015 (or)�+13123149870 (or)


+16363848815





Receive report





Message validity





Message sent as





Preferred connection





Reply via same ctr.





Text





GSM





No





Doesn't matter


No (default)








Doesn't matter


24 hours (default)








Messaging





Nokia 3600/3650�MMS Messaging Settings





Open





SIM messages





Create message





Service commands





Cell broadcast





Text message





Multimedia message





Always on





E-mail





Doesn't matter


No (default)








Press �"Options"





Settings





Help





Exit





Service msg.





Cell broadcast





Other 





Multimedia reception





On receiving msg.





Retrieve immediately





Allow anon. messages





Doesn't matter


No (default)








Receive ads





Doesn't matter


Yes (default)








Receive report





Deny report sending





Message validity





Image size





Default speaker





Doesn't matter


No (default)








1 weekly





Large





Loudspeaker





None





Preferred connection





MultiMedia





Secondary connection





Messaging





Nokia 3600/3650�Email Messaging Settings





Open





SIM messages





Create message





Service commands





Cell broadcast





Text message





Multimedia message





<mobile nmbr>@mobile.mycingular.com





E-mail





Immediately





Press �"Options"





Settings





Help





Exit





Service msg.





Cell broadcast





Other 





My mail address





Outgoing mail server





photo.mycingular.com





Send message





Doesn't matter


No (default)








Send copy to self





Include signature





Username





Password





Incoming mail server





Mailbox type





Doesn't matter


No (default)








Not known yet





photo.mycingular.com





POP3





Photo Messaging





Mailbox name





Photo Messaging





Access point in use





Photo Messaging





Mailboxes





Mailbox in use





Photo Messaging





cing





Security





APOP secure login





Off





Off





Active service settings





Nokia 6200�GPRS/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Settings name





Express





GPRS





Express





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





Normal (or)�Secure





GPRS access point





wap.cingular





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Edit active service settings





Menu





Nokia 6200�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM Data





Home





Bearer settings





Connection settings





Services





Settings





Security settings





Appearance settings





Service inbox settings





Authentication type





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





Data call speed





9600





Data call type





ISDN





14152441012 (or)�18472549271





Normal (or)�Secure





Active service settings





Menu





Menu





Nokia 6200�Messaging Settings





Text Messages





Voice messages





Yes





Allow multimedia reception





Incoming multimedia messages





Service commands





Message settings





Text messages





Multimedia msgs.





Active multimedia settings





Edit multimedia settings








Allow advertisements





E-mail server





Use GPRS





Settings name





Homepage





Session mode





Rename sending profile





Sending profile





Default profile





Message center number





Message center number





Default recipient number





Use GPRS





Connection settings





Retrieve





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





GPRS access point





IP address





GPRS





66.209.11.61





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





wap.cingular





No





No





121 (or)


36245





+14047259015 (or)�+13123149870 (or)


+16363848815





+14047259015 (or)�+13123149870 (or)


+16363848815





Other settings





Save sent  messages





Doesn't matter


Yes (default)








Doesn't matter


No (default)








Bearer settings





Leave blank





Email Settings





Normal (or)�Secure





Multimedia msgs.





Messages





Menu





Home





Active service Settings





Cookie settings





Nokia 6340i�CSD/WAP Settings





Appearance settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Dial-up number





IP address





66.209.11.61





Authentication type





Data call speed





Edit active service settings





Settings name





Home





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Data call type





Automatic





Authority certificates





Home





Normal (or)�Secure





Service Inbox





Settings





Clear the cache





Service inbox settings





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





Services





Nokia 6340i�Messaging Settings





Inbox





Write message





Write-�e-mail





Templates





Archive





Outbox





Service command editor





Voice messages





Delete messages





My folders





Message settings





E-mail server





Rename sending profile





Sending profile





Default profile





Message center number





Message center number





Default recipient number





+14047259015 (or)�+13123149870 (or)


+16363848815





Leave blank





Email Settings





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





Font size





Overwriting in outbox





Overwriting in inbox





Menu





Messages





Active service settings





Nokia 6560�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Settings name





Home





Home





Bearer settings





Connection settings





Services





Home





Security settings





Appearance settings





Edit active service settings





Menu





Settings





Bookmarks





Goto address





Clear cache





Authentication type





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





14152441012 (or)�18472549271





Normal (or)�Secure





Menu





Nokia 6560�Messaging Settings





Create message





Chat





Sent items





Inbox





E-mail server





Use GPRS





Rename sending profile





Sending profile





Default profile





Message center number





Message center number





Default recipient number





No





121 (or)


36245





+14047259015 (or)�+13123149870 (or)


+16363848815





+14047259015 (or)�+13123149870 (or)


+16363848815





Leave blank





Email Settings





Create SMS e-mail





Messages





Archive





Delete messages





Templates





Message settings





Voice messages





These settings are based on other Nokia handsets.  The prototype handset this was based on does not have these settings available.





Active service settings





Nokia 6800�GPRS/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Express





GPRS





Express





Bearer settings





Connection settings





Settings





Security settings





Appearance settings





Service inbox settings





Normal (or)�Secure





GPRS access point





wap.cingular





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Menu





Services





Nokia 6800�CSD/WAP Settings





Home page





Session Mode





http://device.home





Permanent





Connection security





Off





Data bearer





Edit active service settings





Settings name





Home





GSM Data





Home





Bearer settings





Connection settings





Security settings





Appearance settings





Service inbox settings





Authentication type





User name





WAP@CINGULAR.COM





CINGULAR1





Login type





Automatic





Password





IP address





66.209.11.61





Dial-up number





Data call speed





9600





Data call type





ISDN





14152441012 (or)�18472549271





Normal (or)�Secure





No





Display terminal window





Active service settings





Menu





Services





Settings





Nokia 6800�Messaging Settings





Text Messages





Voice messages





Multimedia Msgs.





Yes





Allow multimedia reception





Incoming multimedia messages





Service commansr





Message settings





Text messages





Multimedia msgs.





Active multimedia settings





Edit multimedia settings








Allow advertisements





Default recipient number





Use GPRS





Settings name





Homepage





Session mode





Rename sending profile





Sending profile





Default profile





Message center number





Message center number





Default recipient number





Use GPRS





Connection settings





Retrieve





Multimedia





Multimedia





http://mmsc.cingular.com





Permanent





Data bearer





GPRS access point





IP address





GPRS





66.209.11.61





Authentication type





User name





WAP@CINGULARGPRS.COM





CINGULAR1





Password





wap.cingular





No





No





121 (or)


36245





+14047259015 (or)�+13123149870 (or)


+16363848815





Other settings





Save sent  messages





Doesn't matter


Yes (default)








Doesn't matter


No (default)








Bearer settings





Leave blank





Email Settings





Normal (or)�Secure





Menu





Messages





+14047259015 (or)�+13123149870 (or)


+16363848815





Home





Samsung SGH-S307�CSD/GPRS/WAP Settings





Home





Favorite





Express





Games





Fun Box





Downloads





WAP Browser





http://





Settings





Clear Cache





Homepage URL





http://device.home





Bearer





IP address





66.209.11.61





Login name





Password





Data Call Type





Dial-up Number





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





User Name





Password





APN





wap.cingular





WAP@CINGULARGPRS.COM





CINGULAR1





Push Message





Roam





Profile Name





Express





Secure (or) Non-Secure





Security





GPRS





Homepage URL





http://device.home





Bearer





IP address





66.209.11.61





Profile Name





Home





Secure (or) Non-Secure





Security





Data





Menu





Messages





Samsung SGH-S307�Messaging Settings





Inbox





Write New





Outbox





Settings





Templates





SMS Settings





Email Setings





Service Center





Default Destination





Default Type





Factory set





My Objexts





Voice Mail





Memory Status





Broadcast





Default Validity





Setting Name





SMS Settings





Factory set





Leave blank





Service Center





Default Destination





Default Type





+14047259015 (or)�+13123149870 (or)


+16363848815





Factory set





Default Validity





Setting Name





Email Settings





Factory set





36245





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Home





Samsung SGH-X427�CSD/GPRS/WAP Settings





Home





Favorite





Express





Games





Fun Box





Downloads





WAP Browser





http://





Settings





Clear Cache





Homepage URL





http://device.home





Bearer





IP address





66.209.11.61





Login name





Password





Data Call Type





Dial-up Number





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





User Name





Password





APN





wap.cingular





WAP@CINGULARGPRS.COM





CINGULAR1





Push Message





Roam





Profile Name





Express





Secure (or) Non-Secure





Security





GPRS





Homepage URL





http://device.home





Bearer





IP address





66.209.11.61





Profile Name





Home





Secure (or) Non-Secure





Security





Data





Menu





More games





Messages 





Samsung SGH-X427�Messaging Settings





Inbox





Write New





Outbox





Settings





Templates





SMS Settings





Email Setings





Service Center





Default Destination





Default Type





Factory set





Delete all





Voice Mail





Memory Status





Broadcast





Default Validity





Setting Name





SMS Settings





Factory set





Leave blank





Service Center





Default Destination





Default Type





+14047259015 (or)�+13123149870 (or)


+16363848815





Factory set





Default Validity





Setting Name





Email Settings





Factory set





36245





+14047259015 (or)�+13123149870 (or)


+16363848815





Menu





Settings





Home





Data Counter





Siemens CT56�CSD/GPRS/WAP Settings





Roam





CSD Settings





GPRS Settings





WAP profiles





Home





Display





Setup





Keys





Audio





Phone setup





Connectivity





Network





Security





Accessories





Clock





HTTP profiles





SMS profiles





Fax/data mod.





WAP Parameters





Profile name





9203





Port





Linger time





300





Home page





http://device.home





2nd address





Leave blank





IP address





66.209.11.61





Login name





Password





Call type





Dial-up no.





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





Login name





Password





APN





Deactivate GPRS





Yes





wap.cingular





WAP@CINGULARGPRS.COM





CINGULAR1





Press �"Edit"





Press �"Edit"





Press �"Edit"





Press �"Edit"





Siemens CT56�Messaging Settings





Sending profiles





SMS





Service center





Recipient





Status report





Leave blank





+14047259015 (or)�+13123149870 (or)


+16363848815





Press �"Options"





Press �"Change settings"





Use GPRS





Doesn't matter


No Check (default)





No Check





Data Counter





WAP profiles





Display





Setup





Keys





Audio





Phone setup





Connectivity





Network





Security





Accessories





Clock





HTTP profiles





SMS profiles





Fax/data mod.





Data Counter





Siemens S56�CSD/GPRS Settings





Roam





CSD Settings





GPRS Settings





IrDA





Home





Display





Setup





Keys





Tones/ Alerts





Phone setup





Connectivity





Network





Security





Accessories





Clock





Data Services





Fax/data mod.





Login name





Password





Call type





Dial-up no.





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





Login name





Password





APN





wap.cingular





WAP@CINGULARGPRS.COM





CINGULAR1





HTTP Profile





Mail,Java,Rms





MMS/Wap





Bluetooth





Press �"Options"





Insert From





Change Sett.





Rename





Press �"Edit"





Press �"Edit"





Press �"On"





Home





Siemens S56�WAP Settings





Roam





Home





Homepage





Internet





Resume





Bookmarks





Inbox





Exit browser





Profiles





Profile name





9203





Port





GPRS Linger time





300





Home page





http://device.home





Leave blank





IP address





66.209.11.61





CSD Linger time





Surf & Fun





Press �"Edit"





Home





Connection profile





Messages 





Siemens S56�Messaging Settings





New SMS





New E-mail





New MMS.





Draft





Inbox





SMS





MMS





Service Center





Recipient





SMS via GPRS





No Check





+14047259015 (or)�+13123149870 (or)


+16363848815





E-mail





Cingular MMS





Leave blank





Unsent





Sent





SMS Archive





Text Modules





Msg. Setup





Voice Message





SMS





Activate





Change Sett.





Send & Save





Doesn't matter


No Check (default)





Activate





Change Sett.





Rename





Expire Time





Def. Priority





Def. Duration





Retrieval





Doesn't matter


1 week (default)








Doesn't matter


005.0 (default)








Normal





MMS RelaySrv.





IP Settings





Conn. Profile





http://mmsc.cingular.com





Doesn't matter


Manual (default)








IP Address





66.209.11.61





Port





9203





2nd address





66.209.11.61





2nd Port





9203





Home





Not normally used.�Subscriber must get settings from their ISP





Press �"Options"





Press �"Options"





Data Counter





Siemens SL56�CSD/GPRS Settings





Roam





CSD Settings





GPRS Settings





IrDA





Home





Display





Setup





Keys





Tones/ Alerts





Phone setup





Connectivity





Network





Security





Accessories





Clock





Data Services





Fax/data mod.





Login name





Password





Call type





Dial-up no.





14152441012 (or)�18472549271





ISDN





WAP@CINGULAR.COM





CINGULAR1





Login name





Password





APN





wap.cingular





WAP@CINGULARGPRS.COM





CINGULAR1





HTTP Profile





Mail,Java,Rms





MMS/Wap





ProSlide





Home





Siemens SL56�WAP Settings





Roam





Home





Homepage





Internet





Resume





Bookmarks





Inbox





Exit browser





Profiles





Profile name





9203





Port





GPRS Linger time





300





Home page





http://device.home





Leave blank





IP address





66.209.11.61





CSD Linger time





Surf & Fun





Press �"Edit"





Home





Connection profile





Messages 





Siemens SL56�Messaging Settings





New SMS





New E-mail





New MMS.





Draft





Inbox





SMS





MMS





Service Center





Recipient





SMS via GPRS





No Check





+14047259015 (or)�+13123149870 (or)


+16363848815





E-mail





Cingular MMS





Leave blank





Unsent





Sent





SMS Archive





Text Modules





Msg. Setup





Voice Message





SMS





Activate





Change Sett.





Send & Save





Doesn't matter


No Check (default)





Activate





Change Sett.





Rename





Expire Time





Def. Priority





Def. Duration





Retrieval





Doesn't matter


1 week (default)








Doesn't matter


005.0 (default)








Normal





MMS RelaySrv.





IP Settings





Conn. Profile





http://mmsc.cingular.com





Doesn't matter


Manual (default)








IP Address





66.209.11.61





Port





9203





2nd address





66.209.11.61





2nd Port





9203





Home





Not normally used.�Subscriber must get settings from their ISP





Press �"Options"





Press �"Options"





Control Panel





Connections tab





Internet Accounts





Username





Password





wap.cingular





WAP@CINGULARGPRS.COM





Address





CINGULAR1





Ericsson P800�GPRS Settings





GPRS Timeout





600





Account Name





Accounts Tab





Express





Connection Type





GPRS





Server Tab





Use proxy





No





Proxy Tab





Use Auto IP Address





Use Auto DNS Address





IPv4





Yes





IP Version





Yes





Login Tab





Enable PPP Extensions





Authentication





Yes





Secure





Other Tab





Advanced





Other Tab





Dial-up Timeout





300





Preferred Mode





Automatic





Settings Tab





Data Comm





Data Accounts





APN





SMS Access





User ID





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Pwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T62u�CSD/GPRS Settings





Pref. Service





GPRS and DialUp





GSM





Phone Number





User ID





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Pwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








Express





Cingular Home





These settings are factory set on the Tier 3 test handsets and do not actually have these menu items.  If you encounter a handset that does, please contact Tier 3 so this document can be updated. 





Ericsson T62u�WAP Settings





Push Inbox





WAP Settings





WAP Profiles





Homepage





Browser Tab





Resume





Clear Cache





The test handsets Tier 3 has do not actually have this menu item.  If you encounter a handset that does, please contact Tier 3 so this document can be updated. 








Common





Most T62u's are factory set beyond this point.





Outbox





Templates





Unsent Box








+14047259015 (or)�+13123149870 (or)


+16363848815





Send New





SMS Inbox





Doesn't matter


On (default)








Email GatewSMS





Set on Send





Service Centers





121 (or)


36245





Message Type





Long Messages





Messages tab





Ericsson T62u�Messaging Settings





Email





SMS





Call Voicemail





Not normally used.�Subscriber must get settings from their ISP





Chat





Connect Icon





Data Comm





Data Accounts





APN





SMS Access





User id





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T68i�CSD/GPRS Settings





Pref. Service





Cingular Home





GPRS and GSM





GSM





Phone Number





User id





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Passwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








Express





Photo Messaging





APN





User id





isp.cingular





ISP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Data Compr





Doesn't matter





Cingular Roam





Header Compr





Doesn't matter





Quality of Service





Doesn't matter





These settings are factory set on the Tier 3 test handsets and do not actually have these menu items.  If you encounter a handset that does, please contact Tier 3 so this document can be updated. 





Push Inbox





Homepage





Advanced





Options





Common





Advanced





Delete





Chg. Homepage





Second Account





Ask Before Use





Select Account





Ask me





Home





User id





Security





Password





Show Pictures





Off





Doesn't matter


On (default)








LEAVE BLANK





LEAVE BLANK





WAP Profiles





Homepage





Web Services





Rename





Homepage





Connect Using





EXPRESS





IP Address





66.209.11.61





Ericsson T68i�WAP Settings





Select Profile





Homepage





http://device.home





Press �"OK"





Messages Icon





Service Centers





Email Gateways





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





SMS Type





Set on Send





Subject





Doesn't matter


Off (default)








Long Messages





Doesn't matter


On (default)








Ericsson T68i�Messaging Settings





Email





Text





Call Voicemail





Validity Period





Read Report





Doesn't matter


1 Week (default)





Doesn't matter


Off (default)








Delivery Report





Doesn't matter


Off (default)








Auto Delete





Doesn't matter


Off (default)








Content Alert





Doesn't matter


On (default)








Message Server





http://mmsc.cingular.com





WAP Profile





Homepage





Select





Create New





Options





Options





Multimedia





Not normally used.�Subscriber must get settings from their ISP





Options





Push Inbox





Chat





Inbox





Doesn't matter


On (default)








Clear Pushinbox





Push Access





Doesn't matter


Prompt (default)








Auto Download





Doesn't matter


Prompt (default)








Connect Icon





Data Comm





Data Accounts





APN





SMS Access





User ID





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Pwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T226�CSD/GPRS Settings





Pref. Service





Home





GPRS and GSM





GSM





Phone Number





User ID





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Pwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








Express





Photo Messaging





APN





User ID





isp.cingular





ISP@CINGULARGPRS.COM





Password





CINGULAR1





Pwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T226�WAP Settings





Push Inbox





Home





Advanced





Options





Common





Advanced





Delete





Chg. Homepage





Second Account





Roam





Ask Before Use





Select Account





User ID





Security





Password





Show Pictures





Off





LEAVE BLANK





LEAVE BLANK





WAP Profiles





Homepage





Cingular Icon





Rename





Home





Connect Using





EXPRESS





IP Address





66.209.11.61





Homepage





http://device.home





Press �"YES"





Ask me





Home





Doesn't matter


On (default)








Messages Icon





Service Centers





Email Gateways





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





SMS Type





Set on Send





Subject





Doesn't matter


Off (default)








Long Messages





Doesn't matter


On (default)








Ericsson T226�Messaging Settings





Email





Text





Call Voicemail





Validity Period





Read Report





Doesn't matter


1 Week (default)





Doesn't matter


On (default)








Delivery Report





Doesn't matter


On (default)








Auto Delete





Doesn't matter


Off (default)








Content Alert





Doesn't matter


On (default)








Message Server





http://mmsc.cingular.com





WAP Profile





Home





Select





Create New





Options





Options





MMS





Not normally used.�Subscriber must get settings from their ISP





Options





Push Inbox





Chat





Inbox





Doesn't matter


On (default)








Clear Pushinbox





Push Access





Doesn't matter


Prompt (default)








Connect Icon





Data Comm





Data Accounts





APN





SMS Access





User id





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T306�CSD/GPRS Settings





Pref. Service





Home





GPRS and GSM





GSM





Phone Number





User id





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Passwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








Express





Photo Messaging





APN





User id





isp.cingular





ISP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Push Inbox





Homepage





Advanced





Options





Common





Advanced





Delete





Chg. Homepage





Second Account





Ask Before Use





Select Account





Ask me





Home





User id





Security





Password





Show Pictures





Off





Doesn't matter


On (default)








LEAVE BLANK





LEAVE BLANK





WAP Profiles





Homepage





Web Services





Rename





Homepage





Connect Using





EXPRESS





IP Address





66.209.11.61





Ericsson T306�WAP Settings





Select Profile





Homepage





http://device.home





Press �"OK"





Messages Icon





Service Centers





Email Gateways





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





SMS Type





Set on Send





Subject





Doesn't matter


Off (default)








Long Messages





Doesn't matter


On (default)








Ericsson T306�Messaging Settings





Email





Text





Call Voicemail





Validity Period





Read Report





Doesn't matter


1 Week (default)





Doesn't matter


Off (default)








Delivery Report





Doesn't matter


Off (default)








Auto Delete





Doesn't matter


Off (default)








Content Alert





Doesn't matter


On (default)








Message Server





http://mmsc.cingular.com





WAP Profile





Homepage





Select





Create New





Options





Options





Multimedia





Not normally used.�Subscriber must get settings from their ISP





Options





Push Inbox





Chat





Inbox





Doesn't matter


On (default)








Clear Pushinbox





Push Access





Doesn't matter


Prompt (default)








Auto Download





Doesn't matter


Prompt (default)








Advan. Settings





Connect Icon





Data Comm





Data Accounts





APN





User id





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T316�CSD/GPRS Settings





Pref. Service





Home





GPRS and GSM





GSM





Phone Number





User id





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Passwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








EXPRESS





Photo Messaging





APN





User id





isp.cingular





ISP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





SMS Access





Push Inbox





Home





Advanced





Options





Common





Advanced





Delete





Chg. Homepage





Second Account





Roam





Ask Before Use





Select Account





Ask me





Home





User ID





Security





Password





Show Pictures





Off





Doesn't matter


On (default)








LEAVE BLANK





LEAVE BLANK





WAP Profiles





Homepage





Web Services


Icon





Rename





Home





Connect using





EXPRESS





IP Address





66.209.11.61





Ericsson T316�WAP Settings





Homepage





http://device.home





Press �"YES"





Messages Icon





Service Centers





Email Gateways





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





SMS Type





Set on Send





Subject





Doesn't matter


Off (default)








Long Messages





Doesn't matter


On (default)








Ericsson T316�Messaging Settings





Email





Text





Call Voicemail





Validity Period





Read Report





Doesn't matter


1 Week (default)





Doesn't matter


On (default)








Delivery Report





Doesn't matter


On (default)








Auto Delete





Doesn't matter


Off (default)








Content Alert





Doesn't matter


On (default)








Message Server





http://mmsc.cingular.com





WAP Profile





Home





Select





Create New





Options





Options





MMS





Not normally used.�Subscriber must get settings from their ISP





Options





Push Inbox





Chat





Inbox





Doesn't matter


On (default)








Clear Pushinbox





Push Access





Doesn't matter


Prompt (default)








Connect Icon





Data Comm





Data Accounts





APN





User ID





isp.cingular





ISP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





Ericsson T616�CSD/GPRS Settings





Pref. Service





Home





GPRS and GSM





GSM





Phone Number





User ID





14152441012 (or)�18472549271





WAP@CINGULAR.COM





Password





CINGULAR1





Passwd. Request





Off





Data Rate





14.4 Kbits/s





Dial Type





ISDN





DNS Address





…�NO IP ADDRESS HERE








Photo Messaging





EXPRESS





APN





User ID





wap.cingular





WAP@CINGULARGPRS.COM





Password





CINGULAR1





Passwd. Request





Off





Allow Calls





Automatic





IP Address





…�NO IP ADDRESS HERE





DNS Address





…�NO IP ADDRESS HERE








Advan. Settings





Normal (or)�Secure





Authentication





Quality of Service





Doesn't matter





SMS Access





Homepage





Home





WAP Profiles





Advanced





Delete





Chg. Homepage





Second Account





Roam





http://device.home





Ask Before Use





Select Account





Ask me





Home





User ID





Security





Password





Show Pictures





Off





Doesn't matter


On (default)








LEAVE BLANK





LEAVE BLANK





WAP Options





Connect�icon





Rename





Home





Connect using





EXPRESS





IP Address





66.209.11.61





Ericsson T616�WAP Settings





Press �"OK"





Messages Icon





Service Centers





Email Gateways





+14047259015 (or)�+13123149870 (or)


+16363848815





121 (or)


36245





SMS Type





Set on Send





Email Subject





Doesn't matter


Off (default)








Long Messages





Doesn't matter


On (default)








Ericsson T616�Messaging Settings





Email





Text





Call Voicemail





Validity Period





Read Report





Doesn't matter


1 Week (default)





Doesn't matter


Off (default)








Delivery Report





Doesn't matter


Off (default)








Restrict Messages





Doesn't matter





Content Alert





Doesn't matter


On (default)








Message Server





http://mmsc.cingular.com





WAP Profile





Home





Select





Create New





Options





Options





MMS





Not normally used.�Subscriber must get settings from their ISP





Options





WAP Push





Chat





Inbox





Doesn't matter


On (default)








Clear Inbox





Allow Push





Doesn't matter


Prompt (default)








Doesn't matter


Off (default)











Company Confidential

13/86
Cingular Wireless

