CU Tables
(coefficient of utilization)
The following CU tables are included for several reasons: 
1 They represent common lamp/fixture/trim combinations (luminaires) used in residential lighting. Notice the description for each luminaire. Some are included to demonstrate just how inefficient they are!
2 By using the Lumen Method, a comparison can be made to the effectiveness of each type as indicated by the number of luminaires required.
3 The CU table information can be substituted for a similar luminaire that has no published CU table. This is standard practice in lighting design and is much better than guessing.
4 For example, if the designer was planning to use a Prescolite 7" recessed can with a clear specular Alzak trim outfitted with a 50W PAR30 FL, He might use CU table E (which uses a similar Halo can with a 75W PAR30 Lamp). The lumen value for the 50W lamp would be inserted for the 75W in the Lumen Method Window. As long as the lamp type is the same, the CU table simply reports what percentage of the light actually reaches the WorkPlane. This is handy when finding the CU table which can sometimes be a bit "HOLY GRAILISH".
LightCalc Main Page
	---


	Table A
	[image: image1.png])






[image: image2.png]A [Ccer 80% 70% 50% 30%
WCR | 70% | 60% [30% | 10% | 70% [ 50% | 30% [ 10% | 50% | 30% [ 10% | 50% | 30% [ 10%
7 |76 | 73|70 [ e8| 73| 71 | 68 | 66 | 66 | 64 | 62 | 61| 60| 58
2 | 70| 65| 60 | 56 | 67 | 62 | 58 | 65| 68 | 55| 52 | 54 | 62 | 49
3 | 64| 57 |52 | 48| 62| 56| 53 | 47 | 52 | 48 | 45 | 49| 45| a3

R| 4 [ 59| 51 |45 [ 41 | &7 | 60 | a4 [ 40| 47 | 42| 39 | a4 | 40| 37

C| 5 54| 4639 35| 2] 44 | 39 | 34| 42| 37 | 33| 39| 35 | 32

R| 6 | 50| 41|35 |30 48] 40| 34 [ 30| a7 | 33 | 29| 35 31| 28
7 |45 |37 |31 |27 | 44| 36 [ 30| 26| 34| 29| 25| 32| 38|24
@ | 43| 33|27 23| 41| 32|27 23| 0| 25|28 24|
9 | 39| 30| 24 20| 38| 29| 23|20 27| 22| 19| 26| 21|18
70 | 36| 27 | 21 |18 | 35| 6 | 21 |7 | 25 | 20 [ A7 | 24| 19 | 16
FCR 20%





	Manufacturer:
	Lithonia

	Model:
	LB240A

	Lamps:--2 each
	4' F40T12 34W

	Lumens per Lamp:
	2,650

	Total Lumens:
	5,300


	
	Description:
This luminaire uses two 4' standard fluorescent T12 tubes, is ceiling mounted, and has a wrap around prismatic lens.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
3 luminaires
	Luminaires Per Square Foot Factor 

3 luminaires ÷ 150 sq. ft. =
.02


	---


	Table B
	[image: image3.png][ L






[image: image4.png]30%

58
50
)
38
EE]
]
5
7]
19
17

60
52
46
I

36
7]
]
5
7]
0

61

55
49
)
40
3
]
]
%
)

50%

61

52
45
3
34
£l
%
FE]
£
17

63
55
48
a2
a
EE]
]
%
FE]
]

65
58
52
a7
a2
£
34
El

]
z

0%

64
54
a7
40
35
£l
o
FE]
£
18

20%

67
58
50
)
3
34
3
o
)
]

69
61

55
49
)
a0
3
7]
]
7

72
66
61

56
51

a7
a
a0
a7
3

80%

0% | 70% | 60% | 30% | 10% | 50% | 30% [10% | 50% | 30% [ 10%

66
55
a7
I

35
a

o
FE]
£
18

68
59
51

45
3
3
El

o
)
]

T

63
56
50
45
a

a7
]
]
i

74
68
63
58
53
]
45
4
3
£

WCR | 70% | 50% [ 30%

0

FCR

B [ CCR





	Manufacturer:
	Lithonia

	Model:
	LB440A

	Lamps:--4each
	4' F40T12 34W

	Lumens per Lamp:
	2,650

	Total Lumens:
	10,600


	
	Description:
This luminaire uses four 4' standard fluorescent T12 tubes, is ceiling mounted, and has a wrap around prismatic lens.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
1.5 luminaires
	Luminaires Per Square Foot Factor 

1.5 luminaires ÷ 150 sq. ft. =
.01


	---


	Table C
	[image: image5.png]





[image: image6.png]CCR 80% 0% 50% 30%
WCR | 70% | 50% | 30% [ 10% | 70% | 50% | 30% | 10% | 60% | 30% [10% | 50% | 30% [ 10%
7 451 | 442 | 435 441 | 434 | 427 428 | 421 | 416] 411 | 407|403
2 431 | 415 | 408 423|411 | 401 | 409 | 401 | 393] 401 | 301 | 385
B 409 | 304 | 381 404 | 392 | 370 305 | 387 | 374] 386 | 376 | 368
4 390 375 | 360 383 | 360 | 367 | 377 | 364 | 353] 370 | 358 | 348
s a7t | 354 | 338 374 | 351 | 338 | 362 | 348 | 337] 356 | 344 | 33
6 355 | 338 | 325 353 | 337 | 323| 347 | 333 | 321] 341 | 330] 319
7 340 322 | 308 338 | 316 | 308 | 333 | 316 | 305] 328 | 314303
8 323 | 304 | 202 319 302 | 20| 317 | 301 | 209|313 | 208 | 288
9 300 | 288 | 274 305 | 287 | 274 302 | 265 | 275| 209 | 283 | 2713
0 762 | 260 | 246 778 | 260 | 24| 276 257 | 245] 271 | 256 245
FCR 20%





	Manufacturer:
Model:
	Halo
H7-304

	Trim:
	Black Step Baffle

	Lamps:--1 each
	75W R30 Flood

	Lumens per Lamp:
	900

	Total Lumens:
	900


	
	Description:
This luminaire uses one 75W R30 Flood lamp, is a recessed ceiling can, and is rated using a black step baffle trim. This is a very popular luminaire and has been included to show just how inefficient it is.
The black step baffle absorbs about half the light the lamp is producing!

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
22.8 luminaires
	Luminaires Per Square Foot Factor 

22.8 luminaires ÷ 150 sq. ft. =
.152


	---


	Table D
	[image: image7.png]





[image: image8.png]30%

85
7
69
63
58
53
a8
45
a

£

6
79
72
66
61

56
52
a8
45
a2

a7
a1

75
70
65
60
56
53
]
I

50%

a7
78
7t

64
58
53
]
45
a

£

89
a1

74
67
62
57
52
a8
45
a2

a1

84
78
72
66
62
57
54
50
a7

0%

90
0
72
65
59
53
]
45
4
£

20%

92
]
75
69
63
57
53
]
45
a2

94
a7
80
74
68
3
59
55
51

8

a7
at

6
a1

76
iz

7
3
60
57

80%

0% | 70% | 60% | 30% | 10% | 50% | 30% [10% | 50% | 30% [ 10%

92
a1

72
65
59
54
]
45
4
£

94
84
76
69
63
58
53
]
I
a2

9%
8
a1

75
69
64
59
55
51

8

99
93
88
2
7
73
68
65
61

58

WCR | 70% | 50% [ 30%

0

FCR

D [CCR





	Manufacturer:
Model:
	Halo
H7-426

	Trim:
	Clear Alzak (Specular)

	Lamps:--1 each
	75W R30 Flood

	Lumens per Lamp:
	900

	Total Lumens:
	900


	
	Description:
This luminaire uses one 75W R30 Flood lamp, is a recessed ceiling can, and is rated using a clear Alzak (Specular) trim. This luminaire, while better than the previous one, is still inefficient in comparison to the upcoming combinations.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
11.9 luminaires
	Luminaires Per Square Foot Factor 

11.9 luminaires ÷ 150 sq. ft. =
.079


	


	Table E
	[image: image9.png]





[image: image10.png]CCR 80% 0% 50% 30%
WCR | 70% | 50% | 30% [ 10% | 70% | 50% | 30% | 10% | 60% | 30% [10% | 50% | 30% [ 10%
7 11] 110108 1.08]1.08 [ 1.06[1.05 | 1.04 | 1.03] 1.02] 1.01 [ 1.00
2 1.07] 108|102 1.06] 103 [1.01]102|1.00] 98 | 99 | 98 | 96
B 103 ] 100 87 1.01] @9 [ o7 [ o9 | o7 | @5 | o7 | 95| a4
4 99 | 96 | o4 99 | o6 | @3 | o7 | 84 | 82| 95 | 93| &t
s 9 | 92 | @0 95| @2 |89 | 94| o1 |89 | 02| o0 88
6 93] 90 | &7 93| 89 |87 | o1 | 89 | @6 | o0 | 88| 86
7 CEAED 90 | 86 | 84 | 89 | 86 | 84 | e | 85| &3
8 8 | 84 | 82 7 | 84 | 81 | 86 | 83 | 61 | 86 | 83| &t
9 EHIERED 84| 81 |70 | &4 | &1 |78 | 83| 80| 78
0 53] 79 | 78 2] 70 | 76 | 82| 78 | 76 | @1 | 78| 76
FCR 20%





	Manufacturer:
Model:
	Halo
H7-426

	Trim:
	Clear Alzak (Specular)

	Lamps:--1 each
	75W PAR30 Flood

	Lumens per Lamp:
	1,030 (Halogen)

	Total Lumens:
	1,030


	
	Description:
This luminaire uses one 75W PAR30 Halogen Flood lamp, is a recessed ceiling can, and is rated using a clear Alzak (Specular) trim. This luminaire is the most efficient of the three incandescent cans rated here.
This luminaire is a good choice for general lighting when short range budget is a constraint. Use one of the Epact rated halogen lamps for best efficiency and energy compliance.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
7.8 luminaires
	Luminaires Per Square Foot Factor 

7.8 luminaires ÷ 150 sq. ft. =
.052


	---


	Table F
	[image: image11.png]





[image: image12.png]30%

67
63
58
55
51

a8
a
a

3
3

68
64
60
57
53
50
I
3
a0
£

69
66
62
59
56
53
]
I
a
a1

50%

69
64
59
55
51

a8
45
a

3
£

70
66
61

57
54
50
a7
a
a

£

T

67
64
60
57
53
50
a7
a
a1

0%

T

65
60
56
52
a8
45
a

3
£

20%

73
67
63
59
54
51

a7
a
a

£

74
70
65
62
58
54
51

a8
45
a2

76
72
69
66
63
60
57
54
51

8

80%

0% | 70% | 60% | 30% | 10% | 50% | 30% [10% | 50% | 30% [ 10%

73
66
61

56
52
]
45
4
3
£

74
68
63
59
55
51

a8
a
a

£

75
iz

66
62
59
55
51

a8
45
a2

7
74
70
67
64
61

57
55
52
]

CCR

WCR | 70% | 50% [ 30%

0

FCR





	Manufacturer:
Model:
	Prescolite
CFV26-892

	Trim:
	Clear Alzak (Specular)

	Lamps:--1 each
	26W Double Twin

	Lumens per Lamp:
	1,800

	Total Lumens:
	1,800


	
	Description:
This luminaire uses one vertically positioned 26W Double Twin compact fluorescent lamp, is a recessed ceiling can, and is rated using a clear Alzak (Specular) trim.
Though this luminaire is far more efficient, in the long run, than the three incandescent cans shown previously, the lamps' vertical position is considered by more than a few designers as ugly. For an aesthetic improvement and greater efficiency, refer to the two following compact fluorescent luminaires.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
7.2 luminaires
	Luminaires Per Square Foot Factor 

7.2 luminaires ÷ 150 sq. ft. =
.048


	---


	Table G
	[image: image13.png]Papp—

7






[image: image14.png]30%

60
55
50
46
I

a
]
3
o
)

61

57
52
48
)
a0
3
7]
]
7

62
58
54
50
a7
a
3
3
]
]

50%

62
56
51

46
a2
£
34
3
o
)

63
58
53
49
)
a0
3
]
]
7

65
60
56
52
48
)
a0
a7
]
]

0%

64
58
52
a7
a2
£
34
3
o
)

20%

66
60
54
50
45
a

a7
]
3
i

67
62
57
53
49
45
a

a7
34
3

69
65
61

58
54
51

a7
a
a

£

80%

0% | 70% | 60% | 30% | 10% | 50% | 30% [10% | 50% | 30% [ 10%

65
58
52
a7
a2
£
34
3
o
)

67
61

55
50
45
a

a7
]
3
i

69
63
58
54
50
45
a

3
34
3

70
67
63
59
55
52
a8
45
a

£

WCR | 70% | 50% [ 30%

0

FCR

6 [ CCR





	Manufacturer:
Model:
	Prescolite
CFR818-492

	Trim:
	Clear Alzak (Specular)

	Lamps:--2 each
	18W Double Twin

	Lumens per Lamp:
	1,250

	Total Lumens:
	2,500


	
	Description:
This luminaire uses two horizontally positioned 18W Double Twin compact fluorescent lamps, is a recessed ceiling can, and is rated using a clear Alzak (Specular) trim. The most energy efficient and, over the long run, most cost effective luminaire design. The horizontal orientation of the lamps avoids the unsightliness of the previous luminaire while providing more light through the use of an extra lamp.
The only decision between this and the next unit is how many fixtures the designer wishes in the ceiling.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
5.9 luminaires
	Luminaires Per Square Foot Factor 

5.9 luminaires ÷ 150 sq. ft. =
.039


NOTE:
The above fixture features twin ballasts that can be wired independently. By connecting each of the lamps in each of the cans separately to two different switches, a hi-low dimming effect can be achieved.
	---


	Table H
	[image: image15.png]Papp—

7






[image: image16.png]30%

57
52
48
IE]
3
3
7]
]
5
7]

58
54
49
45
I

£
34
El

]
z

59
55
51

48
)
a

a7
34
El

]

50%

59
53
48
)
40
3
7]
]
5
7]

60
55
50
46
a2
£
34
El

]
z

61

57
53
49
45
a2
3
3
El

]

0%

61

55
49
)
40
3
7]
]
5
FE]

20%

62
57
51

a7
IE]
]
3
El

]
z

64
59
54
50
46
a
3
3
7]
]

65
62
58
55
51

a2
45
4
3
£

80%

0% | 70% | 60% | 30% | 10% | 50% | 30% [10% | 50% | 30% [ 10%

62
55
49
45
40
3
7]
]
5
FE]

63
58
52
a7
IE]
3
3
7]
]
z

65
60
55
51

a7
a
3
3
]
]

67
63
59
56
52
]
45
4
3
£

WCR | 70% | 50% [ 30%

0

FCR

H[CCR





	Manufacturer:
Model:
	Prescolite
CFR826-492

	Trim:
	Clear Alzak (Specular)

	Lamps:--2 each
	26W Double Twin

	Lumens per Lamp:
	1,800

	Total Lumens:
	3,600


	
	Description:
This luminaire uses two horizontally positioned 26W Double Twin compact fluorescent lamps, is a recessed ceiling can, and is rated using a clear Alzak (Specular) trim. The most energy efficient and, over the long run, most cost effective luminaire design. The horizontal orientation of the lamps avoids the unsightliness of vertically oriented compact fluorescent lamps while providing more light.

	Comparison
In a 10' x 15' kitchen with a desired footcandle level of 40, 80% ceiling reflectance, 50% wall reflectance, and 20% floor reflectance:
4.4 luminaires
	Luminaires Per Square Foot Factor 

4.4 luminaires ÷ 150 sq. ft. =
.029


NOTE:
The above fixture features twin ballasts that can be wired independently. By connecting each of the lamps in each of the cans separately to two different switches, a hi-low dimming effect can be achieved.
	---


  LightCalc Main Page
