	Knots - Lashings - Filipino


	This is an alternative to the diagonal lashing that is suitable for use in light constructions. If you have any doubt about its usefulness or will be subjecting it to a large strain you should take the time to tie a proper diagonal lashing instead.



	

	Step by Step Guide

Start with the middle of the rope, tucking the running ends through the middle "loop" after going round both spars. Use the "loop" to pull the spars together.

Now proceed as for a diagonal lashing taking the running end round both spars, keeping both ends together

Separate the ends and take frapping turns between the spars, pulling the rope tight as you do so.

Complete the frapping turns and finish off with a reef knot




